3GPP TSG-RAN WG2 Meeting #105	R2-1902364

Athens, Greece, February 25th – March 1st, 2019

Agenda Item:	11.7.3

Source:	InterDigital (rapporteur)

Title:	TP on UL Data-Data Intra UE prioritization

WID/SID:	FS_NR_IIOT - Release 16

Document for:	Discussion, Decision

# Introduction

This contribution contains a text proposal based on the rapporteur’s summary of the following email discussion:

[104#38][NR/IIOT] Intra UE prioritization UL Data Data (Interdigital) Intended outcome: Report, Identify issues, identify solutions (try to avoid stage-3 details to the extent possible) Deadline: Thursday 2019-02-07

The summary of the email discussion is in the companion contribution in [2].

# References

RP-182090, Revised SID: Study on NR Industrial Internet of Things (IoT), Nokia, Nokia Shanghai Bell.

R2-1901458, “Summary of [104#38][NR/IIOT] Intra UE prioritization UL Data Data (Interdigital)”, InterDigital.

# Text Proposal based on TR 38.825 v 0.1.0

# Intra-UE prioritization/multiplexing

/// Unmodified text not included

## 5.3	Solutions for uplink intra-UE prioritization/multiplexing

Editor’s note: RAN2 is main responsible group, but potential PHY layer impacts and solutions should be analysed by RAN1

/// Start of proposed changes

## 5.3.1		Solutions for resource collision involving data only

Data-only intra-UE prioritization is applicable for resource conflicts between configured/configured, configured/dynamic, or dynamic/dynamic grants.

UE prioritization of a grant when there is at most one dynamic grant in the set of conflicting grants (scenario 2 and configured/configured grant collisions) shall be addressed. MAC specifies currently the UE prioritization of such cases, and modifications to MAC would be required.

RAN2 assumes that the later dynamic grant may always be prioritized over an earlier dynamic grant (scenario 3). One way to realize this is that MAC generates a PDU for each grant and lets L1 handle conflicting transmissions. To be confirmed following progress in RAN1. Other solutions are not precluded.

For cases when MAC prioritizes a grant, MAC prioritizes the grant on which data of the highest priority can be transmitted according to LCP restrictions and priority configured for each logical channel.

/// End of proposed changes