3GPP TSG-RAN WG2 Meeting #105	 R2-1902585

Athens, Greece, 25 Feb - 1 Mar 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 0823 | rev |  | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on dedicated serving cell configuration in Re-establishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-2-9 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In 5.3.7.2 for RRC Connection Re-establishment, it is said ‘UE release the current dedicated ServingCell’ configuration. However, it is unclear to which IE the ‘dedicated ServingCell configuration’ exactly refers. During re-establishment and resume procedure, the CellGroupConfig IE is used to configure MCG, which contains spCellConfig for PCell. The IE spCellConfig contains reconfigurationWithSync, spCellConfigDedicated and other IEs. The IE reconfigurationWithSync also contains some common configuration i.e. spCellConfigCommon. The question is whether dedicated serving cell configuration refers to spCellConfig or spCellConfigDedicated. For both re-establishment and resume, UE ensures having valid and up to date essential system information before initiating those procedures. There is no need for UE to keep the common configuration. So spCellConfig should be released. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE release spCellConfig when initiate RRC Connection Re-establishment; Impact Analysis Applicable to EN-DC or SA SA Impacted functionality Initiation of RRC connection re-establishment Inter-operability for SA - If the network is implemented according to the CR and the UE is not, the information released by the network is different from the information released by the UE. Re-establishment procedure will fail due to the configuration inconsistency between UE and network. - If the UE is implemented according to the CR and the network is not, the information released by the UE is different from the information released by the network. Re-establishment procedure will fail due to the configuration inconsistency between UE and network. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE doesn’t exactly know which IE for the serving cell to release when initiating RRC connection re-establishment. Re-establishment procedure will fail due to the configuration inconsistency between UE and network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF THE FIRST CHANGES

#### 5.3.7.2	Initiation

The UE initiates the procedure when one of the following conditions is met:

1>	upon detecting radio link failure of the MCG, in accordance with 5.3.10; or

1>	upon re-configuration with sync failure of the MCG, in accordance with sub-clause 5.3.5.8.3; or

1>	upon mobility from NR failure, in accordance with sub-clause 5.4.3.5; or

1>	upon integrity check failure indication from lower layers concerning SRB1 or SRB2, except if the integrity check failure is detected on the RRCReestablishment message; or

1>	upon an RRC connection reconfiguration failure, in accordance with sub-clause 5.3.5.8.2.

Upon initiation of the procedure, the UE shall:

1>	stop timer T310, if running;

1>	stop timer T304, if running;

1>	start timer T311;

1>	suspend all RBs, except SRB0;

1>	reset MAC;

1>	release the MCG SCell(s), if configured;

1>	release spCellConfig ;

1>	release delayBudgetReportingConfig, if configured, and stop timer T342, if running;

1>	release overheatingAssistanceConfig, if configured and stop timer T345, if running;

1>	perform cell selection in accordance with the cell selection process as specified in TS 38.304 [20], clause 5.2.6.