# 3GPP TSG RAN WG2 Meeting #105      			                           R2-1902596
Athens, Greece, 25th Feb.- 1st March. 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0075 | rev | 2 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.306 on introducing nr-CGI-Reporting-ENDC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-01-16 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Add one UE capability (nr-CGI-Reporting-ENDC) on whether UE supports SN configured ANR for NR neighbour cells in EN-DC. - Clarify that UE capability (nr-CGI-Reporting) is only applied to NR standalone (i.e. not applied to SN configured ANR towards NR neighbour cells in EN-DC). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add one UE capability (nr-CGI-Reporting-ENDC) on whether UE supports SN configured ANR for NR neighbour cells in EN-DC. - Clarify that UE capability (nr-CGI-Reporting) is only applied to NR standalone (i.e. not applied to SN configured ANR towards NR neighbour cells in EN-DC). Impact Analysis: Impacted functionality: Section 4.2.9: MeasAndMobParameters Chang 1 applies to NSA. Chang 2 applies to SA. Inter-operability: - If the UE is implemented according to the CR and the network is not, the network can’t know whether UE can perform SN configured ANR towards E-UTRA neighbour cells, and thereby SN can’t configure ANR towards E-UTRA neighbour cells. - If the network is implemented according to the CR and the UE is not, the UE can’t report its capability whether it can perform SN configured ANR towards E-UTRA neighbour cells. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE can’t report its capability whether it can perform SN configured ANR towards E-UTRA neighbor cells. It will lead to that SN can’t configure ANR towards E-UTRA neighbor cells. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of 1st change

### 4.2.9	MeasAndMobParameters

| Definitions for parameters | Per | M | FDD-TDD diff | FR1 FR2 diff |
| --- | --- | --- | --- | --- |
| csi-RS-RLM Indicates whether the UE can perform radio link monitoring procedure based on measurement of CSI-RS as specified in TS38.213 [11] and 38.133 [5]. This parameter needs FR1 and FR2 differentiation. | UE | Yes | No | Yes |
| csi-RSRP-AndRSRQ-MeasWithSSB Indicates whether the UE can perform CSI-RSRP and CSI-RSRQ measurement as specified in TS38.215 [13], where CSI-RS resource is configured with an associated SS/PBCH. This parameter needs FR1 and FR2 differentiation. | UE | No | No | Yes |
| csi-RSRP-AndRSRQ-MeasWithoutSSB Indicates whether the UE can perform CSI-RSRP and CSI-RSRQ measurement as specified in TS38.215 [13], where CSI-RS resource is configured for a cell that transmits SS/PBCH block and without an associated SS/PBCH block. This parameter needs FR1 and FR2 differentiation. | UE | No | No | Yes |
| csi-SINR-Meas Indicates whether the UE can perform CSI-SINR measurements based on configured CSI-RS resources as specified in TS38.215 [13]. This parameter needs FR1 and FR2 differentiation. | UE | No | No | Yes |
| eutra-CGI-Reporting Defines whether the UE supports acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the EN-DC is not configured. | UE | Yes | No | No |
| eventA-MeasAndReport Indicates whether the UE supports NR measurements and events A triggered reporting as specified in TS 38.331 [9] | UE | Yes | Yes | No |
| eventB-MeasAndReport Indicates whether the UE supports EUTRA measurement and event B triggered reporting as specified in TS 38.331 [9]. It is mandated if the UE supports EUTRA. | UE | Yes | No | No |
| handover-eLTE Indicates whether the UE supports HO to EUTRA connected to 5GC. It is mandated if the UE supports EUTRA connected to 5GC. | UE | Yes | Yes | Yes |
| handoverFDD-TDD Indicates whether the UE supports HO between FDD and TDD. It is mandated if the UE supports both FDD and TDD. | UE | Yes | No | No |
| handoverInterF Indicates whether the UE supports inter-frequency HO. | UE | Yes | Yes | Yes |
| handoverLTE Indicates whether the UE supports HO to EUTRA connected to EPC. It is mandated if the UE supports EUTRA connected to EPC. | UE | Yes | Yes | Yes |
| independentGapConfig This field indicates whether the UE supports two independent measurement gap configurations for FR1 and FR2 specified in TS 38.133 [5]. | UE | No | Yes | No |
| intraAndInterF-MeasAndReport Indicates whether the UE supports NR intra-frequency and inter-frequency measurements and at least periodical reporting. | UE | Yes | Yes | No |
| nr-CGI-Reporting Defines whether the UE supports acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when EN-DC is not configured. | UE | Yes | No | No |
| nr-CGI-Reporting-ENDC Defines whether the UE supports acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the EN-DC is configured. | UE | Yes | No | No |
| simultaneousRxDataSSB-DiffNumerology Indicates whether the UE supports concurrent intra-frequency measurement on serving cell or neighbouring cell and PDCCH or PDSCH reception from the serving cell with a different numerology. | UE | No | Yes | Yes |
| sftd-MeasPSCell Indicates whether the UE supports SFTD measurements between the Pcell and a configured PSCell. | UE | No | Yes | No |
| sftd-MeasNR-Cell Indicates whether the SFTD measurement between the Pcell and the NR cells is supported by the UE which is capable of EN-DC when EN-DC is not configured. | UE | No | Yes | No |
| ssb-RLM Indicates whether the UE can perform radio link monitoring procedure based on measurement of SS/PBCH block as specified in TS38.213 [11] and 38.133 [5]. | UE | Yes | No | No |
| ssb-AndCSI-RS-RLM Indicates whether the UE can perform radio link monitoring procedure based on measurement of SS/PBCH block and CSI-RS as specified in TS38.213 [11] and 38.133 [5]. | UE | Tbd | No | No |
| ss-SINR-Meas Indicates whether the UE can perform SS-SINR measurement as specified in TS38.215 [13]. This parameter needs FR1 and FR2 differentiation. | UE | No | No | Yes |
| supportedGapPattern Indicates measurement gap pattern(s) optionally supported by the UE. The leading / leftmost bit (bit 0) corresponds to the gap pattern 2, the next bit corresponds to the gap pattern 3, as specified in TS 38.311 [9] and so on. | UE | No | No | No |

End of change