3GPP TSG-RAN WG2 Meeting#105	R2-1900xxx

# Athens, Greece, 25th Feb.- 1st March. 2019

Agenda item:	10.4.4.2

Source:	ZTE Corporation, Sanechips

Title:	Summary of offline discussion#76: Clarification on intra-NR handover related capabilities

WID/SID:	NR_newRAT-Core– Release 15

Document for:	Discussion and Decision

Introduction

This is document of offline discussion#42

R2-1900668	CR to clarify intra-NR handover capabilities	ZTE Corporation, Sanechips	CR	Rel-15	38.306	15.4.0	0086	-	F	NR_newRAT-Core

-	DOCOMO understand these capabilities are for SA

-	Ericsson have the same view. Qualcomm thinks the capabilities must cover SA and EN-DC to cover the case that the measurements are configured by SN. The concern was that there might not be IOT opportunities.

-	Samsung think clarification is needed if the capabilities apply for both EN-DC and SA

=>	Offline discussion to conclude which capabilities are for EN-DC only, SA only or both EN-DC and SA. I applicable to only EN-DC or SA then discussion whether to add an additional capability for the other case.

=>	Revised in R2-1902643 (Offline discussion 76).

In order to progress, we have two phases:

- Phase 1 (by COB Thursday)

  - Clarify the understanding on the applicable cases of current capability bits in MeasAndMobParameters.

  - To identify whether any new capabilities are needed for PSCell change in EN-DC.

- Phase 2 (by COB Friday noon)

  - Produce CR for TS 38331 or TS38.306 based on the conclusion of phase1.

Discussion

Based on the online discussion on [1], companies have different understanding whether the existing capabilities defined in MeasAndMobParameters are applicable to only NR SA(e.g. PCell handover), or it can also be applicable to EN-DC(e.g. PSCell change).

The latest definition of MeasAndMobParameters structure is copied/pasted below:

MeasAndMobParameters information element

-- ASN1START

-- TAG-MEASANDMOBPARAMETERS-START

MeasAndMobParameters ::=                    SEQUENCE {

measAndMobParametersCommon              MeasAndMobParametersCommon              OPTIONAL,

measAndMobParametersXDD-Diff                MeasAndMobParametersXDD-Diff        OPTIONAL,

measAndMobParametersFRX-Diff                MeasAndMobParametersFRX-Diff        OPTIONAL

}

MeasAndMobParametersCommon ::=          SEQUENCE {

supportedGapPattern                 BIT STRING (SIZE (22))          OPTIONAL,

ssb-RLM                             ENUMERATED {supported}          OPTIONAL,

ssb-AndCSI-RS-RLM                   ENUMERATED {supported}          OPTIONAL,

...,

[[

eventB-MeasAndReport        ENUMERATED {supported}      OPTIONAL,

handoverFDD-TDD             ENUMERATED {supported}      OPTIONAL,

eutra-CGI-Reporting         ENUMERATED {supported}      OPTIONAL,

nr-CGI-Reporting            ENUMERATED {supported}      OPTIONAL

]],

[[

independentGapConfig        ENUMERATED {supported}                  OPTIONAL,

periodicEUTRA-MeasAndReport ENUMERATED {supported}                  OPTIONAL,

handoverFR1-FR2             ENUMERATED {supported}                  OPTIONAL,

maxNumberCSI-RS-RRM-RS-SINR ENUMERATED {n4, n8, n16, n32, n64, n96} OPTIONAL

]]

}

MeasAndMobParametersXDD-Diff ::=            SEQUENCE {

intraAndInterF-MeasAndReport        ENUMERATED {supported}  OPTIONAL,

eventA-MeasAndReport                ENUMERATED {supported}  OPTIONAL,

...,

[[

handoverInterF              ENUMERATED {supported}  OPTIONAL,

handoverLTE             ENUMERATED {supported}  OPTIONAL,

handover-eLTE               ENUMERATED {supported}  OPTIONAL

]]

}

MeasAndMobParametersFRX-Diff ::=            SEQUENCE {

ss-SINR-Meas                        ENUMERATED {supported}      OPTIONAL,

csi-RSRP-AndRSRQ-MeasWithSSB        ENUMERATED {supported}      OPTIONAL,

csi-RSRP-AndRSRQ-MeasWithoutSSB     ENUMERATED {supported}      OPTIONAL,

csi-SINR-Meas                       ENUMERATED {supported}      OPTIONAL,

csi-RS-RLM                          ENUMERATED {supported}      OPTIONAL,

...,

[[

handoverInterF              ENUMERATED {supported}  OPTIONAL,

handoverLTE                 ENUMERATED {supported}  OPTIONAL,

handover-eLTE               ENUMERATED {supported}  OPTIONAL

]],

[[

maxNumberResource-CSI-RS-RLM    ENUMERATED {n2, n4, n6, n8}     OPTIONAL

]]

}

-- TAG-MEASANDMOBPARAMETERS-STOP

-- ASN1STOP

In this offline discussion, we only focus on below two scenarios:

- PCell handover in case of NR SA;

- PSCell change in case of EN-DC.

Since inter-RAT measurement and inter-RAT handover are not supported at SN side, so only intra-NR measurement and handover related capabilities are involved in this discussion.

- Measurement related capabilities are blue highlighted.

- Handover related capabilities are yellow highlighted.

Companies are invited to provide their understanding on the applicability of current capabilities. According to the online discussion, for each capability, there are 4 possible understandings:

- Understanding 1: This capability field is only applicable to NR SA(e.g. PCell handover). (It implies that another capability should be introduced in MeasAndMobParametersMRDC for EN-DC);

- Understanding 2: This capability field is only applicable to EN-DC(e.g PSCell change). (It implies that another capability should be introduced in MeasAndMobParameters for NR SA);

- Understanding 3: This capability field is applicable to both NR SA(e.g. PCell handover) and EN-DC(e.g PSCell change). (No impact to ASN.1);

- Understanding 4: This capability field is only applicable to NR SA(e.g. PCell handover), and the features are mandatory supported for EN-DC(e.g. PSCell change) without capability signalling. (No impact to ASN.1)

Question 1: For below intra-NR measurement related capabilities, companies are invited to express their understandings on the applicable scenarios (and to add additional capabilities to the list, if they feel anything is missing).

| Capability field in MeasAndMobParameters | Applicable scenarios |  |  |  |  | Comments |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Only NR SA (e.g. PCell handover) | Only EN-DC (e.g. NR PSCell change) | Both NR SA and EN-DC | Only NR SA, madatory for EN-DC |  |  |  |  |  |
| intraAndInterF-MeasAndReport |  | DCM (at least) Qualcomm | ZTE, Ericsson Huawei CATT |  | [ZTE]: we haven’t seen much functionality difference between PCell handover and PSCell change, to avoid NBCs, we prefer to reuse the current bit for both cases. [Qualcomm] This capability implies the support for SN configured measurement in EN-DC. If we make this applicable to NR SA, it does not allow the UE to support this feature in NR SA, but not SN configured measurement in EN-DC. |  |  |
| eventA-MeasAndReport |  | DCM (at least) Qualcomm | ZTE, Ericsson Huawei CATT |  | [ZTE] same to above comment. [Qualcomm] This capability implies the support for SN configured measurement in EN-DC. If we make this applicable to NR SA, it does not allow the UE to support this feature in NR SA, but not SN configured measurement in EN-DC. |  |  |
| maxNumberCSI-RS-RRM-RS-SINR |  |  | ZTE, DCM, Ericsson Huawei Qualcomm CATT |  | [ZTE] same to above comment. |  |  |
| ss-SINR-Meas |  |  | ZTE, Ericsson Huawei Qualcomm CATT DCM |  | [ZTE] same to above comment. |  |  |
| csi-RSRP-AndRSRQ-MeasWithSSB |  |  | ZTE, Ericsson Huawei Qualcomm CATT DCM |  | [ZTE] same to above comment. |  |  |
| csi-RSRP-AndRSRQ-MeasWithoutSSB |  |  | ZTE, Ericsson Huawei Qualcomm CATT DCM |  | [ZTE] same to above comment. |  |  |
| csi-SINR-Meas |  |  | ZTE, Ericsson Huawei Qualcomm CATT DCM |  | [ZTE] same to above comment. |  |  |

Summary:

For following capabilities, all the six companies agreed that they can be applicable to both EN-DC and NR SA.

- maxNumberCSI-RS-RRM-RS-SINR

- ss-SINR-Meas

- csi-RSRP-AndRSRQ-MeasWithSSB

- csi-RSRP-AndRSRQ-MeasWithoutSSB

- csi-SINR-Meas

For following capabilities, one company think they are only applicable to EN-DC, and one company think they are at least applicable to EN-DC, while four companies think they are applicable to both NR SA and EN-DC.

- intraAndInterF-MeasAndReport

- eventA-MeasAndReport

Proposal 1: 	RAN2 agreed following intra-NR measurement capabilities are applicable to both EN-DC and NR SA. Update the field descriptions in 38.306 to capture this.

- maxNumberCSI-RS-RRM-RS-SINR

- ss-SINR-Meas

- csi-RSRP-AndRSRQ-MeasWithSSB

- csi-RSRP-AndRSRQ-MeasWithoutSSB

- csi-SINR-Meas

For intraAndInterF-MeasAndReport and eventA-MeasAndReport, since companies haven’t reached the consensus, we suggest to discuss this online, how to solve it for NR SA.

Proposal 2:  To discuss the applicability of measurement capabilities intraAndInterF-MeasAndReport and eventA-MeasAndReport.

- Option1: applicable to both NR SA and EN-DC;

- Option2: only applicable to EN-DC.

If the conclusion of online discussion is option2. Then following solutions can be discussed：

Proposal 2.1:  For NR SA, to discuss the solution for intraAndInterF-MeasAndReport and eventA-MeasAndReport, if Option2 is agreed in proposal2.

- Solution 1: Reuse the current capability bits for NR SA;

- Solution 2: Introduce new capability bits for NR SA. Note that, the same bits will be introduced in the same structure ”MeasAndMobParameters”;

- Solution 3: Clarify in spec that these capabilities are mandatory supported for NR SA without capability signalling

For above intra-NR measurement related capabilities, since RAN2 haven’t further discussed the applicability of NR SA scenario, if company think these are (only/at least) applicable to EN-DC, then for NR SA, following solutions can be considered:

- Solution1: Reuse the current capability bits for NR SA.

- Solution2: Introduce new capability bits for NR SA. Note that, the same bits will be introduced in the same structure ”MeasAndMobParameters”;

- Solution3: Clarify in spec that those capability are mandatory supported for NR SA without capability signalling.

Question 1-1: If above measurement related capabilities are (only/at least) applicable to EN-DC, companies are invited to provide their preference on above solutions for NR SA scenario.

| Company | Options | Comments |
| --- | --- | --- |

For handover related capabilities, besides SN triggered intra-SN PSCell change, that the MN can also trigger SN change procedure. As long as SN does not perform full configuration in this case, the UE behaviour is same from UE perspective(e.g. reconfigurationWithSync with/wo key refresh).

Therefore, from MN’s perspective, MN should also be aware of these intra-NR handover capabilities. Otherwise, MN can only trigger SN full configuration via Uu interface (e.g. similar to PSCell release+addition by setting endc-ReleaseAndAdd-r15 to true).

Question 2: For below intra-NR handover related capabilities, companies are invited to express their understandings on the applicable scenarios.

| Capability field in MeasAndMobParameters | Applicable scenarios |  |  |  |  | Comments |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Only NR SA (e.g. PCell handover) | Only EN-DC (e.g. NR PSCell change) | Both NR SA and EN-DC | Only NR SA, madatory for EN-DC |  |  |  |  |  |
| handoverFDD-TDD |  |  |  | ZTE, DCM, Ericsson Huawei Qualcomm CATT | [ZTE]: Since both MN and SN can trigger PSCell change, considering there’s no explicit capabilities in LTE-DC for PSCell change, we assume this feature can be mandatory supported in EN-DC. |  |  |
| handoverFR1-FR2 |  |  |  | ZTE, DCM, Ericsson Huawei Qualcomm CATT | [ZTE] same to above comment. |  |  |
| handoverInterF(in XDD-diff) |  |  |  | ZTE, DCM, Ericsson Huawei Qualcomm CATT | [ZTE] same to above comment. |  |  |
| handoverInterF(in FRX-diff) |  |  |  | ZTE, DCM, Ericsson Huawei Qualcomm CATT | [ZTE] same to above comment. |  |  |

Summary:

For following intra-NR handover related capabilities, all the six companies agreed that they are applicable to NR SA, and for EN-DC, they are mandatory supported without capability signalling.

Proposal 3:  RAN2 agreed the following intra-NR handover capabilities are applicable to only NR SA, these features are mandatory supported for EN-DC without capability signalling, update the field descriptions in 38.306 to capture this.

- handoverFDD-TDD

- handoverFR1-FR2

- handoverInterF(in XDD-diff)

- handoverInterF(in FRX-diff)

Summary

Proposal 1: 	RAN2 agreed following intra-NR measurement capabilities are applicable to both EN-DC and NR SA. Update the field descriptions in 38.306 to capture this.

- maxNumberCSI-RS-RRM-RS-SINR

- ss-SINR-Meas

- csi-RSRP-AndRSRQ-MeasWithSSB

- csi-RSRP-AndRSRQ-MeasWithoutSSB

- csi-SINR-Meas

Proposal 2:  To discuss the applicability of measurement capabilities intraAndInterF-MeasAndReport and eventA-MeasAndReport.

- Option1: applicable to both NR SA and EN-DC;

- Option2: only applicable to EN-DC.

If the conclusion of online discussion is option2. Then following solutions can be discussed：

Proposal 2.1:  For NR SA, to discuss the solution for intraAndInterF-MeasAndReport and eventA-MeasAndReport, if Option2 is agreed in proposal2.

- Solution 1: Reuse the current capability bits for NR SA;

- Solution 2: Introduce new capability bits for NR SA. (Note that, the same bits will be introduced in the same structure ”MeasAndMobParameters”);

- Solution 3: Mandatory supported for NR SA without capability signalling

Proposal 3:  RAN2 agreed the following intra-NR handover capabilities are applicable to only NR SA, the features are mandatory supported for EN-DC without capability signalling, update the field descriptions in 38.306 to capture this.

- handoverFDD-TDD

- handoverFR1-FR2

- handoverInterF(in XDD-diff)

- handoverInterF(in FRX-diff)