3GPP TSG-RAN WG2 Meeting #105	R2-1902650

Athens, Greece, 24 Feb – 1 Mar 2019

Title:	[DRAFT] Reply LS on barred access due to rejection from NG-RAN

Response to:	R2-1900004 (C1-188954)

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN2

To:	CT1

Cc:	SA1

Contact Person:

Name:	Oscar Ohlsson

E-mail Address:	oscar.ohlsson@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

RAN2 thanks CT1 for the LS on barred access due to rejection from NG-RAN in C1-188954.

RAN2’s understanding is that the LS is referring to the RRC connection setup case and does not cover the RRC connection resume case. If an RRC resume request is rejected and AS indicates connection failure to NAS without further specifying the reason for the failure, then NAS will not be able to distinguish reject from other types of failures (e.g. T319 expiry). RAN2 understanding is that this would then result in the UE moving from RRC_INACTIVE to RRC_IDLE and triggering of NAS recovery. However, RAN2 has previously agreed that the UE should remain in RRC_INACTIVE in case of reject. RAN2 has also assumed that the pending procedure will be re-triggered by the UE when the wait timer expires.

RAN2 would also like to point out that a wait timer can also be included in the RRC release message. RRC release with wait timer differs from the reject case in that the UE is in RRC_CONNECTED mode when the wait timer is received, and upon receiving the release message with wait timer, the UE moves to IDLE mode and informs the upper layer that access barring is applicable for all access categories except categories '0' and '2'.

2. Actions

To CT1 group:

ACTION:

- RAN2 kindly requests CT1 to take the above feedback into account and consider any possible updates to CT1 specifications to address the above issue regarding Reject.

- RAN2 kindly requests CT1 to confirm whether the AS behaviour regarding RRC Release with wait time as mentioned above is consistent with the NAS specs.

3. Date of Next TSG RAN WG2 Meetings:

3GPP TSG RAN WG2 #105bis		2019.04.08 – 2019.04.12			Xi’an, China

3GPP TSG RAN WG2 #106		2019.05.13 – 2019.05.17			Reno, USA