3GPP TR 38.840 V0.1.1 (2019-02)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

NR;

Study on UE Power Saving

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1.	Scope	5

2.	References	5

3.	Definitions, symbols and abbreviations	6

3.1.	Definitions	6

3.2.	Symbols	6

3.3.	Abbreviations	7

4.	Introduction	7

5.	UE Power Saving Schemes	8

5.1.	UE adaptation to the traffic and power consumption characteristic	8

5.1.1.	Adaptation to the variation in frequency	8

5.1.2.	Adaptation to the variation in time	9

5.1.3.	Adaptation to number of antenna	11

5.1.4.	Adaptation to DRX operation	12

5.1.5.	Adaptation to achieve reducing PDCCH monitoring/decoding	14

5.1.6.	UE assistance Information	16

5.2.	Power saving signal/channel/procedure for triggering adaptation to UE power Consumption	17

5.2.1.	Power saving signal/channel	17

5.2.2.	Power saving procedure	17

5.2.3.	Additional RS	18

5.3.	Power consumption reduction in RRM measurements	18

5.3.1.	Adapting/Relaxing RRM measurement in time domain	19

5.3.2.	Adapting/Relaxing intra-frequency measurements	19

5.3.3.	Power Saving Schemes for RRM measurements with Additional Resource	20

6.	Higher Layer Procedure for UE Power Saving	21

6.1.	UE paging procedure based on power saving signal/channel/procedure	21

6.2.	UE power saving procedure in transition from RRC_CONNECTED to RRC_IDLE/RRC_INACTIVE state	21

7.	Conclusions	21

8.	Appendix – Evaluation Methodology	21

8.1.	UE power consumption model	21

8.1.1.	UE power consumption model for FR1	21

8.1.2.	UE power consumption model for FR2	23

8.1.3.	UE power consumption scaling for adaptation	24

8.1.4.	UE power consumption model for RRM measurements	26

8.1.4.1.	For intra-frequency measurements:	26

8.1.4.2.	For inter-frequency measurement	27

8.2.	Simulation Assumptions	28

Annex <X>: Change history	33

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

- Scope

The present document captures the findings of the study item, "Study on UE Power Saving in NR" [2].  The purpose of this TR is to study the UE power saving framework taking into consideration of latency and performance in NR as well as network impact.  The study of UE power saving in NR includes the study of the power saving schemes and the associated procedures.  The power saving schemes are to study the UE adaptation to the traffic and UE power consumption characteristics in frequency, time, antenna domains, DRX configuration, and UE processing timeline for UE power saving.   The power saving signal/channel/procedure is used for triggering adaptation of UE power consumption characteristic.  The power saving study also considers UE power consumption reduction in RRM measurements.  The associated procedures for the power saving schemes include the enhancement of UE paging procedure based on the additional power saving signal/channel/procedure and the UE power saving procedure in supporting efficient transition from RRC_CONNECTED to RRC_IDLE/RRC_INACTIVE mode.

This document is a 'living' document, i.e. it is permanently updated and presented to TSG-RAN meetings.

- References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

- 3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

- 3GPP RP-181463: “New SID: Study on UE Power Saving in NR”.

- 3GPP TR 38.913: “Study on Scenarios and Requirements for Next Generation Access Technologies”.

- 3GPP TR 38.802: “Study on New Radio Access Technology - Physical Layer Aspects”,

- 3GPP TS 38.814: " Physical layer procedures for data”

- R1-1813684	Calibration results for power saving study		Huawei, HiSilicon

- R1-1812329	Remaining aspects for evaluation methodology for power saving		vivo

- R1-1812360	Evaluation methodology for UE power saving		MediaTek Inc.

- R1-1813960	Discussion on evaluation methodology for UE power consumption		LG Electronics

- R1-1812640	Evaluation Methodology for UE Power Saving Scheme		CATT

- R1-1813010	Evaluation Methodology for UE power saving		Samsung

- R1-1813181	Evaluations and modeling of UE power consumption		Ericsson

- R1-1813446	UE Power Saving Evaluation Methodology		Qualcomm Incorporated

- R1-1814014	Evaluation methodology for UE power saving techniques		Intel Corporation

- R1-1814080	Consideration on UE power consumption model and preliminary evaluation results	ZTE Corporation

- R1-1900040	Power saving schemes	Huawei, HiSilicon

- R1-1900145	Discussion on UE adaptation to the traffic and UE power consumption characteristics		vivo

- R1-1900192	Adaptation designs for NR UE power saving		MediaTek Inc.

- R1-1900226	On adaptation aspects for NR UE power consumption reduction		ZTE Corporation

- R1-1900305	UE Adaptation to the Traffic and UE Power Consumption Characteristics		OPPO

- R1-1900344	UE Power saving schemes with adaptation		CATT

- R1-1900380	Power consumption adaptation for NR	sony

- R1-1900421	Discussion on UE power saving schemes with adaption to UE traffic	CMCC

- R1-1900508	UE adaptation to the traffic and UE power consumption characteristics	Intel Corporation

- R1-1900599	Discussion on UE adaptation to UE power consumption characteristics	LG Electronics

- R1-1900719	Discussion on UE adaptation to the traffic and UE power consumption characteristics	Spreadtrum Communications

- R1-1900753	Power saving techniques based on UE adaptation	Apple Inc.

- R1-1900813	On power saving techniques 	InterDigital, Inc.

- R1-1900818	PDSCH resource allocation restriction for power saving	ASUSTEK COMPUTER (SHANGHAI)

- R1-1900911	UE Adaptation to the Traffic and UE Power Consumption Characteristics	Qualcomm Incorporated

- R1-1900947	UE power saving schemes	Motorola Mobility, Lenovo

- R1-1900980	Discussion on UE adaptation to the traffic and UE power consumption characteristics	NTT DOCOMO, INC.

- R1-1900992	Power saving during scheduling opportunity is sparse	NEC Corporation

- R1-1901087	On UE adaptation schemes	Samsung

- R1-1901118	Discussion on UE traffic adaptation and power consumption characteristics	PANASONIC

- R1-1901166	Adaptation aspects of NR UE power saving	Ericsson

- R1-1901188	On UE adaptation to the traffic	Nokia, Nokia Shanghai Bell

- R1-1900039	Remaining issues for evaluation methodology for UE power saving	Huawei, HiSilicon

- R1-1900144	Evaluation Results and remaining issues on evaluation methodology for UE power saving		vivo

- R1-1900191	Evaluation of NR power saving designs	MediaTek Inc.

- R1-1901282	Evaluation results of UE power saving	 (Revision of R1-1900225) 	ZTE Corporation

- R1-1900304	Evaluation methodology for UE power saving	OPPO

- R1-1900343	Evaluation Results of UE Power Saving Schemes	CATT

- R1-1900379	Evaluation results for power saving study	sony

- R1-1900507	Evaluation results of UE power saving signals	Intel Corporation

- R1-1900598	Numerical results for UE power saving schemes	LG Electronics

- R1-1900752	Evaluation of UE power saving techniques	Apple Inc.

- R1-1900910	Evaluation Methodology and Results for UE Power Saving	Qualcomm Incorporated

- R1-1901086	Evaluation results for UE power savings	Samsung

- R1-1901165	Evaluations and modeling of UE power consumption	Ericsson

- R1-1901187	Initial evaluation results and consideration on DRX settings	Nokia, Nokia Shanghai Bell

- R1-1900041	Power saving signal/channel/procedure for triggering adaptation  Huawei, HiSilicon

- R1-1901245	Analysis and evaluation of go-to-sleep signaling scheme Huawei, HiSilicon.

- R1-1901246	Analysis and evaluation of wake up signaling scheme Huawei, HiSilicon.

- R1-1900754    Power Saving Techniques based on Explicit Network Indication, Apple

- R1-1900381	Power Consumption Triggering for NR, Sony

- Definitions, symbols and abbreviations

  - Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

  - Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

  - Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

PDCCH	Physical Dedicated Control CHannel

- Introduction

UE battery life is an important aspect of the user’s experience, which will influence the adoption of 5G NR handsets and/or services. It is critical to study UE power consumption to ensure that UE power efficiency for 5G NR UEs can be better than that of LTE, and techniques and designs for improvements are identified and adopted.

ITU-R defines energy efficiency as one of the minimum technical performance requirements for IMT-2020. According to ITU-R report – Minimum requirements related to technical performance for IMT-2020 radio interface(s), “energy efficiency of the device can relate to the support for the following two aspects: a) Efficient data transmission in a loaded case; b) Low energy consumption when there is no data. Efficient data transmission in a loaded case is demonstrated by the average spectral efficiency. Low energy consumption when there is no data can be estimated by the sleep ratio”.

Because NR system supports high speed data transport, the bursty user data would be served by network in very short durations.  One efficient UE power saving mechanism is to trigger UE for network access from power efficient mode.     UE would stay in the power efficient mode, such as micro sleep or OFF period in the long DRX cycle, unless it is informed of network access through UE power saving framework.    Alternatively, network can assist the UE to switch from the “network access” mode to the “power efficient” mode when there is no traffic to deliver, e.g. dynamic UE transition to sleep with network assistance signal.

In addition to minimizing the power consumption with the new wake up/go-to-sleep mechanism, it is equally importance to reduce the power consumption during the network access in RRC_CONNECTED mode.  More than half of the power consumption in LTE is UE in the access mode.   The power saving scheme would focus on minimizing the dominate factor of power consumption during the network access, which includes the processing of aggregated bandwidth, number of active RF chains and active reception/transmission time, and dynamic transition to power efficient mode.   From the field data collected in the LTE network, most of subframes contain no data or small data.   The power saving scheme for the dynamic adaptation to the different data arrival should be studied in RRC_CONNECTED mode.      Dynamic adaptation to traffic in different dimensions, such as carrier, antenna, beamforming, and bandwidth, would be studied in the studied.  Furthermore, methods to enhance the transitions between “network access” mode and power efficient mode should be considered.   Both network-assisted and UE-assisted approaches should be considered for UE power saving mechanism.

UE also consumes a lot of power for RRM measurements.  In particular, UE would need to power up before the DRX ON period to track the channel in preparation for the RRM measurement.  Some of the RRM measurements are not necessary but consumes a lot of UE power, for example, the low mobility UEs does not have to measure as frequent as   high mobility UEs. Network would provide the signalling to assist UE to reduce the power consumption on unnecessary RRM measurements. Additional UE assistance, for example the UE status information, etc, is also useful for the network to enable the UE power consumption reduction on RRM measurements.

The study of UE power saving is to identify the feasibility and benefit of techniques to allow UE implementations which can operate with reduced power consumption.   The study of the UE power saving framework would also take into consideration of latency and performance in NR as well as network impact.

- UE Power Saving Schemes

  - UE adaptation to the traffic and power consumption characteristic

    - Adaptation to the variation in frequency

The UE BWP adaptation is to dynamically switch the BWP by gNB based on, e.g., the traffic to support efficient operation of BWP switching in reducing the UE power consumption.

The UE adaptation to the CA/DC is to fast activation/deactivation of the SCell by gNB based on, e.g., the traffic to support efficient operation for fast SCell activation/deactivation in achieving UE power saving.

The UE power saving schemes for the UE adaptation in frequency domain for further study are as follows,

  - BWP -  UE adaptation to different BWP

    - RS to assist UE channel tracking and measurements to assist BWP switching

      - The assistance may also include CSI measurements (UE processes one BWP at a time)

    - Enhancement of L1 signaling, e.g., power saving signal or DCI for power saving, in triggering the BWP switching

    - Association of BWP and DRX

    - UE assistance information could be considered

  - CA/DC –

    - Quick activation/de-activation (e.g.,L1 signaling, MAC CE enhancement)

    - Adaptation of PDCCH monitoring/search space on activated SCell

      - Including cross carrier scheduling

    - Power adaptation based on the operation in a group of cell in power efficient way

    - CSI/RRM measurements and beam management at non-active SCell

    - UE assistance information could be considered

- The following table is subject to further update, particularly regarding evaluation results/assumptions

Table 1: Power saving schemes with UE adaptation to the BWP switching and fast SCell activation/deactivation

| Company | Power saving scheme | Power saving gain | Power saving gaing for each configuration | UPT/Latency | Estimated Overhead | Evaluation methodology/baseline assumption | Note (include UE throughput) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MediaTek [18][40] | BWP adaptation in combination of SCell activation | [30%-65%] |  |  |  | Traffic model: FTP/Video, IM , VoIP |  |
| CATT[21] | Power saving signal triggering BWP adaptation with on-demand power saving RS (PS-RS) | [5%-44%] |  |  |  | DRX cycle (40,160, 320) ms, FTP3 traffic model with traffic load (0.1, 0.8,3.2) |  |
| Apple [27][47] | UE feedbacks the preferred BWP UE-initiated Fast SCell activation | [Without GTS: 33~65% With GTS: 45%~80% 3%-43%] |  |  |  | FTP3 with size (0.1, 0.5, 2 and 4 Mbyte) | Power saving gain number is against fixed bandwidth part operation. |
| Samsung [34] | Cross-carrier scheduling among carriers in FR1 and FR2 | [40%-57%] |  |  |  |  |  |
| Qualcomm [[48] | SCell dormancy. Applicable to the following schemes: - Dormant BWP - SP search space config | [33%] |  |  |  | Method: SLS Traffic model: FTP3. 4CC | Traffic scales with number of carriers |

    - Adaptation to the variation in time

The UE power saving schemes with UE adaptation to the traffic arrival in the time domain includes cross slot scheduling, single slot scheduling with a gap between PDCCH and PDSCH reception, and multi-slot scheduling.  For both cross-slot scheduling and single slot scheduling, the UE may achieve power consumption by switching to micro sleep after PDCCH reception.  For multi-slot scheduling, the UE may achieve power consumption by skipping the PDCCH monitoring at subsequent slots of PDSCH/PUSCH transmission.

- Cross-slot scheduling

    - Minimum K0 > 0 and aperiodic CSI-RS triggering offset is not within the duration - UE could switch to micro sleep after PDCCH reception – no addition PDSCH and CSI-RS signals reception within the given duration (e.g. the same slot)

      - It is known to the UE at PDCCH decoding

      - Extended micro sleep time and reduce the PDCCH processing in reducing UE power consumption

      - Minimum K2 > 0 is essential to avoid the requirements of fast PDCCH processing

  - The general procedure for the study of the power saving scheme when cross-slot scheduling is used

      - gNB semi-statically configures TDRA to the UE, subject to UE capability (if any)

      - All schedulable TDRA values have K0 > = x and K2 >= x where x > 0

        - Determination of value x is FFS (which may also be done in the WI phase), e.g., may also be impacted by BWP switching triggered by DCI (jointly with cross-slot scheduling, if supported), etc.

      - All aperiodic CSI-RS triggering offsets are not smaller than the value x

      - UE decodes PDCCH and retrieves the index of schedulable TDRA values

      - UE could go to micro sleep after reception of last PDCCH symbol

      - UE processes PDSCH at the indicated starting time from TDRA values

      - Note: DRX cycle assumed in the evaluation results summarized in the table below is not necessilary long DRX cycle; detailed DRX cycle assumption can be found in each reference

      - The following table is subject to further update, particularly regarding evaluation results/assumptions

Table 2: Power saving scheme with cross-slot scheduling

| Company | Power saving scheme | Power saving gain | Power saving gaing for each configuration | UPT/Latency | Estimated Overhead | Evaluation methodology/baseline assumption | Note (include UE throughput) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| HW[16] | Cross-slot scheduling with DRX, inactivity timer configuration, and On Duration. | [17.3%-26.6%] |  |  |  | (DRX cycle, inactivityTimer, onDurationTimer) Case 1(160,100,8), Case 2(160,40,8), Case 3(40,25,4), Case 4 (40,10,4) | Higher power saving gain observed from longer DRX cycle |
| ZTE [19][41] | Cross-slot scheduling with K0 > 0 indicated to the UE in advance | [24%-25%] |  |  |  | DRX configuration ( 160-100-8) |  |
| CATT[21] | Cross-slot scheduling with K0 > 0 indicated to the UE in advance | [0.39%-16.63%] |  |  |  | DRX cycle (160, 320) ms, FTP3 traffic model with traffic load (0.1, 3.2, 5, 10) | Higher power saving gain observed from higher traffic load |
| Apple [27][47] | Cross slot scheduling K0>0 The control of TDRA | [0%-30%] |  |  |  | New power model for same slot scheduling |  |
| QC [30][48] | Configuration of minimum DL scheduling offset that applies to K0 and A-CSI triggering offset | [6%-26%(FR1) 9%-27%(FR2)] |  |  |  | Method: SLS No C-DRX Traffic model: FTP3 |  |
| Samsung [34] [ | Cross-slot scheduling | [30%] |  |  |  | No CDRX, Traffic model FTP 3, x=1 |  |
| Nokia, NSB [37][51] | Cross-slot scheduling K >0 | [25%] |  |  |  | DRX configuration (160, 40, 8) Trafic model FTP3 |  |

  - Same slot scheduling

  - Adaptation of TDRA configurations to achieve UE power saving – ensure the gap between PDCCH reception and PDSCH transmission known to the UE

  - Adjustment of TDRA configuration

    - Selection of TDRA entry in the TDRA table [5].

    - Note: cross-slot scheduling could be incorporated in the TDRA configuration

  - Power model for TDRA power saving scheme is the slot-averaged power for a slot with N-symbol PDSCH aligned to the end of the slot and PDCCH at the beginning of the slot reported by the proposed scheme.

- The general procedure for the study of the power saving scheme when same slot scheduling is used

  - gNB semi-statically configures TDRA to the UE, subject to UE capability (if any).

  - All schedulable TDRA values with K0=0 include at least Y symbols between the last PDCCH symbol and the first PDSCH symbol

    - Y is FFS

      - Note: the value of Y is necessary for power modelling in evaluation, although the specification (if specified) of Y value may also be done in the WI phase

    - Note: in case there is a TDRA entry with K0>0, it is assumed that the gap between the last PDCCH symbol in a first slot and the first PDSCH symbol in a second slot is no less than Y

  - All aperiodic CSI-RS triggering offsets are greater than Y symbols

  - UE decodes PDCCH and retrieves the index of schedulable TDRA values

  - UE could go to micro sleep after reception of last PDCCH symbol

  - UE processes PDSCH at the indicated starting time from TDRA values

  - The following table is subject to further update, particularly regarding evaluation results/assumptions

Table 3: Power saving scheme with same slot scheduling

| Company | Power saving scheme | Power saving gain | Power saving gaing for each configuration | UPT/Latency | Estimated Overhead | Evaluation methodology/baseline assumption | Note (include UE throughput) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Apple [27][47] | Same slot scheduling The control of TDRA | [15% ~ 30%] |  |  |  | New power model for same slot scheduling |  |

  - Multi-slot scheduling – PDCCH decoding in one slot (e.g., one DCI, multiple DCI) supports scheduled PDSCH/PUSCH transmission over multiple slots.

    - Achieving UE power consumption reduction by potentially skipping PDCCH monitoring at subsequent slots of PDSCH/PUSCH transmission.

  - The following table is subject to further update, particularly regarding evaluation results/assumptions

Table 4: Power saving scheme with multi-slot scheduling

| Company | Power saving scheme | Power saving gain | Power saving gaing for each configuration | UPT/Latency | Estimated Overhead | Evaluation methodology/baseline assumption | Note (include UE throughput) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| # ZTE [19][41] | Multi-slot scheduling | [0.65%-1.9%] |  |  |  | DRX configuration ( 160-100-8) |  |
| Samsung[34] | Multi-slot scheduling: one PDCCH schedules N TBs across N consecutive slots. | [0%-45%] | [N = 1-8] | [0 (for N=0) -1.8ms (for N=8)] |  | Baseline: N=1, PDCCH monitoring periodcity 1 slot, Trafic moddel FTP3, No C-DRX | The results shown also reflect combination with reduced PDCCH monitoring |

    - Adaptation to number of antenna

The UE power saving schemes with UE adaptation to the traffic is to reduce the number of antenna/panels or MIMO layers indicated by the network semi-statically or dynamically to achieve the UE power saving. Table 5: Power saving scheme with UE adaptation to the number of antenna/MIMO layer

| Company | Power saving scheme | Power saving gain | Power saving gaing for each configuration | UPT/Latency | Estimated Overhead | Evaluation methodology/baseline assumption | Note (include UE throughput) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| HW[16] | 2Rx for PDCCH-only slot. Between the two slots with PDSCH, within the timer, 4Rx is used for the PDCCH-only slot; For the PDSCH + PDCCH slot and PDSCH-only slot, 4Rx is used | [12%-25%] |  |  |  | FTP 3 with 0.1Mbytes and 0.5Mbytes C-DRX configurations are set to Case1(160, 10, 8), Case2(160, 40, 8) and Case 3(160, 100, 8) |  |
| ZTE[19] | Adaptive configuration with 2 or 4 antennas | [12%-14%] |  |  |  | FTP 3 with 0.1Mbytes CDRX configuration: 160-100-8 |  |
| MediaTek [18][40] | BWP-based adaptation to reduce RX number for PDCCH monitoring | [15%-20%] |  |  |  | Traffic model: FTP/Video, IM , VoIP |  |
| Apple [27][47] | UE is configured with maximum layer of 2 | [30%] |  |  |  | Continuous PDCCH monitoring |  |

    - Adaptation to DRX operation

# The power saving scheme of power saving signal/channel triggering UE adaptation to DRX operation is to configure the power saving signal/channel before or at the beginning of the DRX ON duration to trigger UE waking up only when there is DL data arrival.   UE is not required to wake up at the DRX ON at least for PDCCH monitoring.  if the power saving signal is not detected.   The go-to-sleep signaling is used as the indication allowing UE going back to sleep state after completion of PDSCH reception during the DRX ON period to further reduce the UE power consumption.

# For power saving scheme with UE adaptation to the DRX operation for further study

  - UE adaptation of its behavior to the DRX operation for UE power consumption reduction

    - When is configured with power saving signal/channel, power saving signal/channel as the indication whether to wakeup or not before or at the beginning of DRX ON duration

      - At least for the indication of PDCCH monitoring

    - Preparation period is used for (e.g., to perform channel tracking, CSI measurements, beam tracking).

      - Preparation period can be used in preparation for the PDCCH decoding

      - Preparation period could be before or during the DRX ON duration

      - Network can indicate UE to report CSI before or after the power saving signal/channel (if configured) during the preparation period

      - Network can indicate additional RS transmission (e.g., CSI-RS, TRS, SSB and power saving signal) at the preparation period

    - Go-to-sleep signaling as the indication allowing UE going back to sleep state

      - MAC-CE

      - DCI

      - Power saving signal/channel

    - Constraints on scheduling DCI during DRX_ON

  - The following table is subject to further update, particularly regarding evaluation results/assumptions

Table 6: Power saving scheme with UE adaptation to the DRX operation

| Company | Power saving scheme | Power saving gain | Power saving gaing for each configuration | UPT/Latency | Estimated Overhead | Evaluation methodology/baseline assumption | Note (include UE throughput) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ZTE [19][41] | One stage and two stage power saving signal triggering UE wake up Power saving signal triggering UE go to sleep | [6%-27% 28%-50%] |  |  |  | DRX configuration (160-10-8,160-40-8,160-100-8) DRX configuration (40-10-4, 40-25-4, 160-40-8, 160-100-8, 320-80-10, 320-200-10) |  |
| OPPO[20][42] | WUS triggering DRX | [4%-71%] |  |  |  | Traffic model: FTP3, IM |  |
| CATT[21] | Power saving signal triggering UE wakeup before DRX ON with on-demand PS-RS at pre-processing window | [36%-79%] |  |  |  | DRX cycle (40,160, 320) ms, FTP3 traffic model with traffic load (0.1, 0.8,3.2) |  |
| Intel [45] | Power saving signal triggering UE wakeup and go-to-sleep signal triggering UE back to sleep | [9%-73% for WUS 52%-82% for GTS ] |  |  |  | DRX configuration (40,10,4),(160, 100, 8), (320,80, 10) Traffic model: FTP3, IM, VoIP, web browsing, video streaming |  |
| LG[46] | WUS signal triggered DRX with inactivity timer running | [1%-75%] |  |  |  | Traffic model: FTP3 with file size 0.5 Mbyte | UPT loss |
| QC[30][48] | PDCCH-based WUS signal triggering UE wakeup | [2%-70%] |  |  |  | Method: Numerical simulation DRX configuration (40,10) (40,25) (160,100) Traffic model: FTP3, IM, Web browsing | CSI measurements in pre-processing before the 1st PDSCH reception |
| Samsung [34] | 1-bit Power Saving signal triggering UE wakeup at DRX ON | [0%-90%] |  |  |  | DRX configurations DRX cycle:80/160/320ms Inactivity Timer: 10-100ms OnDuration:2-8ms) Baseline : DRX cycle:160ms, Inactivity timer:100ms, on Duration:8ms |  |
| Huawei [52][53][54] | Power saving signal triggering UE wake-up or power saving signal triggering go-to-sleep signal | [WUS: up to 40% GTS: (40% - 85% for packet size 0.1Mbytes, 35% - 75% for 0.5Mbytes)] |  |  |  | DRX cycle (320/160/40), Inactivity timer(200/100/80/40/25/10) On duration timer (10/8/4) Packet sizes 0.1/0.5Mbytes |  |
| MediaTek [40] | Power saving signal triggers DRX OnDuration, where power saving signal is allocated in the preparation windown allowing periodic BM/CSI report and/or SRS transmission around a SS burst | [9%-30%] |  |  |  | Traffic model: FTP3, IM, VoIP |  |
| Apple[55] | PDCCH based Go-to-sleep signal | [GTS 37%~71%] |  |  |  | FTP3 traffic (inter-arrival time = 2.5ms, 100ms) | With no DRX |
| Sony [56] | WUS triggering wakeup for DRX | [7-11% for PDCCH-based WUS 11-20% for low power WUS] |  |  |  | FTP3 traffic. DRX = {160,100,8} and {160,40,8} |  |

  - Dynamic DRX configuration including at least the following

    - UE is configured with multiple DRX configurations

    - Dynamic selection of DRX configuration by gNB from multiple DRX configurations (e.g., traffic, mobility)

      - UE assistance information may be considered

    - Adaptive parameters setting of one DRX configuration

      - UE assistance information may be considered

      - DRX parameters are indicated by gNB

      - Adaptive UE behavior in the DRX operation (e.g, restart the inactivity timer)

- The following table is subject to further update, particularly regarding evaluation results/assumptions

Table 7: Power saving scheme with multi-DRX configurations

| Company | Power saving scheme | Power saving gain | Power saving gaing for each configuration | UPT/Latency | Estimated Overhead |  | Evaluation methodology/baseline assumption | Note (include UE throughput) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HW[16] | DCI indicated Inactivity timer | [7.6%-42.6%] |  |  |  |  | DCI to trigger the restart of the Inactivity timer |  |
| MediaTek [18][40] | Adaptative DRX configuration based on traffic type | [42%-70%] |  |  |  |  | DRX configuration based on the traffic type |  |
| OPPO[20][42] | Adaptation of Inactivity timer | [38%-46%] |  |  |  |  | Traffic model: FTP3, IM |  |
| Sony[22][56] | DRX with dynamic inactivity timer Dynamic DRX indication | [20%-55% 7%-11%] |  |  |  |  | (1) DRX configuration (320,200, 10) traffic model: FTP3, IM (2): DRX configuration (160,100,10) and (160,40,10). Traffic model: FTP3 |  |
| Nokia, NSB [37][51] | Static DRX configuration (short DRX embedded in long DRX as per 38.321 Section 5.7) | [10%-65%] |  |  |  |  | DRX configuration LongCycle: 320, 160, 20 drx-InactivityTimer 200, 25, 20, 10, 4 drx-onDurationTimer: 10, 8, 4, 2 drx-ShortCycle: 40, 25, 4 drx-shortCycleTimer: 16, 8, 4 Trafic model FTP3 | The case for the evaluations would change one parameters at a time |

    - Adaptation to achieve reducing PDCCH monitoring/decoding

The UE power consumption can be reduced when the number of UE PDCCH monitoring occasions and/or the number of PDCCH blind decoding is reduced.

The power saving schemes to reduce PDCCH monitoring and blind decoding for further studies are as follows,

  - Triggering of PDCCH monitoring – dynamic trigger through L1 signal/signaling

    - Power saving signal triggering PDCCH monitoring

    - Go-to-sleep signaling to skip PDCCH monitoring

  - PDCCH skipping -

    - DCI based indication  for PDCCH skipping (e.g., indication in DCI content, new SFI state).

    - L1 signal/signaling (other than DCI) based triggering  -

  - Multiple CORESET/search space configurations

    - Configuration of different PDCCH periodicities with dynamic signaling

    - Adaptation of CORESET/search space configuration – DCI/timer/HARQ-ACK based indication

    - Dynamic/semi-persistent CORESET/search space ON/OFF

    - Adaptation between DRX ONduration timer and inactivitytimer

    - Separated PDCCH monitoring of DL and UL

  - L1 signaling triggering to assist  UE in reducing the number of PDCCH blind decoding

  - Reduced PDCCH monitoring on SCell (including cross carrier scheduling)

  - Network assistance –  RS is dynamically transmitted based on the need to assist UE performing synchronization, channel tracking, measurements and  channel estimations before PDCCH decoding

- The following table is subject to further update, particularly regarding evaluation results/assumptions

Table 8: Power saving scheme with UE adaptation to reduce PDCCH monitoring

| Company |  | Power saving scheme | Power saving gain | Power saving gaing for each configuration | UPT/Latency | Estimated Overhead | Evaluation methodology/baseline assumption | Note (include UE throughput) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Vivo [39] | (1)Dynamic switching of PDCCH monitoring periodicities by DCI (2)DCI explicit indication of PDCCH monitoring occasions within a periodicity (3)UE monitoring of PDCCH search space for retransmission is triggered by decoding failure of SPS PDSCH |  | [2%-93%] | [(1) 27%-91% (for gaming), 2%-93% (for FTP3); (2) 57%-85% (for gaming), 2%-93% (for FTP3); (3) 20%] |  |  | numerical simulation Traffic model: Gaming and FTP3 for case (1)(2), VoIP for case (3) | The Rel-15 baseline schemes are: PDCCH monitoring with and without C-DRX for case (1)(2) UE monitoring of PDCCH search space for retransmission is configured by RRC for case (3) |
| MediaTek [18][40] | Before data arrival - Switching on/off PDCCH monitoring for next DRX on duration - Adapt PDCCH monitoring period to the data traffic - Adapt scheduling delays, including K0 and K2, to the data traffic - Additional adaptation to DRX After data arrival - Adapt PDCCH monitoring period during data inactivity - Adapt scheduling delays, including K0 and K2, during data inactivity - Combined processing time adaptation |  | [5%-44%] |  |  |  | Traffic model: FTP/Video, IM , VoIP | Background activity window to optimize the sleep time and power consumption by aggregating periodic CSI and power saving signal monitoring around SS burst can improve power saving gain. |
| ZTE [19][41] | Search space skipping |  | [15%-89%] |  |  |  | DRX configuration (40-10-4, 40-25-4, 160-40-8, 160-100-8, 320-80-10, 320-200-10) |  |
| CATT[21] | Power saving signal triggering PDCCH monitoring with on-demand PS-RS |  | [3%-35%] |  |  |  | DRX cycle (40,160, 320) ms, FTP3 traffic model with traffic load (0.1,0.8, 3.2) |  |
| Sony[22] | Reduced PDCCH monitoring while the inactivity timer is running |  | [21%-37%] |  |  |  | DRX configuration (160,100, 8) with FTP3 traffic model, file size 0.1Mbyte |  |
| LG[46] | Different PDCCH periodicity during DRX ON with inactivity timer |  | [22%-36%] |  |  |  | Traffic model: FTP3 with file size 0.5 Mbyte |  |
| QC[30][48] | PDCCH skipping |  | [9%-43%(FR1) 19%-60% (FR2)] |  |  |  | Method: SLS No C-DRX Traffic model: FTP3 |  |
| Samsung [34] | 1-bit Power Saving signal triggering PDCCH monitoring adaptation on search space configuration Reduction on PDCCH candidate in achieving PDCCH blind decoding reduction Power saving signal triggering SPS PDSCH reception |  | [41% 0% - 80% Up to 30% 29%-31%] |  |  |  | Traffic model: FTP3 with file size 0.1 Mbyte baseline: T_PDCCH = 0.5ms, D = 0.5 ms |  |
| Nokia, NSB [37][51] | Reduce PDCCH blind decoding |  | [3-24%] |  |  |  | DRX configuration (160, 40, 8) Trafic model FTP3 |  |
| OPPO[20][42] | PDCCH skipping |  | [49.3% and 75.52%] |  |  |  | System level simulation. The referrence Power configuration is DRX as agreed in previous meetings. Traffic model: FTP3 and IM |  |

	raffic, mobility) ynamic adapbe considerednts (UE processes one BWP at a time)saving schemes and make conclusion on each powe

    - UE assistance Information

The UE assistance information for any UE power saving scheme is for UE to provide the assistance information to the network in configuration for UE adaptation to achieve power saving gain. UE assistance information reported by UE for power saving is a UE recommendation and the network shall make the final decision on whether and how to use that information.

The UE assistance information for the power saving schemes for further studies are as follows,

  - UE assistance information/feedback to assist network in configurations for UE adaptation

    - UE preferred processing timeline parameters, e.g., K0, K1, K2 values

    - UE preferred BWP information/configuration

    - UE preferred antenna configuration, including MIMO layers, antenna panel awareness information

    - UE assistance/feedback on the DRX configurations/parameters

    - UE preferred BWP provided to assist network in BWP switching

    - UE request on SCell/SCG activation/de-activation/configuration

    - UE preferred PDCCH monitoring parameters/search space configuration/maximum number of blind decoding

# Other UE assistance information for the power saving schemes is not precluded.

  - Power saving signal/channel/procedure for triggering adaptation to UE power Consumption

    - Power saving signal/channel

# The power saving signal/channel for UE adaptation includes the following signals/channels for further study

- Existing signal/channel based power saving signal/channel

  - PDCCH channel

  - TRS, CSI-RS type  RS, SSS-like and DMRS

  - PDSCH channel carried MAC CE and/or RRC signaling

- New power saving signal/channel – sequence based

The aspects of the power saving signal/channel used for the UE adaptation to the traffic used for further evaluation of power saving signal design in addition to its triggering to the power saving gain.

- Network resource overhead

  - Resource and/or periodicity of power saving signal/channel

  - Multiplexing capability - include total number of UEs supported

  - Usage of resource

- Coexistence/multiplexing with existed signal/channel of Rel-15

- UE-specific, group-specific, cell-specific power saving signal/channel

- Detection performance

- Complexity

- Power consumption of the power saving signal/channel

- The behaviour when miss detection/false alarm happens

- Multiplexing with other signals/channels

- Number of information bits

The performance evaluation of the power saving signal/channel should target the miss detection at X% and the false alarm rate at Y% with the following aspects identified for the proposed power saving signal/channel

- The target of miss detection X% and the false alarm rate at Y% as baseline for evaluation

    - For power saving signal/channel for wake-up purpose, X=[0.1] and Y=[1]

    - For power saving signal/channel for go-to-sleep purpose, X=[1] and Y=[0.1]

    - Additional X and Y values are not precluded for the proposed power saving signal/channel based on the use cases and scenarios

    - For any other purpose(s) of power saving signal/channel, companies to report X & Y values

- The target of miss detection would be different depending on the behaviour of miss detection of power saving signal/channel.

    - If miss detection behaviour is defined as no subsequnt PDCCH reception, low miss detection rate is required in order to avoid increased latency of missed chance of the scheduling.

    - If miss detection behaviour is defined as subsequent PDCCH reception, low miss detection rate is not required. If the network is congested and power saving signal is not transmitted, UE behaves as if power saving signal is not configured.

- The miss detection performance when multiple power saving signal/channel are multiplexed on the same resource, when applicable

- The performance of the power saving signal/channel should assume realistic implementation limitations, e.g., by using realistic channel estimation and time/frequency offset estimation, etc.

    - Power saving procedure

The power saving signal/channel in triggering UE adaptation to DRX operation for further study is that the power saving signal/channel can be configured along the DRX configuration as the indication for UE to wake up from the sleep state.  RS resources can be considered to assist UE in performing RRM/CSI measurement and channel time/frequency and/or beam tracking.

The power saving signal/channel candidate in triggering UE to achieve reducing PDCCH monitoring for further study  is that the power saving signal/channel can be used to trigger UE to skip the PDCCH monitoring and/or to go to sleep for a period of time.

The power saving signal/channel in triggering UE frequency domain processing adaptation for further study is the power saving signal/channel can be used to trigger the indication of RS configuration for channel tracking, CSI measurements, and beam management for the additional assistance of dynamic switching of BWP or activation of SCell in achieving the power saving gain.  The power saving signal/channel can be used for BWP switching, activation/deactivation of SCell or adaptation of PDCCH monitoring and/or CORESET/search space of PCell/SCell.

For further study, the power saving signal/channel in triggering adaption to the UE processing, such as MIMO configuration/layers, antenna configuration, UE processing time, and background processing can be used as the indication for UE adaptation.  The power saving signal/channel in triggering UE adaptation to the processing is to allow UE in reducing the power consumption by indication of the processing time, such as PDCCH/PDSCH/PUSCH/PUCCH processing or the essential background processing, such as periodic CSI and RRM measurements.

    - Additional RS

Additional RS is the RS provided by gNB to assist UE in performing e.g.,fine synchronization, channel/beam tracking, and/or CSI/RRM measurements in addition to the existing RS in Rel-15.  The additional RS was proposed for study for power saving schemes for e.g, UE adaptation to the DRX operation,  BWP switching, fast SCell activation, reducing PDCCH monitoring, and/or  RRM measurements.   RS design is assumed to reuse Rel-15 waveform.  Power saving signal could be used to meet the purpose of additional RS.

  - Power consumption reduction in RRM measurements

The RRM power saving study should consider RRC IDLE, INACTIVE and CONNECTED states.   The deployment scenarios for further studies of RRM power saving focus on the stationary (e.g., 0km/h), pedestrian (e.g., 3km/h) and vehicular (e.g., 30km/h) scenarios when considering UE power saving techniques for RRM measurements,

- For IDLE/INACTIVE state, at least the following power components are recommended to be considered for RRM measurement power saving evaluation

  - Loop convergence (AGC, TTL & FTL) / time-frequency tracking

    - How many SSB bursts are used for Loop convergence, with consideration of being potentially confined in the same SSB burst or different SSB bursts for serving cells and neighboring cells measurement/ time-frequency tracing

    - FFS: The power value for loop convergence /time-frequency tracking is as the same as SSB processing.

  - Paging

  - SIB1 decoding (PDCCH+PDSCH)

  - Neighbor cell search (within SMTC), if any

  - SSB measurement  (serving cell only / severing cell and neighboring cells, if any)

  - Sleep

- For CONNECTED state, at least the following power components are recommended to be considered for RRM measurement power saving evaluation,

  - Loop convergence (AGC, TTL & FTL) / time-frequency tracking

    - How many SSB bursts are used for Loop convergence, with consideration of being potentially confined in the same SSB burst or different SSB bursts for serving cells and neighboring cells measurement/ time-frequency tracing

    - Based on SSB and/or TRS(if configured)

  - PDCCH-only monitoring during active time

  - SSB measurement  (serving cell only / severing cell and neighboring cells, if any)

  - Neighbor cell search (within SMTC), if any

  - Sleep

The following adaption mechanism of RRM measurement activities for UE power saving:

- gNB controlled RRM measurement operation with UE assistance information reported to gNB, e.g.,

  - mobility related information (e.g., mobility state, history of mobility state, UE's visited cells and cells not reselected due to the ping-pong effect, the number of handovers for certain period, etc.)

  - channel condition (e.g. change in serving RS/signal)

- gNB controlled RRM measurement operation without UE assistance information reported to gNB based on certain conditions, e.g.,

  - Doppler estimation for RRC CONNECTED states

  - cell type (e.g., small cell/macro cell)

- gNB controlled threshold to support UE autonomous RRM measurement adaptation based on e.g.,

  - signal measurements (e.g., RSRP)

  - UE mobility state (e.g., low/medium/high mobility)

  - UE location in the cell (e.g., cell-center/cell-edge)

  - S-measure enhancement (e.g. S-measure for SCell, CSI-RS)

- Other mechanisms/approaches are not precluded

    - Adapting/Relaxing RRM measurement in time domain

- It is observed that the following

  - For certain conditions (e.g., low mobility deployment/UE speed/favorable RSRP conditions), the number of RSRP measurement samples for a given duration (e.g., measurement period / evaluation period) can be relaxed with negligible impact on accuracy achieved by existing Rel-15 measurement.

  - For certain conditions (e.g., favorable RSRP conditions, etc.), reducing RRM measurement activities (e.g., measurement, reporting) for a given time period is beneficial from UE power saving perspective for RRC IDLE/INACTIVE/CONNECTED states.

- For intra frequency and/or inter frequency measurement, the following approaches are to be studied for UE power saving in time domain, including impact on mobility performance

  - Increasing measurement period

  - Reducing number of samples (e.g., OFDM symbols / slots) within a measurement period (e.g., SMTC window)

  - Confining RRM measurements within a measurement window and increasing the periodicity of the measurement window for intra frequency and/or inter frequency measurement

  - Other approaches are not precluded

  - Note: this does not necessarily mean that the techniques studied will have spec impact

- For UE autonomous RRM measurement adaptation in time domain with gNB controlled threshold, the following thresholds and corresponding adaptation schemes can be considered,

  - A RSRP threshold for UE adapting RRM measurement period,

  - A RSRP threshold for UE adapting RRM number of samples within a measurement period,

  - A RSRP threshold and a RSRP variation threshold within a period of time, and based on that, UE can adapt the measurement or report period

  - A RSRP variation threshold within a period of time, and based on that, UE can adapt the measurement or report period

  - A threshold for UE adapting RRM measurement period and the threshold can be at least one of the followings,

    - The amount of time during which the UE stays with a specific cell or beam (for RRM measurement)

    - UE’s active TCI state for PDCCH does not change for specific time period.

    - The number of handovers/reselections for certain period.

  - A threshold which includes UE mobility status, serving cell quality, and based on that, UE can adapt the SMTC window, number of CSI-RS resources sets per target cell, periodicity of CSI-RS resources.

    - Adapting/Relaxing intra-frequency measurements

It is observed that the following mechanisms can be beneficial in achieving the UE power saving,

- Reducing the number of cells for intra-frequency measurement can be beneficial for UE power saving,

  - Assuming that UE can limit the processing for measurement within a constrained time period and/or with reduced complexity.

  - Assuming number of neighbouring cells to be measured is reduced.

- For UE power saving perspective, reducing the need in neighbour cell intra-frequency measurement can be beneficial.

    - Power Saving Schemes for RRM measurements with Additional Resource

While it is observed under certain conditions and deployment scenarios, additional resource for RRM measurement can be beneficial for UE power saving, including at least the following aspects:

- Minimizing/reducing the timing gap between measurement (e.g., SSB) and DRX ON duration (e.g., paging monitoring occasion/reception, data reception, etc.)

- Additional resource around the measurement occasion, e.g., for AGC assistance

- Reducing measurement activities by providing additional resource may provide sufficient measurement/T-F accuracy.

- The followings can be considered as the usage of additional resource for RRM measurement,

  - CSI-RS, including TRS

  - SSS only

  - SSB

  - PSS, SSS and wake-up signaling/paging

    - E.g., including transmitted in SFN (single frequency network) manner

  - DMRS for RMSI PDCCH/PDSCH for standalone

  - Additional new RS/signal (e.g, configuring additional RS next to SSB) in addition to existing RS/signal in Rel-15

  - Note: Existing structure/waveform of Rel-15 signals/channels is recommended to be studied for additional resource

  - Note: companies report the purpose of additional resource for RRM measurement, e.g., measurement, cell search and etc.

- RRC state(s) with the additional resource used for the RRM measurements,

  - RRC IDLE

  - RRC INACTIVE

  - RRC CONNECTED

- Higher Layer Procedures for UE Power Saving

  - UE paging procedure based on power saving signal/channel/procedure

The power saving signal/channel related paging is down-prioritized in the study.

NOTE: It is FFS if further evaluations are required.

Extending the DRX cycle length to 10.24s in idle and inactive mode will be considered. Increasing it above 10.24s will not be studied.

  - UE power saving procedure in transition from RRC_CONNECTED to RRC_IDLE/RRC_INACTIVE state

  - Higher Layer Procedures for the UE Power Saving Schemes in RRC_CONNECTED

The higher layer procedure for the UE power saving schemes includes the required signalling and procedures (when needed) for the proposed power saving schemes in Section 5.

The power saving signal/channel scheme for wake-up purpose is considered jointly with c-DRX. The higher layer procedure in support of the power saving signal/channel scheme for wake-up purpose should be studied for the power saving signal/channel scheme adaptation to DRX operation (Section 5.1.4).

  - Higher Layer Procedures for power consumption reduction in RRM measurements

Relaxing the serving and neighbour cell measurements for NR UE should be studied, considering the mobility related aspects.

- Conclusions

- Appendix – Evaluation Methodology

  - UE power consumption model

The following power states and relative power consumption values for the reference configuration are adopted for the study of UE power saving.

    - UE power consumption model for FR1

Reference Configuration for FR1

- Downlink: TDD

  - Subcarrier spacing (SCS): 30 kHz

  - Number of carrier:  1CC,

  - System Bandwidth: 100 MHz

  - PDCCH region of 2 symbol at beginning of a slot,

    - k0 = 0,

    - maximum number of CCEs = 56,

    - 36 PDCCH blind decoding,

  - PDSCH of max data rate

    - Modulation:  256QAM

    - MIMO configuration: 4x4 MIMO,

    - Number of RBs for TRS = 52,

    - 4RX UE Capability =1

- Uplink: TDD

  - Subcarrier spacing (SCS): 30 kHz SCS,

  - Number of carrier: 1CC,

  - System Bandwidth: 100MHz,

  - Tx antenna configuration: 1TX,

  - Power levels: 0dBm and 23dBm

- Power values are averaged over the operations within a slot.

- Different and/or additional parameter assumptions used for evaluation should be clearly stated.

| Power State | Characteristics | Relative Power |
| --- | --- | --- |
| Deep Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | 1 (Optional: 0.5) |
| Light Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. | 20 |
| Micro sleep | Immediate transition is assumed for power saving study purpose from or to a non-sleep state | 45 |
| PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 100 |
| SSB or CSI-RS proc. | SSB can be used for fine time-frequency sync. and RSRP measurement of the serving/camping cell. TRS is the considered CSI-RS for sync. FFS the power scaling for processing other configurations of CSI-RS. | 100 |
| PDCCH + PDSCH | PDCCH + PDSCH. ACK/NACK in long PUCCH is modeled by UL power state. | 300 |
| UL | Long PUCCH or PUSCH. | 250 (0 dBm) 700 (23 dBm) |

Table 9: UE power consumption model for FR1

- FFS: Power saving signal processing power and transition energy.

- The following additional transition energy and total transition time for the three sleep types are adopted as working assumption for power saving SI:

  - Both ramp down and ramp up transitions are included.

  - Additional processing time for acquiring synchronization is not included in total transition time.

  - There is always a non-sleep power state between adjacent sleeps.

  - FFS: Specify the times and relative power values for the ramp down and ramp up transitions separately.

| Sleep type | Additional transition energy: (Relative power x ms) | Total transition time |
| --- | --- | --- |
| Deep sleep | 450 | 20 ms |
| Light sleep | 100 | 6 ms |
| Micro sleep | 0 | 0 ms* |
| * Immediate transition is assumed for power saving study purpose from or to a non-sleep state |  |  |

Table 10:  UE power consumption during the state transistion

Figure 1:  Illustration of UE power consumption at state transition

    - UE power consumption model for FR2

Reference configuration for FR2:

- Downlink: TDD, FR2,

  - Subcarrier spacing (SCS): 120 kHz

  - Number of carriers: 1CC,

  - System Bandwidth: 100 MHz

  - PDCCH region of 2 symbol at beginning of a slot

    - k0 = 0

    - maximum number of CCEs = 32

    - 20 PDCCH blind decoding

  - PDSCH of max data rate with

    - Modulation: 64QAM

    - MIMO configuration: 2x2

    - Number of RBs for TRS = TBD

    - 2RX UE Capability = 1

- Uplink: TDD, FR2,

  - Subcarrier Spacing (SCS): 120 kHz

  - Number of carrier = 1CC,

  - System Bandwidth = 100MHz

  - Tx antenna configuration: 1TX chain

  - Power levels: [FFS dBm]

- Power values are averaged over the operations within a slot

- Different and/or additional parameter assumptions used for evaluation should be clearly stated.

| Power State | Characteristics | Relative Power |  |
| --- | --- | --- | --- |
|  |  | FR1 | FR2 |
| PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 100 | 175 |
| SSB or CSI-RS proc. | SSB can be used for fine time-frequency sync. and RSRP measurement of the serving/camping cell.. TRS is the considered CSI-RS for sync. FFS the power scaling for processing other configurations of CSI-RS. (Note 2 SSBs in a slot for the ref. config.) | 100 | 175 |
| PDCCH + PDSCH | PDCCH + PDSCH. ACK/NACK in long PUCCH is modeled by UL power state. FFS the power scaling for PDSCH-only slot. | 300 | 350 |
| UL | Long PUCCH or PUSCH. | 250 ( 0 dBm) 700 (23 dBm) | 350 (FFS Tx power level) |

Table 11: UE power consumption model for FR2

- Sleep states power level, energy overhead, and transitions are same as FR1.

    - UE power consumption scaling for adaptation

The power scaling scheme for FR1 and FR2 power states are in Table 4

| Scaling for FR1 | Proposal | Comment |
| --- | --- | --- |
| BWP Bandwidth (DL) | Scaling of X MHz = 0.4 + 0.6 * (X - 20) / 80. Linear interpolation for intermediate bandwidths. Valid only for X = 10, 20, 40, 80, and 100. Above scaling is applicable for FR1 only. In case scaling is needed for FR2, companies can report the assumed scaling factor. | For 10MHz BW, only AL up to 8 can be used for PDCCH The transition time is the same as DCI-based BWP switching delay for Rel-15. FFS: transition energy for BWP switching |
| BWP Bandwidth (UL) | No scaling at 0dBm or 23dBm Above scaling is applicable for FR1 only. In case scaling is needed for FR2, companies can report the assumed scaling factor. |  |
| CA (DL) | 2CC is 1.7x1CC 4CC is 3.4x1CC (i.e. 2x 2CC) Above refers to the worst case CA configuration in terms of power consumption. | Activation/deactivation delay follows RAN4 specification; FFS transition energy Applicable for FR1 and FR2 |
| CA (UL) | Same as downlink at 0dBm. No scaling at 23dBm 2CC is 1.2x1CC at 23dBm Limit scaling up to 2CC. | Applicable for FR1 and FR2 |
| Antenna scaling (DL) | 2Rx power is 0.7x 4Rx power for FR1 1Rx power is 0.7x 2Rx power for FR2 | Assume same number of antenna elements per Rx chain |
| Antenna scaling (UL) | 2Tx power is 1.4x 1Tx power at 0dBm. 1.2x.at 23dBm FR1 only | 2Tx support is not considered for FR2. |
| PDCCH-only | Power of cross-slot scheduling is 0.7x same-slot scheduling FFS for the scaling w.r.t. # of blinding decoding | Applicable for FR1 and FR2 |
| SSB | One SSB power is 0.75 of two SSB power, i.e. 75 power units |  |
| PDSCH-only slot | 280 for FR1 325 for FR2 | This assumes the same number of PDSCH symbols as in the PDCCH+PDSCH case. |
| CSI-RS | FFS for scaling w.r.t. # of symbols for CSI-RS FFS for neighbor cell measurement FFS for #measured cells |  |
| Short PUCCH | Short PUCCH power = 0.3 x uplink power Reference config consists of 1-symbol PUCCH | Applicable for FR1 and FR2. |
| SRS | SRS power = 0.3 x uplink power | Applicable for FR1 and FR2. |

Table 12: UE power consumption scaling for adaptation

- Note: Scaling applies only to non-sleep power states.

- Note: latency involved in antenna scaling is FFS

- For evaluation, when BWP transition duration is one slot or longer, the slot-average power level is 50 power units

  - In case when BWP transition duration is <one slot (if defined), up to each company to report the power level assumed

  - Companies to report the type of BWP transition time (type 1 and/or type 2) assumed in the evaluation

- The power scaling factors for BWP adaptation and number of antenna reduction are not intended to be applicable to the power states associated with RRM power modelling in section 8.1.4.

- For evaluation purpose, it is assumed that a periodicity of max(DRX cycle, [160 msec]) is the baseline for periodic activities, e.g. time/frequency, channel or beam tracking (if applicable)

  - Other periodicity values are not precluded – companies to report if other values are assumed

  - Companies to report detailed assumptions, e.g. the resources used, the relative timing relationship between DRX cycle and periodic activity, whether and how UL reporting is done, etc.

Power scaling scheme for PDCCH candidates processing reduction:

- Scaling for the power reduction due to PDCCH candidates processing (e.g. AL/CCE/BD) reduction is modelled solely based on its effect on micro sleep portion of the PDCCH-only slot

- The UE power scheme should include the portion of PDCCH processing time reduction in accordance to PDCCH candidates (e.g. AL/CCE/BD) reduction

  - Note: In the reference configuration, the first two symbols are PDCCH symbols

- For power scaling for PDCCH candidate reduction (for same slot scheduling only):

# P(α) = α ∙ Pt + (1 – α) ∙ 0.7Pt

where α is the ratio of PDCCH candidates to the max number of PDCCH candidates in the reference configuration (α>0). Pt is the PDCCH-only power for same-slot scheduling.

For UE power modelling, the following power scaling relationship is assumed:

- For simplicity,

  - The slot-averaged power for “PDCCH+PDSCH+PUCCH” is same as “PDCCH+PDSCH”

  - The slot-averaged power for “PDSCH+PUCCH” is same as “PDSCH-only”

  - The slot-averaged power for “PDCCH+PUCCH” is the sum of “PDCCH-only” power and “short PUCCH” power.

    - Note: PDCCH-only with cross-slot scheduling scaling is also applicable

  - Note: it is observed via evaluations (where 0dBm PUCCH Tx power is assumed) that the difference is not significant

- For non-RRM evaluation, for “PDCCH+PDSCH” concurrent in the same slot as “SSB or CSI-RS processing”, assume “PDCCH+PDSCH” power

- For non-RRM evaluation, for “PDSCH only” concurrent in the same slot as “SSB or CSI-RS processing”, assume “PDSCH only” power

- For “PDCCH-only” and “SSB or CSI-RS processing” concurrent in a slot, the slot-averaged power is 0.85x the sum of the respective power, at least for FR1

  - For FR1, 0.85*(100+100) = 170

  - For FR2, 0.85*(175+175) = [300]

- For “SSB processing” and “CSI-RS processing” concurrent in a slot, the slot-averaged power is 0.85x the sum of the respective power, at least for FR1

  - For FR1, 0.85*(100+100) = 170

  - For FR2, 0.85*(175+175) = [300]

  - 2-SSB within a slot is assumed

    - UE power consumption model for RRM measurements

Reference configuration and assumptions for RRM measurements:

- SSB periodicity is 20msec

  - 2 SSBs per slot are measured

- SMTC periodicity is 20msec

- For inter-band

  - Up to [2] frequency layers and

  - Gap pattern ID = 0 defined in TS 38.133 (i.e., measurement gap length of 6msec and measurement gap periodicity of 40msec)

Use the following as a reference for RRM power evaluation:

- Assume SMTC window duration of 2ms (corresponding to 30kHz SCS) for synchronised FR1 scenario.

- Assume SMTC window duration of 5ms  for all other cases

- Note: other values are not precluded

      - For intra-frequency measurements:

| N: Number of cells for intra-frequency measurement | Synchronous case |  | Asynchronous case |  |
| --- | --- | --- | --- | --- |
|  | FR1 | FR2 | FR1 | FR2 |
| N=8 | 150 | 225- | 170 | 285 |
| N=4 | 120 | 195 | 140 | 255 |

Table 13: UE power consumption for the RRM measurements

- All above values are slot-averaged power (Pfr1 or Pfr2 for FR1 or FR2 respectively).

- Synchronous case means actual SSB transmissions from cells are time-aligned e.g., timing of SSBi from celli is aligned with timing of SSBi from cellj.

- Asynchronous case means actual SSB transmissions from cells are not time-aligned e.g., timing of SSBi from celli is not aligned with timing of SSBi from cellj

- The following maximum number of cells for intra-frequency measurement within a slot:

| FR1 | FR2 |
| --- | --- |
| Nmax = 19 | Nmax = 9 |

Table 14:  Maximun numer of cells for intra-frequency measurements

# For combined measurement and search:

To obtain combined neighbor cell measurement and search power, add the difference between full neighbor cell search state power and SSB/CSI-RS proc. state power to the neighbor cell measurement power, as shown below:

| N: Number of cells for intra-frequency measurement & search | Synchronous case |  | Asynchronous case |  |
| --- | --- | --- | --- | --- |
|  | FR1 | FR2 | FR1 | FR2 |
| N=8 | 200 | 320 | 220 | 380 |
| N=4 | 170 | 290 | 190 | 350 |

Table 15: UE power consumption of the combined neighbor cell measurements and cell search

      - For inter-frequency measurement

The inter-frequency measurement is represented as function of the number of frequency layers:

UE needs to monitor Nf frequency layers within a measurement gap.

E3=i=0Nf-1Ei+Et∙Nf+1

Where

- Ei is either Pfr1*Ns (for FR1) or Pfr2*Ns (for FR2) for each frequency layer i

  - Ns is the number of slot over which measurements (for each frequency layer i) are carried out

- Nf is the number of frequency layers measured

- Et = Pt * Tt, where

  - Pt is the switching power consumption

    - [Assume micro sleep power for Pt]

  - Tt is 0.5ms for FR1 and 0.25ms for FR2 (from RAN4)

It can be simplified to the following if Ei is same across frequency layers (i.e. Ei = E for all i).

E3 = E*Nf + Et*(Nf+1)

Note: RAN4 requirement assumes only one frequency layer per measurement gap.

For full neighboring cell search:

The baseline neighbor cell search power is independent of the number of cells on the first order and can be approximated as:

| FR1 | FR2 |
| --- | --- |
| 150 | 270 |

Table 8: UE power consumption of neighboring cell search

- It is expressed as the power averaged over a slot during which search is performed.

  - Simulation Assumptions

Deployment scenarios

- Dense urban

  - Single layer

  - Additional deployment scenarios to be considered

    - Indoor hotspot

    - Rural macro

  - Note: For RRM measurement evaluation, UE power consumption contributed by inter-frequency and/or inter-RAT measurement can be considered.

Performance metrics

- UE power saving gain - percentage of  power consumption reduction of the proposed power saving scheme from the baseline scheme

  - FFS: For the case multiple applications are evaluated, whether power consumption is the overall DoU power across the applications

- System performance

  - Latency

  - Scheduling delay,

  - User throughput,

    - System throughput and/or resource utilization/overhead (if applicable) should be reported as the result of the evaluation, in addition to power saving gain.

- If a new signal/channel is introduced, the performance metrics include

  - Performance

  - Complexity

  - Overhead for reception of the signal/channel

- If the new signal is used for detection,  the performance metrics include

  - False alarm rate

  - Miss-detection rate

.

Link level simulation assumptions:

- Simulation assumptions as specified in Table A1.5-1 in TR38.802 [4] should be the basis for link-level simulation evaluation.

  - Antenna configuration may use IMT-2020 as reference.  The evaluation results should also include the antenna configuration if it is different from the reference antenna configuration,

- For LLS simulation on power saving signal detection performance, at least the following SINR values should be evaluated: -6dB, 3dB, 20dB.

System level simulation assumptions:

- Simulation assumptions as specified in Table A2.1-1 in TR38.802 [4] should be the basis for system-level simulation evaluation.

  - Antenna configuration may use IMT-2020 as reference. The evaluation results should also include the antenna configuration if it is different from the reference antenna configuration,

- It is clarified that user throughput measured from system level simulation does not preclude user perceived throughput taking into account TCP slow start if applicable

- For system level impact evaluation, IMT-2020 simulation assumptions can be adopted.

  - For system level simulation, whether to apply DRX depends on the particular power saving proposal being evaluated and the purpose of the simulation.  Whether DRX is not applied in the simulation should be identified along with the evaluation results

- For reducing simulation time for system level simulation, scaling of the bandwidth and packet size of the traffic model can be considered (e.g. factor of 1/5). Companies to report the scale factor used, if any.

- For system level simulation, at least assume that the traffic model is FTP model 3 with 0.5Mbyte payload and mean inter-arrival time of 200 milliseconds

  - The mean inter-arrival time can be adjusted to achieve different cell loading scenarios.

  - Note: Other variations including different traffic models are not precluded

- The same traffic model settings should be used across all users.

System Configurations:

Reference DRX configurations:

- C-DRX cycle 320msec, inactivity timer {200, 80} msec

  - FR1 On duration: 10 msec

  - FR2 On duration: 5 msec

- C-DRX cycle 160msec, inactivity timer {100, 40} msec

  - FR1 On duration: 8 msec

  - FR2 On duration: 4 msec

- C-DRX cycle 40msec, inactivity timer {25, 10} msec

  - FR1 On duration: 4 msec

  - FR2 On duration: 2 msec

- I-DRX cycle 1.28 sec

  - Group paging rate (for a PO): [10%]

    - P-RNTI is detected but PDSCH decoding results in no match

    - Note: Statistics for the matching case may be further considered based on use case

- Note: The selection and reporting of the settings for short DRX cycle, short DRX cycle timer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, and/or drx-HARQ-RTT-TimerUL should be included in the results of the UE power saving scheme

Traffic model used for the UE power saving scheme evaluation

- Applications with the traffic model for the evaluation of the UE power saving scheme

  - FTP - FTP model 3

    - Other bursty traffic arrival models can be considered

  - Web-browsing

  - Video streaming

  - Instant messaging

  - VoIP

  - Gaming

  - Background app sync

For FTP, instant messaging, and VoIP application, the following traffic models and DRX configuration should be included for evaluation:

|  | FTP traffic | Instant messaging | VoIP |
| --- | --- | --- | --- |
| Model | FTP model 3 | FTP model 3 | As defined in R1-070674. Assume max two packets bundled. |
| Packet size | 0.5 Mbytes | 0.1 Mbytes |  |
| Mean inter-arrival time | 200 ms | 2 sec |  |
| DRX setting | Period = 160 ms Inactivity timer = 100 ms | Period = 320 ms Inactivity timer = 80 ms | Period = 40 ms Inactivity timer = 10 ms |

Note: For ON duration setting, following reference DRX configurations as previously agreed.

- For web-browsing, video streaming, and gaming applications, the traffic models and the delay requirements defined in R1-070674 can be used in the evaluation. The parameters (e.g. packet size) may be updated to be in line with EMBB traffic requirements.

- For background app sync application, for power consumption evaluation purpose, it can be assumed that idle mode operations (inclusive of page detection, RRM, deep sleep and transition overhead) contributes to X% of the use case power. The remaining portion is contributed by intermittent RRC connections due to background activities (FFS: value of X)

- Companies should report the assumptions made in the evaluation

Evaluation methodology of the UE power saving schemes:

- Numerical analysis, system level simulation, and link level simulation are included as evaluation methods for power saving proposals. At least one of the methods should be selected and used for evaluation of a specific power saving proposal.

  - FFS: The criteria for selection of the methods

For power evaluation,

- For evaluation of DRX scenarios, the RRC release timer is set to infinity to ensure C-DRX and I-DRX cases are treated separately.

- Power consumption for channel/time/frequency and beam tracking operations should be taken into account.

Calibrations:

For the purpose of basic calibration of traffic modeling, FTP model 3 (use 0.1 Mbytes packet size, mean inter-arrival time 200msec) and VoIP model (as defined in R1-070674) should be used to generate time distribution for different power states, for the following scenarios

- No C-DRX configured

  - For both VoIP and FTP

- C-DRX cycle 40msec for VoIP

  - 10 msec inactivity timer

  - Assume max two packets bundled

- C-DRX cycle 160msec for FTP

  - 100 msec inactivity timer

The time distribution for different power states shall be reported (e.g. x% in PDCCH-only, y% in PDCCH+PDSCH, z% in micro sleep, etc), as a result of the calibration exercise.

The following simplifying assumptions can be made:

- Power modelling reference configuration for FR1

- Peak throughput. 100MHz

- DL BWP. 10-symbol PDSCH (one symbol occupied by DMRS)

  - Capable of carrying 868584 information bits per slot (Note: a packet can fit within a PDSCH transmission)

- All packets can be successfully decoded on the first transmission

  - No HARQ retransmission

- No UL slot

- Single user

- Short DRX is not configured.

Calibration results

|  | Power States | HW [6] | Vivo [7] | MTK [3] | ZTE [15] | Intel [14] | LGE [9] | CATT [10] | SS [11] | E/// [12] | QCOM [13] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FTP, w/o C-DRX | PDCCH only |  | 99.75% | 99.75% | 99.73% | 99.75% | 99.75% | 99.71% | 99.75% | 99.80% | 99.75% |
|  | PDCCH+PDSCH |  | 0.25% | 0.25% | 0.27% | 0.25% | 0.25% | 0.29% | 0.25% | 0.20% | 0.25% |
|  | Micro sleep |  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Light sleep |  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Deep sleep |  | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| FTP, w/ C-DRX | PDCCH only | 34.96% | 34.68% | 34.62% | 35% | 34.96% | 35.27% | 37.92% | 32.85% | 38.20% | 35.01% |
|  | PDCCH+PDSCH | 0.21% | 0.25% | 0.20% | 0.27% | 0.25% | 0.26% | 0.29% | 0.20% | 0.20% | 0.25% |
|  | Micro sleep | 0.02% | 0.01% | 0.02% | 0.03% | 0.02% | 0.02% | 0.00% | 0.01% | 0% | 0.01% |
|  | Light sleep | 0.15% | 0.13% | 0.17% | 0% | 0.15% | 0.17% | 0.00% | 0.14% | 0% | 0.15% |
|  | Deep sleep | 64.66% | 64.94% | 64.99% | 65% | 64.62% | 64.28% | 61.85% | 66.80% | 61.60% | 64.58% |
| VoIP, w/o C-DRX | PDCCH only |  | 98.59% | 98.56% | 98.43% | 98.75% |  | 98.70% | 98.46% | 98.50% | 98.60% |
|  | PDCCH+PDSCH |  | 1.41% | 1.44% | 1.57% | 1.25% |  | 1.31% | 1.54% | 1.50% | 1.40% |
|  | Micro sleep |  | 0.00% | 0.00% | 0.00% | 0.00% |  | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Light sleep |  | 0.00% | 0.00% | 0.00% | 0.00% |  | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Deep sleep |  | 0.00% | 0.00% | 0.00% | 0.00% |  | 0.00% | 0.00% | 0.00% | 0.00% |
| VoIP, w/ C-DRX | PDCCH only | 22.09% | 22.40% | 21.46% | 19.31% | 18.72% |  | 14.43% | 20.16% | 32.40% | 22.31% |
|  | PDCCH+PDSCH | 1.08% | 1.09% | 1.09% | 0.78% | 1.25% |  | 0.40% | 1.96% | 1.50% | 1.08% |
|  | Micro sleep | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |  | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Light sleep | 1.37% | 0.61% | 0.41% | 0.00% | 0.00% |  | 0.00% | 0.34% | 26.50% | 0.54% |
|  | Deep sleep | 75.45% | 75.90% | 77.04% | 79.92% | 80.03% |  | 85.16% | 77.54% | 39.60% | 76.07% |

Annex <X>:
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc |  | CR | Rev | Cat | Subject/Comment | New version |
| 2018-10 | RAN1#94bis |  | R1-18xxxxx |  |  |  | TR Skeleton | 0.0.0 |