3GPP TSG-RAN WG2 Meeting #105                                     R2-1900198

Athens, Greece, 25th February – 1st March 2019

Source:	CATT

Title:	Enhancement of Paging Procedure with Power Saving Signal

Agenda Item:	11.11.2

Document for:	Discussion and Decision

- Introduction

In the SID of UE power saving in NR [1], objective 3 depends on RAN2 discussion as follows:

- Study the enhancement of higher layer procedures for UE power saving [RAN2] - Study the enhancement of UE paging procedure based on the additional power saving signal/channel/procedure - Study the enhancement of UE power saving procedure in supporting efficient transition from RRC_CONNECTED to RRC_IDLE/RRC_INACTIVE mode

In this contribution, we would like to discuss the enhancement of UE paging procedure based on the additional power saving signal/channel/procedure.

- Discussion

  - Background of Wake Up Signal in LTE

In R15 further eMTC enhancement and further NB-IoT enhancement [1][2][3][4], wake up signal is introduced to reduce the UE power consumption of idle mode paging. The following is the paging procedure with WUS.

When DRX is used and the UE detects WUS the UE shall monitor the following PO. When extended DRX is used and the UE detects WUS the UE shall monitor the following numPOs POs or until a paging message including the UE's NAS identity is received, whichever is earlier. If the UE does not detect WUS the UE is not required to monitor the following PO(s).

The UE determines the end of WUS (the actual subframe g0) based on start of the first PO and the parameter time-offset broadcasted in system information, as follows:

g0 = PO – timeoffset

As the power consumption of detecting WUS is less than that of detecting PO, UE power consumption can be reduced with the introduction of paging with WUS.

  - Paging with power saving signal/channel in NR

- The general paging procedure with power saving signal/channel

Studying the enhancement of UE paging procedure based on the additional power saving signal/channel is a sub-objective of UE power saving SI. The existing agreements of WUS in LTE can be re-considered in NR. At least the general paging procedure with WUS can be re-used in paging with power saving signal/channel in NR. Considering only DRX is supported in NR, the general paging procedure with power saving signal/channel in NR can be:

When the UE detects power saving signal/channel, the UE shall monitor the following PO. If the UE does not detect power saving signal/channel, the UE is not required to monitor the following PO.

Figure 1 is the paging procedure with power saving signal/channel.

Figure 1 Paging Procedure with Power Saving Signal/Channel

Proposal 1: The general paging procedure with power saving signal/channel in NR is:

When the UE detects power saving signal/channel, the UE shall monitor the following PO. If the UE does not detect power saving signal/channel, the UE is not required to monitor the following PO.

- Time domain of power saving signal

The design of power saving signal/channel is depended on RAN1. However, how to determine time domain of power saving signal is in RAN2’s scope.

For time domain of WUS in R15 LTE, the UE determines the end of WUS based on the following PO and a parameter time-offset. When DRX is applied, there is one to one mapping between WUS and PO. The same principle can be applied in NR.

Proposal 2: There is one to one mapping between power saving signal/channel and PO.

In NR, beam sweeping is applied. The same power saving signal/channel needs to be repeated in all transmitted beams. This is different with LTE. In order to determining time domain of power saving signal/channel, there are several possible options.

Option 1: UE determines time domain of power saving signal/channel (e.g. the end of power saving signal/channel) based on the start of the following PO.

This is similar as WUS mechanism in LTE.

Option 2: UE determines time domain of power saving signal/channel (e.g. the start of power saving signal/channel) based on configured parameters.

The time duration of beam sweeping with power saving signal/channel may be different with PDCCH monitoring occasion for paging. In addition, similar as PO configuration, some slots are UL slots which cannot be used for power saving signal/channel. It is more flexible if UE determines time domain of power saving signal/channel based on configured parameters.

Figure 2 shows POs with the following parameters:

Ns =2;

Number of SSBs X = 4;

firstPDCCH-MonitoringOccasionOfPO = {3,8}

Monitoring-periodicity-PDCCH-slot = 2

Monitoring-offset-PDCCH-slot=1

Monitoring-symbols-PDCCH-within-slot = 00100000100000

PDCCH Corset Length in symbols = 3

Figure 2 Paging Occasion

With option 1, assume offset between the end of power saving signal/channel and the start slot of PO is 2 slots, and the duration of power saving signal/channel is 4 continuous symbols. Based on these configurations, occasion of power saving signal/channel with option 1 is shown in Figure3,

Figure 3 Occasion of Power saving with option 1

With option 2, assume SFN of power saving occasion is same as SFN of PO. Slot 5, slot 6 and slot 7 are UL slots which can’t be configured for power saving signal/channel. In configurations of PO, the first PDCCH monitoring occasion for paging of each PO of the PF (i.e. firstPDCCH-MonitoringOccasionOfPO) is introduced to skip some slots which cannot be used for paging. Similar as paging mechanism, the start slot of power saving occasion is configured with {1,4}.  The monitoring symbols of power saving signal/channel within a slot = 11001100000000. Based on these configurations and one to one mapping between power saving signal/channel and PO, occasion of power saving signal/channel with option 2 is shown in Figure4,

Figure 3 Occasion of Power saving with option 2

Proposal 3: RAN2 considers the following options for determining time domain of power saving signal:

- Option 1: UE determines time domain of power saving signal/channel based on the start of the following PO.

- Option 2: UE determines time domain of power saving signal/channel based on configured parameters.

- Conclusion

In this contribution, we discuss the general paging procedure with power saving signal/channel, and how to determine time domain of power saving signal. We propose:

Proposal 1: The general paging procedure with power saving signal/channel in NR is:

When the UE detects power saving signal/channel, the UE shall monitor the following PO. If the UE does not detect power saving signal/channel, the UE is not required to monitor the following PO.

Proposal 2: There is one to one mapping between power saving signal/channel and PO.

Proposal 3: RAN2 considers the following options for determining time domain of power saving signal:

- Option 1: UE determines time domain of power saving signal/channel based on the start of the following PO.

- Option 2: UE determines time domain of power saving signal/channel based on configured parameters.

- Reference

[1] R2-1809254, Introduction of further eMTC enhancements in 36.304, Nokia, Nokia Shanghai Bell

[2] R2-1813078, Define relation between WUS occasion and PO, Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, Huawei, HiSilicon, Ericssion, ZTE Corporation

[3] R2-1813092, Introduction of Rel-15 eMTC enhancements (other than EDT), Qualcomm Incorporated

[4] TS 36.304 v15.2.0