3GPP TSG-RAN2 105                                                                                          R2-1900665

Athens, Greece, 25th Feb – 1st Mar 2019

Title:	[Draft] LS on CSI-RS RRM measurement

Response to:

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	RAN WG2

To:	RAN WG1

Cc:

Contact Person:

Name: ZTE

E-mail Address:	liu dot jing30 (at) ZTE dot com dot cn

Attachments:

1. Overall Description:

RAN2 has discussed the CSI-RS RRM measurement framework defined in TS38.214 clause 5.1.6.1.3, RAN2 has identified the following misalignment between the RAN1 and RAN2 specs:

- RAN1 specs specify the maximum number of CSI-RS resources “per frequency layer”. However, from RAN2 perspective, the concept of “per frequency layer” defined in RAN1 specs doesn’t exist. RAN2 has no restriction on the type of CSI-RS resources to be indicated in a single MO. It is feasible for the network to include multiple cell’s CSI-RS resources in a single MO as long as the cells have the same pointA.

Based on the above, RAN2 would like to know whether the “per frequency layer” refers to the CSI-RS resources associated to same pointA?

In addition, for UE CSI-RS measurement behaviour upon DRX configuration, RAN2 understands that the expectation from the UE side that the CSI-RS resources may not be available other than during the active time is only applicable to CSI-RS measurement on serving cell. This is because, it is not possible for a neighbour cell to get the DRX active time information of UEs in serving cell and transmit CSI-RS resources accordingly. If this is the case, then RAN2 kindly asks RAN1 to clarify that this is applicable only to serving cells.

2. Actions:

To RAN1 group.

ACTION: 	RAN2 respectfully asks RAN1 to:

- Clarify the definition of the “per frequency layer” in the description of the maximum number of CSI-RS resources. Specifically, RAN2 kindly requests RAN1 to clarify whether this refers to the CSI-RS resources associated to the same pointA?

- Clarify in the RAN1 specifications that the expectation of the UE that the CSI-RS resources may not be available other than during the active time is applicable only to the CSI-RS resources in the serving cell (and not to the neighbour cell resources for RRM)

3. Date of Next TSG-RAN1 Meetings:

TSG-RAN2 Meeting #105-bis		8th April  – 12th April 2019		China, Xi’an

TSG-RAN2 Meeting #106		13th May  – 17th May 2019		Reno, USA