3GPP TSG-RAN2 Meeting #105	R2-1900690

# Athens, Greece, 25 Feb – 1 Mar 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 0857 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | State of MCG SCells on resume (38.331) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When a UE gets suspended, the MCG SCell configuration are stored in the UE Inactive AS context. When a UE resumes, the MCG SCell configuration are restored from the UE Inactive AS context, unless the RRCResume contains the fullConfig flag. In the current specs, the state of the MCG SCells on resumption from suspended state is ambiguous, as the state of the SCells is not part of the UE Inactive AS context. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added procedure to explicitly put the MCG SCells into deactivated state, the same way as when SCells are added the first time. Impact analysis Impacted functionality: - CA functionality Inter-operability: If the network is implemented according to this CR while the UE is not, the UE may start some of the MCG SCells in activated state (i.e. if they were in activated state on suspension), while the network assumes they were in deactivated state, thereby creating a state mismtach between the network and the UE. If the UE is implemented according to this CR while the network is not, the UE will start all of the MCG SCells in deactivated state, while the network may assume the some of the SCells were in activated state (i.e if they were in activated state on suspension), thereby creating a state mismtach between the network and the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity about the state of the MCG SCells on resume, possibly resulting in state mismatch between the network and UE, consequently leading to errors in CA operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.3 Reception of the RRCResume by the UE |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF FIRST CHANGES

#### 5.3.5.3	Reception of the RRCResume by the UE

The UE shall:

1>	stop timer T319;

1>	stop timer T380, if running;

1>	if the RRCResume includes the fullConfig:

2>	perform the full configuration procedure as specified in 5.3.5.11;

1>	else:

2>	if drb-ContinueROHC is included:

3>	indicate to lower layers that drb-ContinueROHC is configured;

2>	restore the masterCellGroup and pdcp-Config from the UE Inactive AS context;

2>	configure lower layers to consider the MCG SCells, if configured, to be in deactivated state;

1>	discard the UE Inactive AS context except the ran-NotificationAreaInfo;

- if the RRCResume includes the masterCellGroup:

2>	perform the cell group configuration for the received masterCellGroup according to 5.3.5.5;

<<skipped parts>>

END OF FIRST CHANGES