3GPP TSG-RAN WG2 Meeting #105	R2-1901201

Athens, Greece, 25 Feb – 1 Mar 2019	Revision of: R2-1818248

Agenda item:	11.6.4.1

Source: 	Huawei, HiSilicon

Title: 	Considerations on mobility management in NTN

Document for:	Discussion and Decision

Introduction

In the RAN2 #103bis meeting, it was agreed to study mobility issue in NTN. In this contribution, we provide our further consideration on mobility management of NTN.

Compared to the previous version, the following change is made:

- Modify proposal 2

Discussion

In the last RAN3 meeting, the following 6 scenarios are considered in the study item:

- Scenario A: Transparent GEO (NTN beam foot print fixed on earth)

- Scenario B: Regenerative GEO (NTN beam foot print fixed on earth)

- Scenario C1: Transparent LEO (NTN beam foot print fixed on earth)

- Scenario C2: Transparent LEO (NTN beam foot print moving on earth)

- Scenario D1: Regenerative LEO (NTN beam foot print fixed on earth)

- Scenario D2: Regenerative LEO (NTN beam foot print moving on earth)

According to the 6 scenarios, we discuss the mobility management of NTN by considering the two scenarios:

Scenario 1: Cell fixed on earth

Corresponding to NTN beam foot print fixed on earth, as shown in Fig.1. As illustrated in this figure, cell 1and cell 2 are served by gNB1, and cell 3 and cell 4 are served by gNB2 at T1. At T2, although gNB1 and gNB 2 have moved, gNB1 still can serve cell 1 and cell 2, and gNB2 still can serve cell 3 and cell 4 by adjusting gNB beam direction. At T3, cell 1 still is served by gNB1 but cell 2 is served by gNB2 because the gNB 1 moves too far and it is difficult to cover cell2. Similarly, gNB2 can serve cell 3 but not cell 4. Cell 4 will be served by other gNB. Note that the locations of UEs are fixed in this figure.

Figure 1 Cell fixed on earth scenario

Scenario 2: Cell moving on earth

Corresponding to NTN beam foot print moving on earth, as shown in Fig. 2. As shown in this figure, the locations of UEs are fixed and the cells are moving with the moving of gNB. During the motion, the association relationship between cell and gNB does not change.

Figure 2 Cell moving on earth scenario

  - Mobility management in scenario 1

In this scenario, the location of cell is fixed and handover occurs when UE moves out the coverage of its serving cell just like terrestrial networks. We consider the mobility management in this scenario by considering the moving of gNB and the moving of UE respectively.

- The moving of gNB

As shown in Fig. 1, the serving gNB of cell 2 may become gNB2 from gNB1 at T3. During this procedure, the serving cell of UE2 does not change but the serving gNB changes. The following issues to be solved are identified for the change of serving gNB in such case:

- Security update

Change of gNB would require the change of key. Therefore a security update procedure is needed for such case.

- UE context transfer

The UE context needs to be transferred from the old gNB to the new gNB and possibly some RRC parameters need to be update as well.

- TA and SSB tracking issues due to sudden change of Propagation delay

The propagation delay may change suddenly due change of gNB, this would result in TA and SSB timing change in a sudden way.

Observation 1: for scenario 1, there are the following issues because of the moving of gNB:

1) Security update issue;

2) UE context transfer issue (RAN3 impact);

3) Sudden TA change issue;

4) SSB tracking issue (RAN1 and RAN4 impact).

Since the satellite moving orbit is relative fixed, the previous serving gNB knows the information of the later serving gNB, so the previous gNB could trigger intra-cell handover with a new Key based on the later gNB when the serving gNB switch will occur. After finishing the intra-cell handover, the sudden TA change issue also can be solved. Hence, intra-cell handover may can be considered as a potential solution of security update issue and sudden TA change issue for scenario 1.

Proposal 1: for scenario 1, intra-cell handover can be considered as a potential solution of security update issue and sudden TA change issue.

- The moving of UE

There are two cases can be considered for the moving of UE.

Case 1: the target gNB does not change during handover procedure, like the UEs perform handover during the time from T1 to T2;

Case 2: the target gNB changes during handover procedure, like the UEs perform handover during the time from T2 to T3.

For case 1, if there is Xn interface between source gNB and target gNB just like terrestrial networks, the current Xn based handover mechanism could be reused as baseline.

Observation 2: for case 1 in scenario 1, if there is Xn interface between source gNB and target gNB just like terrestrial networks, the current handover mechanism could be reused as baseline.

For case 2, the UE may suffer security update issue during handover procedure. For example, at some time from T2 to T3, UE performs handover preparation from cell 1 to cell 2 and obtains handover command from gNB1, but the serving gNB of cell 2 becomes gNB2 when the UE performs handover at T3. In this case, although the UE context and handover command created by gNB1 can be transferred to gNB2 by Xn interface, the UE still cannot finish initial access because the Key in handover command does not work in gNB2. So, RAN2 should consider the Key issue brought by the target gNB change during handover procedure in scenario 1.

Proposal 2: for case 2 in scenario 2, the security update issue brought by the target gNB change during handover procedure should be considered.

Mobility management in scenario 2

In this scenario, the cells move with the moving of gNBs and the moving speed is related to the altitude of gNB. Taking LEO at 600km as an example, the radius could range from 50Km to 500Km [1], a UE may just stay in one cell within several minutes. The UE needs to measure the neighbour cells and perform handover procedure frequently, which is power-consuming from UE perspective. In the terrestrial network, only a small number of UEs will trigger handover at the same time since the ratio of moving UE is low (except the rail way scenario). However, almost all UEs, except those UEs which move quickly like gNB, need to perform handover in NTN. In this way, the network has to handle massive handover request and switch the path of UEs, which will increase the signalling overhead of networks largely.

Observation 3: the frequent handover leads to additional UE power consumption and large networks signalling overhead.

Therefore, RAN2 should consider how to handle frequent handover issue. Considering the relative fixed moving orbit of satellite, the network knows the next serving cell of one UE. Conditional handover could be reused in NTN. The network could pre-configure handover resource of target cell to UE with a trigger condition, i.e. a timer or a certain location. When the timer expires or UE arrives this location, handover is triggered, i.e. UE use the pre-configured handover resources to perform initial access in target cell. In this way, frequent measurement for UE can be avoided, this will also reduce the UE power consumption resulting from frequent handover. But for CHO we still need to wait for the progress of the corresponding WI.

Proposal 3: RAN2 should consider to enhance handover mechanism to handle the frequent handover issue in scenario 2, e.g. conditional handover, but we still need to wait for the progress of the corresponding WI.

Mobility management for long RTT

Moreover, the RTT in NTN is higher than that of terrestrial networks due to the high altitude of gNB. The RTT of GEO satellite reaches up to 280ms [1] and the time will be double in the scenario that DU is deployed in satellite. The long RTT means that reporting measurement results will consume more time, and then the handover decision may be not timely. Besides, the long RTT results in the long latency of handover command. Late handover decision and long transmission latency of handover command may cause that UE has move out the coverage of source cell and couldn’t receive handover command from the source cell, and leads to handover failure further.

Observation 4: the high RTT in NTN may lead to handover failure.

To handle this issue, UL based handover may be a potential solution. By making handover decision based on UL signal quality instead of DL signal quality the network could avoid the latency of measurement result reporting, and then reduce the total handover latency.

Proposal 4: RAN2 could consider to reduce the handover latency to avoid handover failure.

Conclusion

In this contribution, we considered mobility management of NTN, and we get the following proposals:

Observation 1: for scenario 1, there are the following issues because of the moving of gNB:

1) Security update issue;

2) UE context transfer issue (RAN3 impact);

3) Sudden TA change issue;

4) SSB tracking issue (RAN1 and RAN4 impact).

Proposal 1: for scenario 1, intra-cell handover can be considered as a potential solution of security update issue and sudden TA change issue.

Observation 2: for case 1 in scenario 1, if there is Xn interface between source gNB and target gNB just like terrestrial networks, the current handover mechanism could be reused as baseline.

Proposal 2: for case 2 in scenario 2, the security update issue brought by the target gNB change during handover procedure should be considered.

Observation 3: the frequent handover leads to additional UE power consumption and large networks signalling overhead.

Proposal 3: RAN2 should consider to enhance handover mechanism to handle the frequent handover issue in scenario 2, e.g. conditional handover, but we still need to wait for the progress of the corresponding WI.

Observation 4: the high RTT in NTN may lead to handover failure.

Proposal 4: RAN2 could consider to reduce the handover latency to avoid handover failure.

References

[1] 3GPP TR 38.811: “Study on New Radio (NR) to support non terrestrial networks”