3GPP TSG-RAN2 Meeting #105	R2-1901262

Athens, Greece, 25 February - 1 March 2019

Agenda Item:	10.4.5.3 Idle/inactive paging

Source: 	Ericsson

Title:  	Clarification for paging and UL symbols

Document for:	Discussion and Decision

# Introduction

In this contribution the PDCCH monitoring occasions for paging and the configuration of UL symbols is discussed.

# Discussion

The possible slot formats are DL, UL and flexible symbols, as specified in section 11.1 in 38.213.

The DL symbols are configured from the left, indicated by nrofDownlinkSymbols in tdd-UL-DL-ConfigurationCommon IE, while UL symbols are configured from the right, indicated by nrofUÖlinkSymbols in tdd-UL-DL-ConfigurationCommon IE. The symbols that are not configured as UL or DL are flexible symbols, as depicted below:

A flexible symbol can be reconfigured as UL or DL symbol with tdd-UL-DL-ConfigurationDedicated IE.

The UE numbers the PDCCH monitoring occasions for paging sequentially, as specified in section 7.1 in 38.304. To guarantee that NW and UE use the same PO it is proposed to clarify that the UE shall skip the UL symbols configured with tdd-UL-DL-ConfigurationCommon IE in Idle, Inactive and Connected mode:

Proposal 1: UE shall skip symbols configured as UL via tdd-UL-DL-ConfigurationCommon IE for paging.

This implies that when a flexible symbols is configured as an UL symbol via tdd-UL-DL-ConfigurationDedicated IE this symbol cannot be used for paging, but that the UE may monitor the PO:

Observation 1: A flexible symbol that has been configured as UL symbol via tdd-UL-DL-ConfigurationDedicated IE is not used for paging, but UE monitors this symbol for paging.

Clarification for 38.304 is provided for discussion and agreement [1].

# Summary

Proposal 1: UE shall skip symbols configured as UL via tdd-UL-DL-ConfigurationCommon IE for paging.

# References

- R2-1901263, Clarification for paging and UL symbols, Ericsson, CR 38.331, REL-15