3GPP TSG-RAN WG2 #105	R2-1901299

# Athens, Greece, 25th Feb – 1st Mar 2019

Agenda Item:	11.7.2.2

Source:	Ericsson

Title:	TSN traffic patterns

Document for:	Discussion, Decision

# 1. Introduction

Following up on the email discussion [104#36] NR IIOT TSN Traffic Pattern [2] related to TSN message periodicities that are non-divisible by the NR supported periodicities, we highlight how de-jittering buffers and adjustable configured grants can be utilized to mitigate this issue.

# 2. Discussion

## The requirements of TSN flows

The main target of a Time Sensitive Network (TSN) is to provide deterministic transport services over IEEE standard 802.3 Ethernet wired networks. Several of the TSN features and standards generally target at: 1) zero packet loss due to buffer congestion, 2) extremely low packet loss due to failures in equipment, bit errors, or in the control plane. 3) guaranteed upper bounds on end-to-end latency. The demanding requirements of use-cases that require transport from TSN networks are also summarized in TR 22.804 and TS 22.104 to some extent as well.

## 2.2 Periodic TSN flows

Communication in TSN happens in TSN flows between the end points that are called Talkers and Listeners. One specific feature in TSN is for example, that flows are subject to an agreement, as arranged between the transmitters (Talkers) and the network till the receivers (Listeners), that ensures low and bounded latency transmissions without unforeseen queueing.

TSN traffic is quite often periodic and the TSN methods work on the basis of worst-case scenario to guarantee the delivery of the packet with bounded latency. This bounded delay guarantee (delay that cannot exceed a threshold), as well as traffic that cannot arrive earlier of what expected. One can describe TSN traffic as traffic passing through nodes of the network within specific time frames.  In order to synchronize the pacing of the packet delivery, all network devices and hosts can synchronize their internal clocks to an accuracy between 1 us and 10 us, based on a variant of the IEEE 1588 Precision Time Protocol (PTP).

## 2.3 Protection of TSN Flows

An important part for providing on-time delivery of TSN frames is 802.1Qbv, that defines the time windows that the TSN Ethernet frames move inside the network. The TSN network also includes some necessary controller that learns the network capabilities and control it so to serve the requirements of the applications. These entities are the CNC and the CUC.

The CNC discovers the physical topology and the CUC requests network resources. As a next step, the CNC will calculate the needed schedules based on the collected physical topology and the network resources. At a later phase that schedule is distributed to each switch. These entities ensure the exclusive use of the common hardware resources in time domain, like the common cables between the switches (i.e., in that case the cable will be granted to only specific device at a specific point in time).

The table below gives such an example of the schedules where a window is shown for the transmitter and the receiver. These windows confine that transmissions and receptions should take place only within these time windows.  Similar time windows are being delivered also to TSN switches where receptions and transmissions are bounded to specific time windows.

Table 1: An example of Computations on Transmit and Receive Windows inside a TSN network.

| Stread ID | Dest. MAC | Size | Period | … | Talker Name | Talker Transmit Windows | Receiver Name | Receiver Window | Transmission Duration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Flow 1 | 02:00:53:A0:02:a2 | 64 | 1 ms | … | TSN-TL1 | 207-220 μs | Tsn-TL2 | 320-333 μs | 100 μs |
| Flow 2 | 03:00:5E:A0:03:eA | 64 | 1 ms | … | TSN-TL3 | 707-720 μs | Tsn-TL4 | 920-933 μs | 100 μs |

Observation 1: The transmission and reception of the TSN flows are required to be within very specific time windows.

During the email discussion, it is mentioned that non-divisible periodicities from the 5G system are not being mentioned at the TS 22.104 and the TR 22.804. We would like to highlight though that these are only examples and the 5G system, integrated to a TSN network, will need to transfer industrial type of traffic. This traffic is generated many times from existing hardware, for example PLCs, where the periodicities cannot be adjusted. Such hardware typically takes years, if not decades, to be replaced. The periodicity of such hardware devices come from their frequency of operation (i.e., 100 or 120 Hz) that typically provides non-divisible periodicities for the TSN network.

## 2.4 Transferring TSN flows

The 5G system is not a TSN-compatible switch and there are potential sources of delay variance (jitter). One such source is for example the mismatch between the TSN traffic periodicities and the Configured Grant periodicities. Such mismatch causes the TSN packet to wait for the configured grant to become available. This waiting period most of the time is not constant, thus contributing to the increase of jitter.

In other words, TSN supports periodicities of much higher granularity, since CNC has control of the network resources (time domain planning). For example, it can support a smart grid application in which traffic period is 1/60 Hz or 1/1200 Hz, i.e. 16.667 ms or 0.833 ms respectively. For the latter case of a TSN traffic periodicity of 0.833ms and a latency guarantee of 0.833ms, the 5G system can use subcarrier spacing of 15 KHz to support these requirements.

Observation 2: TSN flows have often non-divisible traffic periodicities.

Since the TSN and the Configured Grants periodicities do not typically match, the latter needs to be adjusted to reduce the mis-aligned latency (mis-aligned latency is denoted as the time that the TSN traffic waits for the configured grant to become available).

For supporting this type of traffic (periodicity of 0.833 ms and latency target of 0.833 ms) over the 5G one example is given. The RAN delivery target is set to 0.5 ms. The configured grant resources are allocated every 0.5ms with a duration of 0.5ms (using a 7-symbol long mini-slot). That makes resources available at time 0, 0.5,1, 1.5, 2, 2.5, 3, 3.5, 4, 4.5, 5, 5.5 milliseconds and so on.

From a TSN flow perspective, the TSN flows during the transmission over the 5G system do not have a constant E2E transmission time, since TSN flows need to wait a non-constant amount of time before the configured grant becomes available. This can be decomposed to a constant delivery RAN target and a variable waiting time for the configured grant to become available. This is shown at Table 2, where the RAN delivery target remains the same and the mis-alignment latency varies.

Even if the configured grants become much more granular and are adjusted as close to the TSN periodicities, still jitter arises.

Table 2 Configured Grants to support the TSN traffic reducing the mis-alignment latency.

|  | 1st | 2nd | 3rd | 4th | 5th | 6th | 7th | 8th |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Arrival time (ms) | 0 | 0.833 | 1.667 | 2.5 | 3.333 | 4.167 | 5 | 5.833 |
| RAN delivery target (ms) | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |
| Start of the CG resource with the least alignment latency | 0 | 1 | 2 | 2.5 | 3.5 | 4.5 | 5 | 6 |
| Mis-align latency (ms) | 0 | 0.166 | 0.333 | 0 | 0.166 | 0.333 | 0 | 0.166 |

Observation 3: Mis-alignment of TSN flow periodicity and CG periodicity causes the increase of jitter in the E2E delivery.

At the same time this means that not all available configured grants will be used, for that scenario only the 1st 3rd and 5th configured grants will be used, while the 2nd and the 4th are not utilized since there is no TSN traffic waiting to be transmitted. This pattern of configured grants continues over time to stay on par with the TSN pattern requirements.

Observation 4: Adjusting the configured grants allows a closer matching of configured grants and TSN patterns requirements.

As illustrated above, from the knowledge on the TSN traffic periodicity and the configured-grant/SPS periodicity, gNB can find out those configured-grant/SPS resource allocation timings that will not be used by TSN traffic. Since no TSN traffic is expected from these allocated resources, it would be beneficial for system capacity point of view for gNB to schedule transmissions on those configured-grant/SPS resources for other traffic.

For configured grant, the transmission from the UE might not be skipped unless an LCH restriction is introduced so that only the LCH that carries this periodic TSN flow can be mapped to the configured grant.  Some more efficient adjustments can be considered as well, such as indicating an active/in-active pattern of configured-grant/SPS resource allocation.

Observation 5: Efficient adjustment of SPS/CG resource allocation timing, e.g., by network configuration of an active/in-active pattern is beneficial.

The Figure 1 below show how this problem of jitter in the E2E delivery can be problematic for transferring these packets towards the TSN domain.

Figure 1 An example on the jitter accumulated at the egress of 5G towards the next TSN switch.

One possible solution is to use de-jitter buffer at the egress of the 5G system to de-jitter the TSN packets at expected times to the next node. The de-jitter buffer masks out the jitter inside the 5G system and makes sure that no packet is being delivered outside the agreed time period.

Table 3 Handling jitter sources with the de-jitter buffer delaying the needed time packets.

|  | 1st | 2nd | 3rd | 4th | 5th | 6th | 7th | 8th |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Arrival time(ms) | 0 | 0.833 | 1.667 | 2.5 | 3.333 | 4.167 | 5 | 5.833 |
| Used CG resource | 0 | 1 | 2 | 2.5 | 3.5 | 4.5 | 5 | 6 |
| Mis-align latency(ms) | 0 | 0.166 | 0.333 | 0 | 0.166 | 0.333 | 0 | 0.166 |
| RAN delivery target(ms) | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |
| De-jitter buffer time(ms) | 0.333 | 0.166 | 0 | 0.333 | 0.166 | 0 | 0.333 | 0.166 |

Observation 6: Introduction of de-jitter buffer at the egress of 5G system reduces the delivery jitter at the next TSN switch.

In addition, the de-jitter buffer can be also seen as a potential place to optimize accordingly other jitter sources inside the 5G system.

## 2.5 Traffic information about TSN flows

As TSN flows require very precise handling from the 5G system, information about their characteristics are beneficial to the 5G System. This type of information can be further characterized as information needed for the incoming flows and for the outgoing flows.

The first type of information for the incoming TSN flows, relates to the traffic characteristics (time of arrival and traffic patterns) that allow the RAN to have a much more accurate configuration to meet the QoS requirement with appropriate packet forwarding treatment. Any uncertainty related to the incoming TSN packets at RAN will force the RAN to overschedule resources (counting for also worst case scenarios) to keep the tight requirements of TSN traffic. That will of course lead to a waste of resources before the RAN learns the pattern, if the latter is possible. This type of traffic information for the TSN flows can also help with the admission control allowing the RAN to avoid more conservative estimates that might lead to unnecessary waste of resources.  This information is available at the CNC and can be provided to the RAN beforehand.

The second type of information relates to the outgoing TSN flows, where TSN traffic information can help the egress points, UE and UPF for traffic pacing. This information is needed for making sure that the egress points de-jitter the packets at the needed time frames that the next TSN switch expects them. This information that relates closer to the Qbv type of information where the specific time window per TSN switch are given, can help on the configuration of the egress points. This way, RAN is required to deliver the data to the 5G system egress point within the packet delay budget, but RAN is free in deciding when to transmit within this packet delay budget, i.e. RAN does not need itself to provide a deterministic latency for each packet. As explained above, the deterministic latency is achieved on higher layers at the 5G system egress point by pacing/dejittering.

Figure 2 depicts the two different type of information required for incoming and outgoing TSN packets .

Figure 2 Ingress and egress type of information about the TSN flows.

Observation 7: The information of the TSN flows at both the ingress and the egress point is beneficial for the 5G system to meet the TSN requirements.

Observation 8: The ingress information, such as periodicity, packet size and time information, can be provided to RAN to improve scheduling efficiency.

Observation 9: The egress information is needed for the de-jittering (pacing) function and should be provided to UE for DL traffic and UPF for UL traffic.

Observation 10: With de-jittering (pacing) function, there is no requirement on RAN to provide the same deterministic latency for every packet, but instead only that packets are delivered within their delay budget.

With the above observations, we propose that

Proposal 1: Send an LS to SA2 to indicate that RAN2 considers there are benefits to integrate TSN flow information in 5G system for a) improving RAN scheduling efficiency (with ingress traffic information) and b) enabling deterministic latencies by pacing/de-jittering (with help of egress traffic information).

# 3. Summary

In the previous sections we made the following observations:

Observation 1: The transmission and reception of the TSN flows are required to be within very specific time windows.

Observation 2: TSN flows have often non-divisible traffic periodicities.

Observation 3: Mis-alignment of TSN flow periodicity and CG periodicity causes the increase of jitter in the E2E delivery.

Observation 4: Adjusting the configured grants allows a closer matching of configured grants and TSN patterns requirements.

Observation 5: Efficient adjustment of SPS/CG resource allocation timing, e.g., by network configuration of an active/in-active pattern is beneficial.

Observation 6: Introduction of de-jitter buffer at the egress of 5G system reduces the delivery jitter at the next TSN switch.

Observation 7: The information of the TSN flows at both the ingress and the egress point is beneficial for the 5G system to meet the TSN requirements.

Observation 8: The ingress information, such as periodicity, packet size and time information, can be provided to RAN to improve scheduling efficiency.

Observation 9: The egress information is needed for the de-jittering (pacing) function and should be provided to UE for DL traffic and UPF for UL traffic.

Observation 10: With de-jittering (pacing) function, there is no requirement on RAN to provide the same deterministic latency for every packet, but instead only that packets are delivered within their delay budget.

Based on the discussion in the previous sections we propose the following:

Proposal 1: Send an LS to SA2 to indicate that RAN2 considers there are benefits to integrate TSN flow information in 5G system for a) improving RAN scheduling efficiency (with ingress traffic information) and b) enabling deterministic latencies by pacing/de-jittering (with help of egress traffic information).

# 4	References

Report from [104#36][NR/IIOT]TSN Traffic Patterns