3GPP TSG-RAN WG2 #105	 TDoc R2-1901649

# Athens, Greece, 25th February – 1st March 2019

Agenda Item:	11.4.2.2

Source:	Ericsson

Title:	Bandwidth parts and Resource Pools for NR V2X

Document for:	Discussion, Decision

# Introduction

In latest RAN1 ad-hoc meeting, the following agreement was made with respect to the need of bandwidth part (BWP) in NR sidelink.

From RAN1-ah1901: Agreements: - Confirm the working assumption - Working assumption: only one SL BWP is configured in a carrier for a NR V2X UE Agreements: - Configuration for SL BWP is separated from Uu BWP configuration signalling. - UE is not expected to use different numerology in the configured SL BWP and active UL BWP in the same carrier at a given time. - FFS the time scale - FFS relation to DL BWP including initial Uu BWP - FFS relation in terms of frequency location and bandwidth

In this paper, we analyse the impact on BWP, mainly focusing on RAN2 issues.

# Discussion

As highlight in above section, RAN1 has concluded in the last RAN1 meeting, there will be SL BWP in NR, but that there will be only one BWP configured in a given SL carrier. Therefore, RAN2 does not need to handle procedures about activation/deactivation of SL BWP.

RAN1 agreed that only one SL BWP is configured in a carrier for a NR V2X UE, therefore RAN2 does not need to specify procedures to activate/deactivate a SL BWP.

In our understanding, the notion of resource pool is more general than that of BWP since pools can be defined in time and frequency domain whereas BWPs only refer to the frequency domain. Also, it may happen that dedicated resource pools are configured by gNB to certain UEs. Therefore, it will be beneficial to reuse the concept of resource pool within the SL-BWP. Moreover, since a UE needs to monitor the (pre-)configured resource pools to decode SCI, it is natural that a transmitting resource pool is ‘narrower’ in frequency compared to BWP. Note that in NR Rel-15, a UE is not required to monitor/receive/transmit anything outside an active BWP.

A SL TX pool is defined as a set of time frequency resources whose frequency component is a subset of the SL BWP.

It was also agreed in RAN1#94bis that multiple resource pools can be configured to a single UE. This is similar to LTE TX resource pools configuration, e.g. when the eNB configures the UE with different pools for different zones. However, a UE should select one TX pools out of the configured resource pools based on certain criteria. For example, based on its geographical location and/or its RF bandwidth.

As in LTE, although multiple resource pools can be configured to a single UE, the UE chooses one TX pool.

Regarding the receiving pool, one may wonder what the difference between SL BWP and RX pool is. We note that a BWP indicates the portion of the spectrum in which the UE receives transmissions of a certain numerology. Therefore, from frequency perspective there can be a one-to-one association between RX pool and BWP. However, while in Uu the time resources in which the UE shall monitor DL transmissions are handled by DRX, in SL there is the need to define an RX pool to account for the time resources in which the UE shall monitor SL transmissions.

A SL RX pool is defined as a set of time/frequency resources whose frequency component is equal to the SL BWP.

# Conclusion

In section 2 we made the following observations:

Observation 1	RAN1 agreed that only one SL BWP is configured in a carrier for a NR V2X UE, therefore RAN2 does not need to specify procedures to activate/deactivate a SL BWP.

Based on the discussion in section 2 we propose the following:

Proposal 1	A SL TX pool is defined as a set of time frequency resources whose frequency component is a subset of the SL BWP.

Proposal 2	As in LTE, although multiple resource pools can be configured to a single UE, the UE choose one TX pool.

Proposal 3	A SL RX pool is defined as a set of time/frequency resources whose frequency component is equal to the SL BWP.