3GPP TSG-RAN2 Meeting RAN2#105 	R2-1902186

Athens, Greece, 25th February – 1st March 2019

Agenda Item	: 11.7.2.4 (FS_NR_IIOT)

Source	: LG Electronics Inc.

Title	: Integrity verification failure handling for DRB

Document for	: Discussion and Decision

Introduction

For IIOT, the integrity protection function should be supported. The security requirement in TR 22.804 is as listed below.

8.2.5.1	Data security and privacy requirements

| Reference number | Requirements | Use case requirement reference |
| --- | --- | --- |
| Nse.Sp.1 | The 5G system shall support data integrity protection and per message authentication of the sender. | Future of factory 2.5, Future of factory 5.5, Smart Cities 1.1, Electric Power Distribution 1.5, Electric Power Distribution 2.5, Centralised Power Generation 1.1, PMSE 1.4, 2.2, 3.2 |
| Nse.Sp.2 | The 5G system shall provide mechanism for denial-of-service prevention. | Electric-Power Distribution 3.9 |

NOTE: The 5G system should not prevent the use of OTT security protocols.

For SRBs, when the integrity verification failure (hereinafter IP failure) happens, the RRC Connection Re-establishment procedure is performed. However, in DRBs, when the IP failure happens, the UE does not perform the RRC Connection Re-establishment procedure, but just discards the packet with IP failure.

From IIOT service point of view, if the IP failure happens, it may cause a production downtime because the loss of the packets may lead to an emergency stop. Hence, minimizing the loss of packets is important even if the packet loss is caused by IP failure.

In this contribution, we show our views on above issue.

Discussion

As well known, the IP failure mainly happens with two reasons; security attack (i.e. intruder changes the contents of the PDU) and protocol error (due to e.g. bunch of packet discard or residual protocol error). If the IP failure is caused by the security attack, the RRC Connection Re-establishment procedure should be performed.

However, if the IP failure is caused by the protocol error, the problem only resides in that DRB, and performing the RRC connection re-establishment is too much for the problem. In this case, we think refreshing only that DRB would be sufficient. Refreshing the DRB may be achieved by the PDCP/RLC re-establishment or DRB release/addition.

Observation. The RRC Connection Re-establishment is too much for the IP failure caused by protocol error.

With the observation above, a next question is how to know whether the IP failure is caused by protocol error or security attack. From UE point of view, the UE does not know whether the IP failure is caused by protocol error or security attack. However, from network point of view, the network would know a cause of the IP failure if the network knows the COUNT value of the packet associated with IP failure. This is because if the IP failure is caused by protocol error, the COUNT value of the PDU associated with IP failure would be most likely different from the COUNT value of the PDU which was transmitted by the network.

With the above understanding, if the UE reports the COUNT value of the packet associated with the IP failure, the network can decide whether the IP failure is caused by the security attack or the protocol error. The network can request to the UE to perform either RRC Connection Re-establishment or DRB release/addition based on the IP failure cause.

To inform the IP failure with the COUNT value, a possible way is to trigger the PDCP Status Report after IP failure on DRB. According to the current specification, PDCP Status Report contains the COUNT value, e.g., FMC. This can be used for the network to determine whether the IP failure is caused by protocol error or security attack by the network. Furthermore, the network may extend the use of the PDCP Status Report, e.g. retransmit the packet discarded by IP failure.

Proposal. Trigger PDCP Status Report when IP failure is detected for a DRB.

Conclusion

In this contribution, we show our views on IP failure reporting. Based on the above discussion, we propose followings.

Observation. The RRC Connection Re-establishment is too much for the IP failure caused by protocol error.

Proposal. Trigger PDCP Status Report when IP failure is detected for a DRB.