3GPP TSG-RAN WG2 #105	Tdoc R2-1902355

Athens, Greece, 25th February – 1st March 2019

Revision of R2-1901369 with updated Text Proposal as a result of agreements in the RAN2#105 IIoT Session.

Agenda Item:	11.7.2.3

Source:	Ericsson

Title:	[104#37][NR/IIoT] Ethernet Header Compression email discussion report, TP

Document for:	Discussion, Agreement

# Introduction

In RAN2#104, within the study on NR Industrial Internet of Things (NR-IIoT), Ethernet header compression was initially discussed, and the following email discussion was started:

[104#37][NR/IIOT] Ethernet Header Compression (Ericsson)

Intended outcome: TP for next meeting, including some expected performance numbers, identify the fields that can be compressed/removed?, which frame structures that will be addressed, the method how to specify in a WI phase (e.g. ROHC addition or other).

Deadline:  Thursday 2019-02-07

Below questions will guide us through the open issues in this topic as outlined by companies’ contributions to RAN2#104.

The email discussion was structured into two phases:

- 1. Input phase, intermediate deadline 2019-01-31

- 2. Summary/TP review phase, deadline 2019-02-07

The summary sections are highlighted in blue.

# Discussion

This email discussion is structured as follows: we first discuss the scenarios and general benefit of Ethernet header compression, we then analyse which Ethernet fields may be compressed and evaluate the gains. Then we discuss applicability of the compression to certain industrial communication protocols, as well as to IP. Afterwards we discuss the protocol layer and network termination point where Ethernet header compression shall be considered. Finally, we discuss how this functionality can be specified.

## Ethernet fields to compress

One general question for header compression is which Ethernet header fields shall be considered in the compression. As an overview, Figure 1 shows the IEEE 802.3 MAC frame format.

Figure 1, Example of IEEE 802.3 MAC frame format – 802.1Q (2014)

Some parts of the Ethernet frame do not need transmission over the 5G system, therefore TS 23.501 states for the PDU session type:

Ethernet Preamble and Start of Frame delimiter are not sent over 5GS:

-	For UL traffic the UE strips the preamble and frame check sequence (FCS) from the Ethernet frame.

-	For DL traffic the PDU Session Anchor strips the preamble and frame check sequence (FCS) from the Ethernet frame.

Do companies agree that PREAMBLE, SFD (start of frame delimiter) and FCS (frame check sum) are not transmitted over the 5G system there are not considered in header compression?  This was among others discussed in R2-1816765.

### Preamble, SFD and FCS are ignored in Ethernet header compression?

| Company | [yes/no ] | Comment – additional fields to be ignored? |
| --- | --- | --- |
| Mediatek | Yes | Ignore all physical layer functionality. |
| Qualcomm | Yes | Can ignore them given 5GS ingress point is expected to remove them as noted in 23.501. |
| Huawei | Yes | N/A |
| LG | Yes | According to TS23.501, the PREAMBLE, SFD, and FCS fields do not need to be transmitted to the network. Thus, UE should remove those field. |
| Ericsson | Yes | Static fields can be removed. Also, since CRC check is performed by lower layers it possible to also remove FCS. |
| ZTE | Yes | N/A |
| Nokia | Yes | These fields have no meaning when Ethernet frames are carried over the 3GPP network. |
| Intel | Yes |  |
| Samsung | Yes | No use to have these fields. |
| OPPO | Yes | As it is specified, preamble, SFD and FCS are not transmitted in 5GS, so these fields are ignored in Ethernet header compression. |
| vivo | Yes | SA2 has specified that the preamble and SFD are not sent over 5GS. For FCS, this field also can be removed as it is only used to check whether an error has occurred during Ethernet frame transmission. |
| CATT | Yes |  |
| CMCC | Yes | These fields can be ignored in Ethernet header compression which is not transmitted in 5GS. |

Summary

Companies are aligned that Preamble, SFD and FCS are ignored in Ethernet header compression, i.e. do not need transmission.

Preamble, SFD and FCS are ignored are not transmitted thus not considered in Ethernet header compression.

Several contributions to RAN2#104 discussed which header fields are to be considered for compression, e.g. R2-1817269, R2-1816765, R2-1817175.

Among others the following fields are up for discussion: DESTINATION ADDRESS, SOURCE ADDRESS, Q-TAG, MAC CLIENT LENGTH/TYPE. Companies are invited to provide their opinion stating also details of the Q-TAG content, e.g. considering how static the considered fields are.

### Header fields at least considered for compression are: DST ADR, SRC ADR, ETHER TYPE?

| Company | Agreement [yes / no ] | Comment |
| --- | --- | --- |
| Mediatek | Yes | As these fields are mandatory. |
| Qualcomm | Yes |  |
| Huawei | Yes, but using UDC (Uplink Data Compression) based mechanism | All the fields in a packet that are possibly static/same as those in another packet e.g. VLAN-1, EtherCAT header for EtherCAT, Frame ID for ProfiNet. |
| LG | Yes | The DST ADR, SRC SDR and ETHER TYPE are the static part of the Ethernet header. Thus, the UE does not need to transmit those fields after exchanging the header information. Considering that, the Context ID should be piggybacked in a compressed packet as in ROHC. |
| Ericsson | Yes |  |
| ZTE | Partially Yes | We agree that DST ADR and SRC ADR could be compressed. But for ETHER TYPE, it should be clarified which field it refers to, e.g., the two bytes LENGTH/TYPE field of Ethernet Frame or only the ETHER TYPE sub-field of SNAP header field in IEEE 802.3 with SNAP Ethernet Frame? We understand the ETHER TYPE here means the two bytes after SRC SDR. With the following Ethernet protocols comparison, we understand that in the different Ethernet protocols, the two bytes after SRC SDR may have different meaning. In order to determine what these two bytes mean, the compressor inevitably needs to parse these two bytes. Once these two bytes are parsed, all the following several header bytes can be determined (e.g. LENGTH/TYPE/802.1Q tag, IEEE 802.2 field, SNAP Header). Taken into account that these header fields are usually static, they may also be compressed together. In a summary, if only two bytes after SRC SDR are compressed, it may cause confusion between compressor and decompressor. If these two bytes are parsed and with more header fields being compressed, it may bring more compression gain but cause more overhead to compressor and decompressor. Therefore, we need to think more about not compressing these two bytes or compressing more bytes. |
| Nokia | Yes | These are the most relevant headers in all scenarios. |
| Intel | Yes |  |
| Samsung | Yes | Omitting these fields could be one of the solutions for compression but UDC(Uplink data compression) can be applied to all Ethernet header fields. |
| OPPO | Yes, and | These three fields should be considered for header compression since they are the common part in different Ethernet frame formats. DST ADR and SRC ADR is static on one link, so they can be classified into static class, and transmit only once. But for ETHER TYPE, it can be classified into different class and compressed with different operation if different Ethernet format is used, e.g. ETHER TYPE is identified as type information and classified into changing class if the Ethernet frame format is related to Ethernet II. |
| vivo | Yes, depending on the interpretation of the field. | For a specific stream of transceiver, DST ADR and SRC ADR is fixed, which shall be considered for compression. As the “etherType” field can be interpreted as either the “etherType” field or the “length” field, how to compress this field may need to be discussed further. |
| CATT | Yes | UDC (Uplink Data Compression) is a possible way for achieving this. |
| CMCC | Yes | The DST ADR, SRC SDR and ETHER TYPE are the static part of the Ethernet header. And we think UDC (Uplink data compression) can also be applied to this. |

Summary

All companies are aligned that at least DST ADR, SRC ADR are considered for compression.

From 13 participating companies 11 are aligned that also ETHER TYPE should be considered. 2 companies suggest further discussion on whether the ETHER TYPE field can be compressed, due to its different length/content. Given the majority view, it is observed:

At least header fields DST ADR, SRC ADR, ETHER ETYPE are considered for compression.

Tagging (Q-Tags) is widely used in Ethernet networks for traffic separation and marking. During forwarding various tags may be added to and removed from user data frames by the tag encoding and decoding functions of L2 nodes. The most common Q-Tags named explicitly for Ethernet header compression related contributions are:

- C-Tag: Tag Protocol Identifier (TPID) and Tag Control Information (TCI)

- S-Tag: Tag Protocol Identifier (TPID) + Tag Control Information (TCI)

These Q-Tags include not only a VLAN ID, but bits used for QoS purposes (e.g., PCP, DE) as well. Compression may not consider for example those QoS related bits.

### Are Q-TAGs (i.e. C-TAG and S-TAG) considered for compression? Including which internal fields?

| Company | Yes / No | Which parts of C/S-TAG? | Comment |
| --- | --- | --- | --- |
| Mediatek | Yes | Double Tags (C-TAG & S-TAG) including PCP/DE fields | VLAN tags are unlikely to change in a given QoS flow because these fields are provisioned on a port by port basis. As a result these fields are likely to remain static and thereby easily compressible when they are present. |
| Qualcomm | Yes | All fields in C/S-TAG | The fields in C/S-TAG including PCP and DEI are likely static too. |
| Huawei | Yes | All | Static fields can be compressed using UDC based mechanism. |
| LG | Yes | Both | The C-TAG and S-TAG would not be changed after the session is established. Thus, the UE does not need to transmit those fields after exchanging the header information. Considering that, the Context ID should be piggybacked in a compressed packet as in ROHC. |
| Ericsson | Yes | All |  |
| ZTE | Yes | The whole 802.1Q tag field (both C-TAG and S-TAG, including: TPID, PRI, CFI, VID sub-fields) | The 802.1Q tag is used for VLAN identification and usually static. Once the Ethernet header fields are parsed and 802.1Q tag is being used, these fields can be compressed (See our comments for 2.1.2). |
| Nokia | Yes | All | C-TAGs are probably more common across the 5G radio links in the network edges, but S-TAGs can be considered as well. |
| Intel | Yes | TPID and VLAN identifier (VID) | We prefer not to compress QoS related bits like PCP, DE. |
| Samsung | Yes | All | It depends on how to compress them. For example, UDC(Uplink data compression) can be applied to all Ethernet header fields. |
| OPPO | Yes | TPID and VID | Compress TPID and VID at least since the two fields are static. |
| vivo | Yes | TPID and VID | For TPID, it is used to differentiate the untagged ethernt frame, which is fixed value if a Q-tag is included in ethernet frame. For a specific stream originated from a transmitter to 5GS, the VID in each packet are usually unchanged, which can be compressed. |
| CATT | Yes | All | All fields which are likely static can be considered for compression. |
| CMCC | Yes | All | All static fields in the packet all can be compressed by UDC mechanism. |

Summary

All companies are aligned that Q-TAGs are considered for compression. Among the 13 participating, companies, 3 suggest to compress only the TPID and VID fields within the Q-TAG, while 10 companies suggest to compress all fields. Given majority view, we observe:

Q-TAGs (including all sub-fields of C-TAG and S-TAG) are considered for compression.

There may be many other fields within the Ethernet header. The presence of those fields depends heavily on the network scenario.

### Should further fields be considered for compression? Which ones?

| Company | Answer (list of fields) | Comment |
| --- | --- | --- |
| Mediatek | None | Options beyond double tagging (i.e. stacking) should not be considered. |
| Qualcomm | No | Open to considering other fields with good justification |
| Huawei | All content in the IIoT packet can be considered to be compressed by using UDC based mechanism. | In addition to all the Ethernet header fields, the IP/TCP header and even the payload can be considered to be compressed by using UDC based mechanism. The UDC solution in LTE can be reused and can be extended to DL as well. |
| LG | No |  |
| Ericsson | No | For the use-case targeted in this SI, to keep the header compression complexity low, we should not consider any further Ethernet header fields. |
| ZTE | Yes | According to our comments for 2.1.2, besides LENGTH/TYPE and 802.1Q tag, the following other fields may also need to be compressed after the Ethernet header fields are parsed: IEEE 802.2 field, SNAP Header. |
| Nokia | No | Specific upper layer protocols can be considered separately but for Ethernet no additional fields seem relevant. |
| Intel | No other fields |  |
| Samsung | All | Again, It depends on how to compress them. For example, we don't need to decide which field should be compressed or not if UDC is applied. |
| OPPO | FFS | RAN2 can further consider other static fields to be compressed: D/S-SAP if LLC header is present, and OUI if SNAP extension is present. |
| CATT | Other fields even payload by using UDC based solution | UDC based solution can be used to compress any fields even the payload. Details are FFS. |
| CMCC | All | All the static content can be compressed by UDC mechanism. |

Summary

6 companies suggest to not consider further fields in header compression. 4 companies suggest that further fields are used, depending on whether UDC header compression is applied as solution. It is noted that those further fields mentioned by some of these companies are not part of the Ethernet header though. In addition, two companies suggest other fields to be compressed in case of header compression addressing less common encapsulation formats like SNAP/LLC (802.2)

.

No further Ethernet (IEEE 802.3) header fields should be considered for compression.

Another related question is how to consider (if present) padding inside the Ethernet frame from compression perspective. E.g. R2-1817175 discusses the relation of padding and the frame length field.

Detecting padding is simple if the Ethernet frame contains a Length field (note that this is not the case for majority of traffic). In all other cases, it may require deep packet inspection and analysing information of the encapsulated protocol (e.g., IP packet length, etc.) to find out whether padding is present in the frame and how many padding bytes are there. So, considering padding in compression can be a resource-intensive process.

### How is padding considered in compression?

| Company | [considered or not] | Comment |
| --- | --- | --- |
| Mediatek | Considered | Note the min payload size not requiring padding is (bytes): 46 (normal header), 42 (with c-tag), 38 (with c&s-tags) Any packets below this size can be considered for padding removal at the compressor or to restore the padding at the decompressor. The removal simply entails the removal of the trailing padding bytes of the packet. While the restoration entails adding the appropriate padding byes to the tail of the packet. |
| Qualcomm | No | Padding removal is not straightforward given that all ethernet frame types do not have a Length field. If at all necessary, padding should be removed at 5GS ingress point (ie, UPF/UE) along with removal of Preamble, FCS etc. Such a padding removal is in SA2 scope. |
| Huawei | Considered | Support to remove the padding from the Ethernet payload for further efficiency. Padding removal function is configurable depends on whether the length field is present. |
| LG | Not considered | The padding is not a part of the Ethernet header. It would be hard to identify the padding. |
| Ericsson | Not considered | Would create complexity e.g. to be considered in existing header compression schemes, since padding is not part of the header. Furthermore, there is not necessarily a length field i.e. for the majority of traffic it may be required to analyse information of the encapsulated protocol which may be too complicated for the achievable gain. |
| ZTE | Not considered | The following figure is an example of Ethernet frame with minimum 64 bytes length, including the Ethernet header and payload: With the minimum Ethernet header (DST ADR(6Bytes)+SRC ADR(6Bytes)+Length/Type(2Bytes)=14Bytes), minimum TCP/IP header (IP header(20Bytes)+CRC(4Bytes)=24Bytes) and the FCS 4 bytes, the total header/FCS bytes are 42Bytes. And according to the TS 22.104, the minimum MTU sizes is 20Bytes. That means with minimum header/FCS and MTU sizes, only 2Bytes Ethernet padding are needed. In more general cases, the Ethernet header size may be larger than 14Bytes (e.g., at most 30Bytes with IEEE802.3 frame structure including C/S-Tag and SNAP field). The IP/TCP/RTP header size may be larger than 24Bytes (e.g. at most 60Bytes with IP+TCP+RTP+CRC). The MTU size may be larger than 20Byes (usually 20~250Bytes). Then the total length of Ethernet frame would be larger than 64 bytes and only need to be Byte alignment. That means the Ethernet padding bits may be less than 8bits. In a summary, we think the padding may be only several bits or 2 bytes. The benefit of compressing such padding may be marginal and it is not worth the cost of deep packet inspection and analysing information of the encapsulated protocol. |
| Nokia | Not considered | Most real Ethernet protocols do not use the length field so padding removal would have to be based on specific upper layer headers such as IP packet length. The initial solution should focus on Ethernet level only. |
| Intel | Considered | There is always a tradeoff between compression gain and implementation complexity. Given that there is higher gain of Ethernet header compression at smaller packet sizes, it would be better to consider removing padding. |
| Samsung | Considered | Again, It depends on how to compress them. For example, UDC would be applied to both header and data including padding. |
| OPPO | Not considered | The padding is not a part of the Ethernet header. And to compress padding is valuable only when the overhead of data field is less than 46byte. |
| vivo | considered | In IIOT use case，small payload （20~50bites）with padding is common case，removing padding can reduce a large overhead of Ethernet frame transmission. |
| CATT | Not considered | We agree with QC. Padding can be removed at 5GS ingress point (i.e., UPF/UE) along with removal of Preamble, FCS etc. Such a padding removal is in SA2 scope. |
| CMCC | considered | In order to achieve more efficient, we prefer to use UDC to compress the static part. |

Summary

Among the 13 participating companies, 6 companies suggest to consider padding, while 7 companies suggest to not consider padding, pointing among others to the lack of a length field in the header, which would increase complexity of the header compression when considering padding.

Whether the additional complexity of removing padding (i.e. Ethernet payload) is worth to be considered in Ethernet header compression must be justified.

## Evaluation of expected compression gains

In submissions to RAN2#104, initial analyses of the expected compression gains were done, e.g. R2-1817175, R2-1817269, R2-1816689.

First of all, to analyse the gains, assumptions of the Ethernet payload sizes have to be considered. TS 22.104 (current CR S1-183276) considers for example MTU sizes between 20 and 250 bytes in several periodic deterministic communication service use-cases, and mentions also 1k-1.5k as payload size for certain use-cases in electrical power distribution and control-to-control motion control. The question is which of those scenarios should be considered to calculate the gain of header compression, and how to estimate the payload size distribution for the evaluation.

IEEE 802.3 states that the minimum frame length is 64 bytes. Any frame which is received, and which is less than 64 bytes is illegal and dropped by the receiving end.

Size of the Ethernet header is network scenario dependant as well.

### Assumed payload/header sizes when compressing header?

| Company | Payload size range assumed [byte] | Header size range assumed [byte] | Comment |
| --- | --- | --- | --- |
| Mediatek | 20 – 250 20: to evaluate padding removal, Up to 250: where compression gains are significant | Uncompressed header a combination of: 14 (dst,src,et), 18 (dst,src,ctag,et), or 22 (dst,src,stag,ctag,et) | Reducing the overheads introduce by RoHC need to be considered: - Context Id (4, 8, 12, 16 bits) - Checksum (1 byte) - Packet Type (1 byte) - Profile Id (1 byte) - Sequence number (4, 6, 8, 12 bits) - Used for feedback, but not needed since Ethernet packets are self-contained |
| Qualcomm | 46 bytes and above | 14 bytes | Minimum allowed payload for Ethernet is 46 bytes which leads to total size of 64 bytes (including 14 bytes thernet header and 4 bytes FCS). Compressed header assumed to be 2 bytes. |
| Huawei | 20 ~250 | 14~22 | As described in 2.1.4, should not only consider the header but also the payload to be compressed. |
| LG | Minimum size : 46 Maximum size: 200 | 14 – 22 (dst, src, ethertype, stag,ctag) | After exchanging the static header information between compression and de-compression, the Context ID is required. - Context ID: 1byte |
| Ericsson | 46 - | 18 | Scenario dependent. Significant gain can be provided only for small Ethernet frames (64-128 bytes). Overall gain depends on the Frame size distribution. Details in R2-1817175 Ethernet Header Compression |
| ZTE | 30~250Bytes (with minimum 64bytes Ethernet frame length, if 30Bytes Ethernet header is used and 4Bytes FCS is included, the minimum payload would be 30Bytes) | 12~30Bytes (12 Bytes: only compressing the DST ADR and SRC ADR. 30 Bytes: compressing all the header fields of IEEE802.3 frame structure including C/S-Tag and SNAP fields.) | Compared with RoHC, we agree with some above comments that after Ethernet header compression, the following additional information (e.g., final compressed header) would be needed and length of 1 Byte may be enough for each field: - Packet Type:1Byte - Profile ID: 1Byte - Context ID: 1Byte - CRC: 1Byte For the Sequence number, as the maximum Ethernet data field length is 1500Bytes which is generally enough to accommodate most of the IIoT data in one packet, no segmentation is needed. If the Ethernet data needs to be segmented in Uu interface, it could be implemented based on the Uu interface scheme (e.g. implemented in RLC layer). |
| Nokia | Min size 38-46 | 14-22 | Most relevant are e.g. industrial protocols that can fit their headers and payload into the minimum sized Ethernet frame. Applications using larger frame sizes (upto 1500 bytes) are not really the key use case here. |
| Intel | 20-50 | 22 | Regarding payload size, we prefer to evaluate the most demanding scenarios where the required latency is tight (0.5~2 ms). In TS 22.104 Table 5.2-1, the message size for such scenarios are 20, 40, and 50 bytes. Other message sizes are not excluded for evaluation, and it is expected that the gain of header compression decreases when message size increases. As to header size range, it depends on whether we consider Preamble, SFD and FCS are part of header before compression. By definition, Preamble and SFD are not part of Ethernet MAC frame, but FCS is. So in our assumption, following fields are considered in the header before compression: Destination Address, Source Address, Length/Type field and 802.1Q tag, FCS) It remains to be discussed whether FCS should be considered or not as the gain from Ethernet header compression given that TS 23.501 already requires that FCS is not transmitted in 5GS. |
| Samsung | 46 ~ | 18 | It depends on scenario. |
| OPPO | Min size 46, or 38 | 12-20 (12 Bytes: only compressing the DST ADR and SRC ADR. 20 Bytes: compressing DST ADR, SRC ADR, Ether Type, all static field in C/S-Tag), or 12-26 (26 Bytes: compressing DST ADR, SRC ADR, Ether Type, all static field in C/S-Tag, D/S-SAP,OUI) | Scenario dependent. For 802.3 format, higher compression efficiency can be achieved if D/S-SAP and OUI can be considered for header compression. |
| vivo | 20~250bytes | 18 bytes of Ethernet header which has one Q-tag in frame | Payload size with 20~250 bytes are common case in industrial area, it can be considered for compression. For the use case with payload size with 1k-1.5k, it is not necessary to perfom Ethernet header compression as the actual gain is very small even though removing all the fields in Ethernet header. |
| CATT | ~46 bytes | 14-22bytes | At least for header compression, compressed header assumed to be 1~2 bytes. UDC is FFS. |
| CMCC | 46 bytes ~ | 14 bytes ~ | Scenario dependent. |

Summary

The indicated values are considered for the text proposal below.

Companies are invited to provide some expected gain figures in this section, stating the assumed payload size and compression ratio for a given header structure and/or referring to existing evaluation results. These first results may be based on a minimum set of headers considered for compression, while the subsequent questions aim at expected results considering a more sophisticated compression method.

### Are expected gains considering simple/minimum compression setup significant?

| Company | Gains significant? [yes/no] | Approximate gains for payload/ header sizes, and compression ratios | Comment Explanation of assumed compression ratio |
| --- | --- | --- | --- |
| Mediatek | Yes | An uncompressed Ethernet header (14, 18, 22 from 2.2.1) provides the following savings for a scheme capable of compressing the header to 2 bytes: Original 14: 86% Original 18: 89% Original 22: 91% For a 64 byte packet the net savings for a 2 byte header is: Original 14: 19% Original 18: 25% Original 22: 31% | Ethernet header fields are all considered “static” and therefore the purpose of the compression scheme is merely to relay the packet stream between the compressor and its peer decompressor together with some form of feedback. The overhead introduced by such a compression scheme can be of the order of 1 to 2 bytes and largely dependent on the size of the CID. |
| Qualcomm | Yes | 20% for 46 bytes payload | (46+14) bytes is total size of payload and uncompressed ethernet header. (46+2) bytes is total size of payload and compressed ethernet header. |
| Huawei | yes | Assume that the payload size is 20 byte, Ethernet header is 18 byte, and IP header is 40 byte. The compression ratios can be at least 44%. | Assumed compression ratio when UDC is applied: Ethernet header 18->2 IP header 40->28 Payload 20->15 |
| LG | Depending on payload size. | Considering maximum payload size (e.g., 200) and header size (e.g., 22), the compression ratios is 9%. Considering minimum payload size (e.g., 46) and header size (e.g., 14), the compression ratios is 21%. | The maximum size of the payload is 200 and the maximum size of the Ethernet header is 22. The Ethernet header can be compressed to 1 byte (for Context ID) Considering that, the compression ratios can be 9%. The minimum size of the payload is 46 and the minimum size of the Ethernet header is 14. Thus, the compression ratios can be 21%. |
| Ericsson | Yes/Maybe. | Savings from approx. 20% for small frames; less with increased frame sized. | Depending on scenario etc. Details in R2-1817175 Ethernet Header Compression |
| ZTE | Yes | 3.03%~43.33% | We assume the gain can be the ratio of the length of the head fields which are compressed to the length of the final transmitted Ethernet frame. We give the following example formula: Gain = (Compressible Ethernet header – final compressed header(fixed 4Bytes according to our comments for 2.2.1)) / (Ethernet header + payload (including possible padding)) Here, (Ethernet header + payload (including possible padding)) should be not less than 60Bytes. Based on the combination of minimum/maximum header and minimum/maximum payload in the following table, we can calculate the approximate gain range is 3.03%~43.33%: Compressible Ethernet header; Ethernet header; payload; Gain(%). 12; 14; 46; 13.33. 12; 14; 250; 3.03. 30; 30; 30; 43.33. 30; 30; 250; 9.29. |
| Nokia | Yes (for specific apps) | Best case is reducing the overall minimum frame of 60 bytes to ~39-47 bytes by compressing the whole Eth header to 1 byte. | Assuming a very small payload inside the Ethernet frame, not exceeding the 60 byte minimum for the whole frame size. These are used e.g. in specific industrial applications requiring deterministic low latency delivery. |
| Intel | Yes | Compression gain is 31.25%, 31.25% and 27.78% for message size of 20, 40, and 50 bytes without removing the padding bytes, respectively. Compression gain is 65.63%, 34.38% and 27.78% for message size of 20, 40, and 50 bytes after removing the padding bytes, respectively. | Details in R2-1816765. |
| Samsung | Yes | At least 40% | In Rel-15 LTE UDC SI phase, many companies simulated the UDC algorithm for various traffics. The minimum compression ratio was almost 40%. Note that this compression ratio is for the total size of header and payload. |
| OPPO | Yes | Min: ~15% | Assume (46+14) bytes is total size of payload and uncompressed Ethernet header, and (46+5) bytes is related to the compressed packet. |
| vivo | Yes, depending on payload size | For the payload size 20~250bytes, the gain is from approx. 30% to 5%. | Compression gain of Ethernet header depends on payload size and padding, gain will be small when the payload size exceeds 100 bytes. |
| CATT | Yes | Considering minimum payload size (e.g., 46) and header size (e.g., 22), the compression ratios is 31%. | UDC is FFS. |
| CMCC | Yes | At least 40%, based on the evaluation result in LTE UDC SI. |  |

Summary

All companies see significant expected gains in header compression, while some of which point out that this is depending on packet size and is only the case for very small payload sizes.

Ethernet header compression gains are considered significant.

Involving more fields in the compression may increase both gain and complexity of compression. The added complexity of header compression (e.g., CPU cycles, memory, etc. required for compression) should be balanced with the achievable gains. The question is whether companies see benefit in terms of significant further gain considering those more sophisticated compression methods.

### Are further significant gains expected with more than simple/minimum compression setup?

| Company | Approximate gains for payload sizes, and compression ratios | Comment Explanation of assumed compression ratio Additional headers considered |
| --- | --- | --- |
| Mediatek | No further gains | A sophisticated compression mechanism beyond handling static fields is not required. |
| Qualcomm |  | Agree with Mediatek. Focus should be on removal of static fields. |
| Huawei | Same as 2.2.2 | Same as 2.2.2 |
| LG | No | Only static part of the Ethernet header should be considered. |
| Ericsson | No | Focus on static fields. |
| ZTE | N/A | See our comments for 2.2.2 |
| Nokia | No | Support for specific upper layer protocols can be considered separately but at this phase the focus should be on the Ethernet headers identified in 2.1.2 and 2.1.3. |
| Intel | No |  |
| Samsung | Based on traffic pattern | The traffic pattern has an impact on the performance of UDC algorithm. |
| OPPO |  | Focus on static fields. |
| CATT | Yes | Payload size gains can be achieved with UDC |
| CMCC | Yes | We prefer to compress the static content for both header and payload by UDC mechanism. |

Summary

7 companies do not think further significant gains are to be expected with more than minimum compression setup, e.g. more than focussing on static fields. Other companies point out the dependency on traffic pattern, compression method, and whether padding is considered for compression.

## Applicability to certain industrial communication protocols

Beside support of generic Ethernet header compression, i.e. for IEEE 802.3 and/or IEEE 802.1Q, additional support for e.g. a wide range of industrial field-bus technologies building on those protocols, could potentially be considered, as discussed in R2-1817572. This points behind header compression as it requires deep packet inspection and support of many flavours of industrial protocols. Should those or other communication protocols building on Ethernet be considered for compression generically or in particular?

### Should further industrial communication protocols above Ethernet be considered in header compression?

| Company | [yes / no] | Comment (if yes, which ones and why?) |
| --- | --- | --- |
| Mediatek | No | The starting point is to focus on the objective of Ethernet header compression and to de-prioritize the widening of the WI scope given the time constraints. |
| Qualcomm | No | Not in Rel-16 without a strong justification including clear demand from IIoT use cases and substantial gains. |
| Huawei | Yes | Same as 2.2, by using UDC based mechanism, all the contents including Ethernet header, IP header and payload can be simultaneously compressed with the existing UDC algorithm. |
| LG | No |  |
| Ericsson | No | Focus on Ethernet header compression only. |
| ZTE | Maybe | The header of EtherCAT, Profinet and PROFIsafe as discussed in R2-1817572 can also be considered if they are used widely in TSN network. This can be confirmed by SA2. |
| Nokia | No | Prioritize Ethernet header compression. Additional upper layer protocols could be considered as a second step based on need. |
| Intel | No | Our understanding is that the specific protocols to support depends on requirements. So far, it seems that there are no requirements have been specified to support industrial communications protocols above Ethernet from TS 22.104, or TS 22.261. Note that TS 23.501 supports Ethernet as PDU session type (in addition to IP protocols), but protocols above Ethernet (except for IP) are not supported. We also note that the additional headers fields from some of the protocols above Ethernet are rather small compared with Ethernet header, therefore compression Ethernet header alone can already achieve sufficient gain. On the other hand, supporting many flavours of industrial protocols add more complexity, and requires collaboration from other WGs (e.g. SA1 and SA2). |
| Samsung | Yes | UDC would be applied to both header and payload(including other upper layer headers). Considering the time constraints, it would be better to use the conventional mechanism, which was already specified in PDCP specification. |
| vivo | No | The current Ethernet packet filter set does not contain parameters related to the industrial communication protocols above Ethernet, compression those fields of the industrial communication protocols above Ethernet will leads to high specification complexity. If RAN parses packet and differentiates data stream while performing Ethernet header compression, the coordination between compressor and de-compressor may be needed to setup specific context like ROHC. |
| CATT | Yes | UDC can be used to compress the whole packet |
| CMCC | Yes | As stated above, UDC is possible to applied to both header and payload. |

Summary

While 7 companies would not like to consider industrial protocols above Ethernet, 4 companies would like to consider them, and 1 company is undecided. We note that the SID points at Ethernet header compression only, not considering further protocols.

Further industrial protocols above Ethernet are not in focus to be considered in structure-aware Ethernet header compression.

## Protocol layer and network termination point

Another discussion point is the placement of the header compression function, i.e. in terms of protocol layer as well as network protocol termination point. IP header compression today is done by RoHC, which is part of PDCP, thus compression is done between UE and gNB. Ethernet header compression could be anchored similarly on PDCP. See for example R2-1817913 or R2-1816765 for further details. An alternative option, mentioned in R2-1816689, would be header compression between UE and UPF, where the (Ethernet) PDU session is terminated.

Figure 1: Potential Ethernet header compression anchor points, from R2-1816689.

In the following, companies are invited to provide their view and reasoning on protocol layer and network termination point for the Ethernet header compression protocol.

### Which protocol layer and network termination point shall support the Ethernet header compression functionality?

| Company | protocol layer | network termination point | Comments – Why? |
| --- | --- | --- | --- |
| Mediatek | PDCP | gNB | Since PDCP already supports RoHC, it is the natural place to do Ethernet header compression. It would result in a self-contained solution within RAN2’s remit. |
| Qualcomm | PDCP | gNB | PDCP layer in gNB because easier to build on current RoHC framework, leading to smaller specification impact and easier implementation. |
| Huawei | PDCP | gNB | LTE UDC solution can be reused. Same as in LTE, PDCP protocol layer to support the UDC function. |
| LG | PDCP | gNB | The RLC and MAC cannot identify the Ethernet header since the ciphering is performed in PDCP. Thus, the Ethernet compression can be applied in the SDAP or PDCP. Typically, PDCP is in charge of the compression mechanism such as ROHC and UDC. Otherwise, SDAP maps the QoS flow to DRB. Consequently, we think that the Ethernet header compression should be performed in PDCP. Considering that the Ethernet header compression is performed in PDCP, the network termination point should be gNB. |
| Ericsson | PDCP | gNB | Agree with Mediatek. |
| ZTE | PDCP | gNB | Agree with some above comments. Moreover, the Ethernet header compression configuration and reconfiguration done by gNB would be more efficiently than by UPF. |
| Nokia | PDCP | gNB | RoHC reuse, limiting impact to UE and RAN only. |
| Intel | PDCP | gNB | As explained in R2-1816765, although UPF as protocol termination point has the benefits of reduced overhead over N3 interface, the gain is not that critical since N3 interface should have sufficient bandwidth. On the other hand, having UPF as protocol termination point has following drawbacks: - There is no interface defined between UE and UPF. To support UPF as protocol termination point, a new protocol layer on top of SDAP should be introduced with UE and UPF as protocol termination points. This could result in significant complexity increase. - Even if a new protocol layer can be introduced, the interaction between gNB and UPF regarding Ethernet header compression will be complicated especially when Ethernet frames carry IP packet. For IP packet, ROHC can be used to reduce header overhead. For correct ROHC operation, it is expected that gNB should be able to perform Ethernet header compression related operations, which breaks protocol isolation principle and adds unnecessary complexity. Regarding to which protocol layer Ethernet header compression functionality belongs. Firstly, it is not suitable to perform Ethernet header compression in MAC or RLC layer due to the encryption / integrity protection performed in PDCP layer. Secondly, SDAP layer is not a suitable layer either. SDAP layer mainly maps QoS flows to DRBs. Adding Ethernet header compression functionality does not conform to the main functionality of SDAP layer. Lastly, adding a new layer on top of SDAP layer purely for Ethernet header compression is not justified. There are additional complexity when Ethernet header compression and ROHC are used together if Ethernet header compression is in a different layer other than PDCP. Given that ROHC is PDCP layer functionality and Ethernet header compression has the same motivation as ROHC, it is natural that Ethernet header compression functionality is in PDCP layer. |
| Samsung | PDCP | gNB | UDC was already specified in LTE PDCP specification. |
| OPPO | PDCP | gNB | Agree with Qualcomm. |
| vivo | PDCP | gNB | We would prefer to reuse the framework of ROHC and UDC as implemented in the PDCP |
| CATT | PDCP | gNB | Since PDCP already supports RoHC and UDC (in LTE), it is the natural place to do Ethernet packet compression. |
| CMCC | PDCP | gNB | The existing framework of ROHC and UDC is preferred. |

Summary:

Companies are aligned that Ethernet header compression shall be anchored at gNB at PDCP.

PDCP at gNB is the network anchor for Ethernet header compression.

## Relation of Ethernet and IP header compression

Ethernet may transport IP or non-IP traffic. This aspect is mentioned in e.g. R2-1817572. It shall be discussed whether as part of this study, also IP header compression shall be considered when Ethernet header compression is employed. If yes, independent/separate header compression solutions can be envisaged as well as a joint/integrated header compression of Ethernet and IP. Companies are invited to share their view on this issue below.

### Is IP header compression also applicable when Ethernet header compression is done, and if, separately or jointly?

| Company | [Eth xor IP / both separate / both joint] | Comment |
| --- | --- | --- |
| Mediatek | Ethernet/ separate | As indicated in section 2.3.1, we should only focus on the objective of Ethernet header compression in this WI, given the limited time allocated. |
| Qualcomm | No | Most real-time industrial applications don’t use IP Ethertype (e.g. Profinet (Ethertype 0x8892), EtherCAT (Ethertype 0x88A4), SERCOS (Ethertype 0x88CD) and OPC (Ethertype 0xB62C)). Protocols that use IP Ethertype should first be studied to see if payloads are indeed small to benefit from header compression. |
| Huawei | both joint, by using UDC based mechanism | For UDC based mechanism, all the fields in a packet can be jointly compressed using UDC algorithm. It is more efficient and simpler. |
| LG | Ethernet/ separate | Considering the packet can be encapsulated by IP and Ethernet, the Ethernet compression and ROHC should be separated. |
| Ericsson | No joint algorithm | Modular approach. First start could be to work on Ethernet header compression only, given also the SI scope. |
| ZTE | both separate | As it’s better to compress the header as much as possible to improve the transmission efficiency, we think both header compression schemes could be supported. For simplicity, flexibility and also considering that the IP header compression has already been supported in RAN, the two compression schemes should be performed separately. |
| Nokia | No joint algorithm necessary | The main solution/profile to be defined should be Ethernet only. That should work in sequence/stack with existing IP-based RoHC profiles. In the future an integrated UDP/IP/Ethernet profile could be considered as an optimization. |
| Intel | Both separate | We think IP header compression is applicable when Ethernet header compression is done, and the compression for IP header and Ethernet header should be done separately. |
| Samsung | Both joint | Again, it depends on how to compress them. For example, UDC can be applied to both IP header and Ethernet header. That's why the network doesn't have to configure both UDC and ROHC. It is enough to configure UDC only. |
| OPPO | Both separate | Agree with ZTE and Intel. |
| vivo | Both separate | According to the current industrial network, the IP is widely used above the Ethernet protocol. To simplify the compression of the Ethernet header, we prefer to have separate header compression modules for IP header and Ethernet header, and we can re-use the ROHC for the IP header compression. |
| CATT | Separate or joint with UDC | With UDC, both can be done jointly. Otherwise, with an adhoc solution they should be separate. |
| CMCC | Both joint if UDC is applied | If UDC is applied, both IP header and Ethernet header can be compressed jointly. |

Summary

Some companies suggest to focus on Ethernet header compression while majority would consider both Ethernet and IP header compression to be considered.

The majority of companies suggest separate Ethernet and IP header compression. Joint header compression is seen as beneficial by 4 companies if a solution e.g. based on UDC were used.

A structure-aware Ethernet header compression scheme does not consider IP header compression within a joint solution.

## Specification

Clarification is required on how the Ethernet header compression function can actually be specified i.e. which specification body would be responsible and/or initiate the specification of the solution. R2-1817269, R2-1816765 and  R2-1817913 discuss different options, e.g.:

- As new ROHC profile. Considering existing RoHC framework as specified by IETF (RFC 3095, https://tools.ietf.org/html/rfc3095) as generic framework, for which new Ethernet profile can be specified.

  - Development could be done by 3GPP RAN2. Potentially verified by IETF and/or IEEE.

  - Alternatives? IETF: was responsible working group for RoHC, inactive for several years (as noted in R2-1817269), and that IETF is actually responsible for IP and not Ethernet. Or, IEEE, as responsible standardization body for Ethernet.

- Extension of Static Context Header Compression (SCHC), IETF, https://tools.ietf.org/wg/lpwan/draft-ietf-lpwan-ipv6-static-context-hc/.

- New specification, developed by 3GPP RAN2.

What are the companies’ views on how Ethernet header compression should be specified?

### How should Ethernet header compression be specified?

| Company | Answer [ROHC,SCHC, New] | Comment |
| --- | --- | --- |
| Mediatek | New PDCP based compression | This can be in the form of an Extended PDCP specification, defining: - Simple RoHC based solution given the static nature of the Ethernet fields, or - A new tailored compression scheme independently devised and managed by RAN2. |
| Qualcomm | RoHC with 3GPP RAN2 specification | RoHC based approach has several benefits. For instance, it can build on existing RoHC framework support in NR and will thus have smaller specification impact and easier implementation. RoHC based approach however will require development of a RoHC profile, which will be easier than RAN2 developing a completely new compression scheme for Ethernet. Our preferred view is that RAN2 develops the new RoHC profile and works with other organizations on dependencies such as RoHC profile ID assignment. Update: RoHC based approach is better than UDC for Ethernet header compression in TSN use cases as explained below: - RoHC is less complex and better suited for stringent TSN latency requirements: TSN use cases can require end-to-end latency as low as 0.5 ms. Compression time and decompression time also contribute towards end-to-end latency. More complex compression scheme leads to more compression time and decompression time, and makes it challenging to meet end-to-end latency requirements. Structure-awareness (knowledge of structure of fields being compressed) can help reduce complexity. RoHC can be structure-aware since RoHC profile for Ethernet can be defined based on ethernet header fields. UDC is designed to learn structure of information on the fly and this learning introduces additional complexity. - RoHC is more robust to packet losses: This is because RoHC’s contexts can rely on static fields (which do not change during lifetime of flow) in ethernet headers, whereas UDC context depends on previous packets. Note that TSN use cases can tolerate some packet losses (even with their high availability requirements) if they have non-zero survival time. |
| Huawei | Using UDC based mechanism in PDCP | Reuse LTE UDC, and extend UDC to DL as well. Update: ROHC and UDC have diverse pros and cons: • ROHC needs to gradually establish compression/decompression context which will incur extra delay, compared with UDC establishment of compression/decompression context. • Overhead of ROHC could be large as ROHC needs to sync compressor and decompressor by exchanging messages on context. • UDC can compress/decompress jointly Ethernet header and IP header, even payload, which can decrease overall compression/decompression latency and complexity, compared with sequential processing Ethernet header compression/decompress and IP header compression/decompression based on ROHC. • UDC’s straightforward matching/comparison mechanism requires less feedback overhead and lower latency. • Reusing LTE UDC as baseline can minimize standardization effort. |
| LG | SCHC | The Ethernet header compression is not supported by ROHC. It means that the Ethernet packet should not be compressed by ROHC unless the new ROHC profile for Ethernet header compression is introduced. In our view, the definition of the new ROHC profile is not a part of the 3GPP work. For this reason, the 3GPP applies the compression mechanism defined by IETF. We think that the SCHC defined by the IETF can be a compression method for Ethernet compression. However, RAN2 should look over the SCHC mechanism in detail. |
| Ericsson | RoHC profile with 3GPP RAN2 specification | Agree with Qualcomm. |
| ZTE | New PDCP based compression, specified by 3GPP RAN2 | Agree with Mediatek. |
| Nokia | RoHC profile with 3GPP RAN2 specification, with IETF review | Agree with Qualcomm and Ericsson. RoHC is the right tool for the need which is URLLC/TSN type of communication with small frame sizes due to its efficiency compared to generic approaches such as UDC/DEFLATE. |
| Intel | New PDCP based compression | As discussed in section 2.4.1, our preference is to specify Ethernet header compression in PDCP layer, so our preference is to specify Ethernet header compression functionality in RAN2 directly in TS 38.323 without defining additional ROHC profile or using SCHC. Regarding defining ROHC profile, as explained in R2-1816765, although there might be potential benefit of reusing ROHC framework, there are also several issues: - ROHC profiles are defined by IETF for TCP/UDP/RTP/IP protocols, and Ethernet is defined by IEEE. Strictly speaking, 3GPP cannot define a ROHC profile unless 3GPP plans to communicate IETF/IEEE to adopt new ROHC profile. In addition, ROHC profile identifiers are registered by Internet Assigned Numbers Authority (IANA), therefore liaison with IANA is also needed. Such collaboration/liaison with other standard bodies add uncertainties and could delay the work completion. - As ROHC framework and profiles are defined by IETF, defining a ROHC profile in RAN2 needs expertise for IETF. Although not a show stopper, this is a disadvantage compared with directly defining Ethernet header compression in RAN2 directly. Regarding Static Context Header Compression (SCHC), our understanding is that SCHC is defined by IETF for IPv6 and UDP protocols, and therefore are not applicable for Ethernet header compression. |
| Samsung | UDC (Uplink data compression) | Already specified in LTE PDCP specification. The enhancement can be studied further. |
| OPPO | RoHC with 3GPP RAN2 specification | Agree with Qualcomm. |
| vivo | New compression algorithm | To avoid the impacts on the other specification group(s), we would prefer to have a new compression scheme implemented in PDCP. |
| CATT | UDC based solution or new PDCP based compression | Re-using LTE UDC can be the most straightforward approach. Otherwise we agree with Mediatek’s views. |
| CMCC | UDC | The UDC mechanism specified in LTE PDCP specification be regarded as baseline. Necessary enhancement can be studied. |

Summary

From the 12 companies:

- 4 suggest ROHC-based scheme

- 2 suggest ROHC-based scheme or new PDCP based scheme

- 2 suggest new PDCP based scheme (not ROHC)

- 4 suggest UDC based scheme.

- 1 suggest SCHC based scheme.

It becomes obvious from this outcome that support is seen among companies for three different solution approaches.

The following potential solutions for Ethernet header compression are considered for further evaluation: 1) ROHC-based scheme, 2) new PDCP-based scheme (not ROHC), 3) UDC-based scheme.

As became apparent when considering all questions of this email discussion there is a dependency between header field selection and expected gains on the solution approach taken. To provide a recommendation for header compression overall, as a result of the study item, we suggest discussing and down-selecting which of the header compression solutions should be further considered.

Down-select among the header compression solutions.

# Conclusion

Done after intermediate deadline for input, 2019-01-31

- Summaries for the questions discussed in this email discussion are provided within the sections above, highlighted in blue. Resulting observations and proposals are listed as follows, and a text proposal is given below.

Observation 1	Preamble, SFD and FCS are ignored are not transmitted thus not considered in Ethernet header compression.

Observation 2	At least header fields DST ADR, SRC ADR, ETHER ETYPE are considered for compression.

Observation 3	Q-TAGs (including all sub-fields of C-TAG and S-TAG) are considered for compression.

Observation 4	No further Ethernet (IEEE 802.3) header fields should be considered for compression.

Observation 5	Whether the additional complexity of removing padding (i.e. Ethernet payload) is worth to be considered in Ethernet header compression must be justified.

Observation 6	Ethernet header compression gains are considered significant.

Observation 7	Further industrial protocols above Ethernet are not in focus to be considered in structure-aware Ethernet header compression.

Observation 8	PDCP at gNB is the network anchor for Ethernet header compression.

Observation 9	A structure-aware Ethernet header compression scheme does not consider IP header compression within a joint solution.

Observation 10	The following potential solutions for Ethernet header compression are considered for further evaluation: 1) ROHC-based scheme, 2) new PDCP-based scheme (not ROHC), 3) UDC-based scheme.

Proposal 1	Down-select among the header compression solutions.

# Text proposal TR 38.825

## 6.6	Ethernet header compression

### 6.6.1	Scenario and benefits assessment

Editor’s note: RAN2 responsibility

Ethernet header compression is a means to reduce the overhead induced by transport of the Ethernet header. Ethernet frames may be transported over the 5G system in an Ethernet PDU session type. Header compression may especially be beneficial when payload sizes are relatively small compared to the overall size of the frame, which is typically the case in Industrial IoT networks based on Ethernet.

#### 6.6.1.1	Ethernet header fields

As an overview, Figure 1 shows the IEEE 802.3 MAC Ethernet [ref] frame format.

Figure 1, Example of IEEE 802.3 MAC frame format – 802.1Q ([ref] 2014)

Some parts of the Ethernet frame do not need transmission over the 5G system, therefore TS 23.501 [ref] states for the PDU session type:

Ethernet Preamble and Start of Frame delimiter are not sent over 5GS:

-	For UL traffic the UE strips the preamble and frame check sequence (FCS) from the Ethernet frame.

-	For DL traffic the PDU Session Anchor strips the preamble and frame check sequence (FCS) from the Ethernet frame.

Therefore, the fields PREAMBLE, SFD and FRAME CHECK SEQUENCE are not transmitted over the 5G system and can just be ignored in Ethernet header compression.

Furthermore, the DESTINATION ADDRESS (6 octets) and SOURCE ADDRESS (6 octets) fields provide the basic identifies of a station within the Ethernet layer. These fields may be static among subsequent packets, which can be exploited for compression.

The LENGTH/TYPE (2 octets) field may be used for length information or to define the 802.1QTagType, and can be used in compression. In most case where this is a TYPE-field the value would be static.

Tagging (Q-Tags) is widely used in Ethernet networks for traffic separation and marking. During forwarding various tags may be added to and removed from user data frames by the tag encoding and decoding functions of L2 nodes. The most common Q-Tags named explicitly for Ethernet header compression related contributions are:

- C-Tag: Tag Protocol Identifier (TPID) and Tag Control Information (TCI)

- S-Tag: Tag Protocol Identifier (TPID) + Tag Control Information (TCI)

These Q-Tags include not only a VLAN IDENTIFIER, but bits used for QoS purposes (e.g., PCP, DE) as well. Compression may not consider for example those QoS related bits.

Q-TAGs and all their sub-fields may be static, which can be used in compression.

There may be further fields within the Ethernet header for certain use-cases, which were determined not to be useful to be considered in structure-aware header compression.

#### 6.6.1.2 Padding

The allowed minimum Ethernet frame length is 64 bytes. Any frame which is less than 64 bytes is illegal and dropped. Padding will be used to fill up the payload field of the Ethernet frame for payload data sizes smaller than the allowed field sizes. Padding is redundant, and furthermore, the 5G system transporting Ethernet frames does not expect Ethernet frame payload sizes of a certain defined length. Therefore, padding may either be removed, e.g. at PDU session level, or compressed as part of the header compression. Due to the absence of a length field in the Ethernet header indicating the actual payload size, additional complexity is required, when determining and compressing padding part of the payload field. The complexity property is related to the applied compression solution.

#### 6.6.1.3 Benefit assessment

First of all, to analyse the gains, assumptions of the Ethernet payload sizes have to be considered. TS 22.104 (current CR S1-183276) considers for example MTU sizes between 20 and 250 byte in several periodic deterministic communication service use-cases, and mentions also 1k-1.5k as payload size for certain use-cases in electrical power distribution and control-to-control motion control.

Given the minimum Ethernet frame size of 64byte, and assuming an Ethernet header size between 14byte and 22byte (considering SOURCE ADR, DESTINATION ADR, LENGTH/TYPE and potentially Q-TAGs), payload up to 40byte in the minimum Ethernet frame size can be considered. Padding would be used for lower actual payload sizes.

To estimate the benefit of header compression, an assumption needs to be taken to which size the header fields can be compressed. The following tables assume compression from 18 byte to 1, 3, 5 bytes respectively.

From R2-1816689 relative gains in terms of reduction of overall frame size, assuming 1 byte compressed header.

| Header size without compression | Overall frame size without compression | Header size with compression | Overall frame size with compression | Relative reduction in overall frame size |
| --- | --- | --- | --- | --- |
| 18 | 64 | 1 | 47 | 27% |
| 18 | 128 | 1 | 111 | 13% |
| 18 | 1518 | 1 | 1507 | 1% |

From R2-1817175, relative gains in terms of reduction of overall frame size, assuming 3bytes and 5bytes compressed header (from 18byte uncompressed header).

Significant gains become obvious in the range of ~20-27% for frame sizes of 64byte, as well as ~10-13% for frame sizes of 128 byte, and moderate gains of ~5% for frame sizes of 256 bytes. For the maximum frame size of 1518 only marginal gains of around 1% are observed.

There is a trade-off between compression complexity and efficiency, i.e. to which minimum size the header can be compressed, with which effort e.g. in terms memory/cycle times. Given the analysis above showing high gains already for compression towards 3 or 5 bytes headers (instead of towards 1 byte), it appears sufficient to consider a simple solution only e.g. focussing on static header fields.

It is noted that potential additional gain in compressing/removing padding is not considered in above numbers. Also, compression schemes going beyond compressing the Ethernet header are not considered in the analysis above.

#### 6.6.1.4 Industrial communication protocols using Ethernet

Beside support of generic Ethernet header compression, i.e. for IEEE 802.3 and/or IEEE 802.1Q, additional support for e.g. a wide range of industrial field-bus technologies building on those protocols, could potentially be considered, as discussed in R2-1817572. It would require deep packet inspection to support many flavours of industrial protocols, leading to higher complexities.

### 6.6.2	Potential solutions

Editor’s note: RAN2 responsibility

#### 6.6.2.1 Protocol layer

One aspect of the solution development of the Ethernet header compression function is its placement, i.e. in terms of protocol layer as well as network protocol termination point. IP header compression today is done by Robust Header Compression (RoHC) [ref], which is part of PDCP, thus compression is done between UE and gNB. Ethernet header compression could be anchored similarly on PDCP. See for example R2-1817913 or R2-1816765 for further details. An alternative option, mentioned in R2-1816689, would be header compression between UE and UPF, where the (Ethernet) PDU session is terminated.

Figure 1: Potential Ethernet header compression anchor points, from R2-1816689.

Overhead reduction resulting from Ethernet header compression may be only relevant over the radio interface Uu, assuming that the (wired) N3 interface has sufficient bandwidth. Therefore, Ethernet header compression between UE and gNB is the only relevant option (i.e. indicated as 2) in the figure. Furthermore, the natural protocol level choice is PDCP, as it is the layer that today provides ROHC functionality. As concluded from R2-1816765, it would not be suitable to perform Ethernet header compression in MAC or RLC layer due to the encryption / integrity protection performed in PDCP layer. SDAP layer is not a suitable layer either. SDAP layer mainly maps QoS flows to DRBs. Adding Ethernet header compression functionality does not conform to the main functionality of SDAP layer. Lastly, adding a new layer on top of SDAP layer purely for Ethernet header compression is not justified. There is additional complexity when Ethernet header compression and ROHC are used together if Ethernet header compression is in a different layer other than PDCP. Given that ROHC is PDCP layer functionality and Ethernet header compression has the same motivation as ROHC, it is natural that Ethernet header compression functionality is in PDCP layer.

#### 6.6.2.3 Relation to IP header compression

Ethernet may transport IP or non-IP traffic. This aspect is mentioned in e.g. R2-1817572. IP header compression may be considered when Ethernet header compression is employed, in which case both independent/separate header compression solutions can be envisaged as well as a joint/integrated header compression of Ethernet and IP. Given the complexities inherent in a joint compression scheme, and the benefits of a modular approach, the developed structure-aware Ethernet header compression scheme should not consider IP header compression within a joint solution.

#### 6.6.2.4 Design principles

Based on the analysis above, as a summary, the solution for header compression would build on the following principles:

- Preamble, SFD and FCS are ignored are not transmitted thus not considered in Ethernet header compression.

- Ethernet header compression considers the header fields DESTINATION ADDRESS, SOURCE ADDRESS, TYPE/LENGTH, Q-TAGs (including all sub-fields), but no further fields of the Ethernet header for structure-aware compression solution.

- Additional complexity of removing padding in Ethernet header compression must be justified..

- Further industrial protocols above Ethernet are not considered in structure-aware Ethernet header compression.

- The developed structure-aware Ethernet header compression scheme does not consider IP header compression within a joint solution.

- For structure-agnostic compression scheme, further fields, further industrial protocols above Ethernet, IP header are compressed together with Ethernet header.

- PDCP at gNB is the network anchor for Ethernet header compression.

#### 6.6.2.3 Solution approaches

The following potential solution approaches were discussed in this study:

New PDCP solution

Ethernet header compression may be specified by RAN2 within a new solution integrated in PDCP. This study did not compare the performance in terms of complexity and compression ratio/gains between New PDCP solution and other solutions such as ROHC.The advantage of this approach would lie in the standardization process, i.e. a pure RAN2-based solution would avoid potential impact/dependencies to other specification groups or bodies.

ROHC-based solution

Ethernet header compression may build on the ROHC-framework, which is currently used for IP-header compression and applied on PDCP layer. A ROHC profile specific to the Ethernet header would need to be defined, meaning that the existing ROHC framework and features would be reused and do not need to be developed. R2-1817913 analyses the benefits of this approach. For example, robustness of compression against packet losses and in-built handling for multiple flows. On the other hand, ROHC profiles are defined by IETF for TCP/UDP/RTP/IP protocols, and Ethernet is defined by IEEE. It is unclear how 3GPP can define a new ROHC profile and if/how IETF/IEEE adopts such new ROHC profile. In addition, ROHC profile identifiers may need registration with IANA [ref]. Such collaboration/liaison with other standard bodies may add uncertainties and could delay the work completion.

UDC-based solution

In LTE, Uplink Data Compression (UDC) [ref] is defined in PDCP specification and based on IETF RFC 1951 (DEFLATE Compressed Data Format Specification) [ref], and allows a generic compression of PDCP SDU data. It is unaware of the header structures of higher layer PDCP payload, i.e. Ethernet in this case. This way, also further headers of protocol layers above Ethernet including IP header, as well as padding and payload data, may be compressed. This study did not compare the performance in terms of complexity between UDC-based and other header-structure-aware schemes such as ROHC. An advantage of the UDC-based approach would also be that it is a pure RAN2 based solution without impacting to other specification groups or bodies.