3GPP TSG-RAN WG2 Meeting #105	R2-1902550

Athens, Greece, 24 Feb – 1 Mar 2019

Title:	[DRAFT] LS on MBSFN subframe pattern configuration for LTE CRS rate matching

Release:	Rel-15

Work Item:	NR

Source:	RAN2

To:	RAN1

Cc:

Contact Person:

Name:	Oscar Ohlsson

E-mail Address:	oscar.ohlsson@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

RAN2 observed that the current definition of the EUTRA-MBSFN-SubframeConfig in 38.331 does not support a deployment with subframe offset between overlaid NR- and LTE cells since it does not contain a field to inform the UE about that offset.

RAN2 discussed the need for the new slot offset parameter in the configuration of MBSFN subframe pattern which is used for LTE CRS rate matching for NR PDSCH.

The slot offset parameter is needed to indicate the MBSFN subframe pattern in deployments where the NR and LTE cells operate with a slot/subframe offset. Without this parameter the UE will be unaware which subframes are MBSFN subframes in the underlying LTE cell. This in turn implies that the UE will rate match its PDSCH around LTE CRSs which are actually not transmitted. The gNB would have to transmit the PDSCH as if there were LTE CRSs and the UE will decode the PDSCH accordingly. This results in a unnecessary throughput degradation of the NR PDSCH.

2. Actions

To RAN1 group:

ACTION: RAN2 respectfully asks RAN1 to inform RAN2 if RAN1 has concerns to introduce the above-mentioned slot offset parameter in the EUTRA-MBSFN-SubframeConfig.

3. Date of Next TSG RAN WG2 Meetings:

3GPP TSG RAN WG2 #105bis		2019.04.08 – 2019.04.12			Xi’an, China

3GPP TSG RAN WG2 #106		2019.05.13 – 2019.05.17			Reno, USA