3GPP TSG-RAN2 Meeting #105	R2-1902682

# Athens, Greece, 2019-02-25 – 2019-03-01

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 0822 | rev | 2 | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EUTRA-MBSFN-SubframeConfig |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Qualcomm Incorporated, Intel Corporation, NTT DOCOMO INC., Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  |  | Date: |  | 2019-03-01 |
| Category: | F |  |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  |  | As part of the “RateMatchPatternLTE-CRS” the network configures the LTE MBSFN pattern in the field “mbsfn-SubframeConfigList”. Among others, it includes the (LTE) fields “radioframeAllocationOffset”, “subframeAllocation1” and “subframeAllocation2”. Since the NR IE “EUTRA-MBSFN-SubframeConfig” is basically a mirror of the “MBSFN-SubframeConfig” in 36.331, 38.331 just refers to the corresponding field descriptions in 36.331. For example: radioFrameAllocationPeriod, radioFrameAllocationOffset Radio-frames that contain MBSFN subframes occur when equation SFN mod radioFrameAllocationPeriod = radioFrameAllocationOffset is satisfied. Value n1 for radioframeAllocationPeriod denotes value 1, n2 denotes value 2, and so on. When fourFrames is used for subframeAllocation, the equation defines the first radio frame referred to in the description below. Values n1 and n2 are not applicable when fourFrames is used. In the context of LTE CRS rate matching in NR it is however ambiguous whether “SFN” refers to the SFN of the NR cell which provides the configuration or to the SFN of the LTE cell which applies the actual MBSFN pattern. If the field radioframeAllocationOffset is interpreted in relation to the LTE cell’s timing, the NR-connected UE would have to acquire LTE PSS/SSS/MIB. We assume that this was not the intention as it would complicate the UE implementation significantly. We rather assume that an NR UE should interpret the radioFrameAllocationOffset as the offset between the NR SFN and the given MBSFN pattern. Furthermore, according to 36.331, the UE interprets the subframe allocation bitmaps differently depending on whether the (LTE) cell uses TDD or FDD. Like for the “SFN”, the UE should not be required to acquire MIB and SIB1 of the LTE cell to determine the duplex mode. It may simply assume that the LTE cell uses the same duplex mode as the NR cell for which the rate matching is configured. |  |  |  |  |  |  |  |
| Summary of change: |  |  |  | 1) EUTRA-MBSFN-SubframeConfigList-> radioFrameAllocationOffset: Clarify in the field description of radioframeAllocationPeriod in 38.331 that the SFN refers to the SFN of the NR serving cell. 2) EUTRA-MBSFN-SubframeConfigList-> subframeAllocation: Clarify in the field descriptions of subframeAllocation and subframeAllocation2 that the UE assumes the duplex mode (FDD / TDD) of the NR cell. Impact Analysis: Impacted 5G architecture options: Standalone, EN-DC Impacted functionality: PDSCH rate matching around LTE CRSs. If the UE is implemented according to the change but the NW is not, 1) the first change could imply that the UE uses the NR SFN whereas the NW expects it to acquire the LTE SFN. If the SFNs are not aligned, the UE assumes LTE MBSFN subframes to appear in wrong slots which could lead to decoding faliures and RLF. 2) the second change does not result in an interoperability issue. If the NW is implemented according to the change but the UE is not, 1) the first change could imply that the UE acquires the LTE SFN whereas the NW expects it to use the NR SFN. If the SFNs are not aligned, the UE assumes LTE MBSFN subframes to appear in wrong slots which could lead to decoding faliures and RLF. 2) the second change does not result in an interoperability issue. |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  | It is unclear whether the LTE-CRS pattern and the EUTRA-MBSFN-SubframeConfig provided through NR RRC is defined in relation to the NR- or LTE frame boundaries. |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |  |
| Other specs |  |  | X |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |

#### –	RateMatchPatternLTE-CRS

The IE RateMatchPatternLTE-CRS is used to configure a pattern to rate match around LTE CRS. See TS 38214 Clause 5.1.4.2.

RateMatchPatternLTE-CRS information element

-- ASN1START

-- TAG-RATEMATCHPATTERNLTE-CRS-START

RateMatchPatternLTE-CRS ::=         SEQUENCE {

carrierFreqDL                       INTEGER (0..16383),

carrierBandwidthDL                  ENUMERATED {n6, n15, n25, n50, n75, n100, spare2, spare1},

mbsfn-SubframeConfigList            EUTRA-MBSFN-SubframeConfigList                                              OPTIONAL,   -- Need M

nrofCRS-Ports                       ENUMERATED {n1, n2, n4},

v-Shift                             ENUMERATED {n0, n1, n2, n3, n4, n5}

}

-- TAG-RATEMATCHPATTERNLTE-CRS-STOP

-- ASN1STOP

RateMatchPatternLTE-CRS field descriptions

carrierBandwidthDL BW of the LTE carrier in number of PRBs (see TS 38.214 [19], clause 5.1.4.2).

carrierFreqDL Center of the LTE carrier (see TS 38.214 [19], clause 5.1.4.2).

mbsfn-SubframeConfigList LTE MBSFN subframe configuration (see TS 38.214 [19], clause 5.1.4.2) FFS_ASN1: Import the LTE MBSFN-SubframeConfigList.

nrofCRS-Ports Number of LTE CRS antenna port to rate-match around (see TS 38.214 [19], clause 5.1.4.2).

v-Shift Shifting value v-shift in LTE to rate match around LTE CRS (see TS 38.214 [19], clause 5.1.4.2).

=== Unmodified Sections omitted ===

#### –	EUTRA-MBSFN-SubframeConfigList

The IE EUTRA-MBSFN-SubframeConfigList is used to define an E-UTRA MBSFN subframe pattern (for the purpose of NR rate matching).

EUTRA-MBSFN-SubframeConfigList information element

-- ASN1START

-- TAG-EUTRA-MBSFN-SUBFRAMECONFIGLIST-START

EUTRA-MBSFN-SubframeConfigList ::= 	SEQUENCE (SIZE (1..maxMBSFN-Allocations)) OF EUTRA-MBSFN-SubframeConfig

EUTRA-MBSFN-SubframeConfig ::=      SEQUENCE {

radioframeAllocationPeriod          ENUMERATED {n1, n2, n4, n8, n16, n32},

radioframeAllocationOffset          INTEGER (0..7),

subframeAllocation1                 CHOICE {

oneFrame                            BIT STRING (SIZE(6)),

fourFrames                          BIT STRING (SIZE(24))

},

subframeAllocation2                 CHOICE {

oneFrame                            BIT STRING (SIZE(2)),

fourFrames                          BIT STRING (SIZE(8))

}                                                                                                                   OPTIONAL,   -- Need R

...

}

-- TAG-EUTRA-MBSFN-SUBFRAMECONFIGLIST-STOP

-- ASN1STOP

EUTRA-MBSFN-SubframeConfig field descriptions

radioframeAllocationOffset Field as defined in MBSFN-SubframeConfig in TS 36.331 [10]

radioframeAllocationPeriod Field as defined in MBSFN-SubframeConfig in TS 36.331 [10], where SFN refers to the SFN of the NR serving cell.

subframeAllocation1 Field as defined in MBSFN-SubframeConfig in TS 36.331 [10, where the UE assumes the duplex mode (FDD or TDD) of the NR cell for which the EUTRA-MBSFN-SubframeConfig is provided.

subframeAllocation2 Field as defined in MBSFN-SubframeConfig-v1430 in TS 36.331 [10], where the UE assumes the duplex mode (FDD or TDD) of the NR cell for which the EUTRA-MBSFN-SubframeConfig is provided.