3GPP TSG-RAN WG2 Meeting #105bis                                                 	R2-1903358

Xi’an, China, April 8th –12th 2019

Source:	CATT

Title:	Considerations on Carrying UE Capability ID via RRC Signalling

Agenda Item:	11.5.2

Document for:	Discussion and Decision

Introduction

During RAN#83 meeting, the new WI on optimisations on UE radio capability signalling [1] was approved. The objectives are below:

The work is expected to proceed as follows: - Specify the mechanism to optimize the UE Radio Capability signaling using UE capability identity (in coordination with SA2); [RAN2, RAN3]; - Signalling enhancements to be specified for both E-UTRA and NR; - After initial discussion of UE capability identify, whether simple delta signalling for the UE capability identity is specified will be concluded as part of the work; - RAN2 to specify the mechanism for the segmentation of UE Radio Capability signaling at RRC (for cases when the UE capability size exceeds the maximum PDCP SDU size). The work encompasses both 5GS and EPS. Work will be done in collaboration with SA WGs for the related system architectural aspects and CT WGs for the CN interfaces.

In this contribution, we discuss the mechanism to optimize the UE radio capability signaling using UE capability identity.

Discussion

During the SI, RAN2 has the agreements on UE capability ID signaling:

Agreements

1:	RAN2 to conclude NAS signalling of UE Capability ID is introduced in the normative phase

2:	Including UE capability ID in UECapabilityInformation message should be supported.

In SA2#131 meeting, [2] was agreed. UE indicates the UE capability ID via NAS signalling. Figure-1a/1b provides the proposed signalling procedures in [2].

Figure 1a: UE Capability ID transfer using NAS signalling (AMF has the UE capability ID)

Figure 1b: UE Capability ID transfer using NAS signalling (AMF does not have the UE capability ID)

From SA2 perspective, no matter AMF has the UE capability ID or not, gNB will acquire the UE capability ID from AMF. It implies that it is unnecessary to include UE capability ID in UECapabilityInformation message.

The requirements of including UE capability ID in UECapabilityInformation message are supporting of 'delta signalling' and signalling compression based on UE capability ID [3]. Both features are within the RAN2’s working scope. Therefore, whether including UE capability ID in UECapabilityInformation message should be decided depending on 'delta signalling' or signalling compression based on UE capability ID is introduced or not.

Proposal: whether including UE capability ID in UECapabilityInformation message or not should depend on whether 'delta signalling' or signalling compression based on UE capability ID is introduced or not.

Conclusion

In this contribution, we primarily analyzed the UE capability ID via RRC and gave the following proposal:

Proposal: whether including UE capability ID in UECapabilityInformation message or not should depend on whether 'delta signalling' or signalling compression based on UE capability ID is introduced or not.

Reference

[1] RP-190657, New WI: Optimisations on UE radio capability signalling	MediaTek Inc., CATT

[2] S2-1902914, On remaining open issues for FS_RACS, SA2#131

[3] R2-1903359, Consideration on Delta Signaling for ID-based Solution	CATT