3GPP TSG-RAN WG2 #105bis	Tdoc R2-1903836

Xi’An, China, 8 – 12 April 2019	Revision of R2-1900741

Agenda Item:	12.1.7

Source:	Ericsson

Title:	MPDCCH performance improvement for LTE-M

Document for:	Discussion, Decision

# 1	Introduction

The Rel-16 work item (WI) on “Additional MTC enhancements for LTE” [1] has the following objectives on stand-alone deployment for LTE-MTC:

Improved DL transmission efficiency and/or UE power consumption: - […] - Specify MPDCCH performance improvement by using CRS at least for connected mode [RAN1, RAN2, RAN4] - […]

RAN1 started the discussion on this objective in RAN1#94 and in subsequent meetings (RAN1#94bis RAN1#95 and RAN1#96) reached the following agreements relevant to RAN2 [2]:

RAN1#94: Agreement CRS for improving channel estimation on USS MPDCCH is supported Agreement The relation between the MPDCCH DMRS and CRS is defined for enabling improved channel estimation. FFS the details on specifying the relation (e.g., whether the ‘definition’ means predefined in specification or done by signalling, relation between the CRS ports and DMRS ports, power offset between DMRS-CRS). Agreement eNB enables/disables “CRS for improving channel estimation on MPDCCH” via dedicated or broadcasted RRC signaling. FFS the details on the signaling. Agreement CRS for improving channel estimation on Type0-MPDCCH CSS MPDCCH is supported. RAN1#94bis: Agreement CRS for improving channel estimation on MPDCCH in idle mode is supported. FFS the details. Agreement For localized MPDCCH, the following mappings are supported between CRS and DMRS: - Predefined mapping - Mapping based on CSI report FFS the details. Agreement For distributed MPDCCH, mapping between CRS and DMRS is defined in specifications. - FFS the details (e.g. number of PRBs over which mapping applies, fixed or configurable) Agreement Power offset between CRS and DMRS antenna ports of MPDCCH is signaled by eNB. FFS the signaling details (e.g. provisioning for default power offset value). RAN1#95: Agreement For distributed MPDCCH, predefined mapping between CRS ports and MPDCCH DMRS ports is supported. Agreement For localized/distributed MPDCCH, the predefined mapping between CRS ports and MPDCCH ports is based on precoder cycling across time and frequency domain. FFS the details. Agreement Reuse existing LTE Rel-8 codebooks for precoding of MPDCCH is the starting point. - Unless performance gains are idenfied further optimization on top of Rel-8 codebook will not be introduced Agreement For UE in Connected/Idle mode, the power offset between CRS and DMRS antenna ports of MPDCCH is indicated by SIB. Agreement - For UE in Idle mode, the configuration of CRS for MPDCCH performance improvement is indicated via SIB/SI. FFS the details of configuration. - For UE in Connected mode, the configuration of CRS for MPDCCH performance improvement is indicated via UE-specific RRC signalling. FFS the details of configuration. RAN1#96: Agreement Precoding cycling in time domain with a granularity of YCH is supported. FFS whether futher values are supported in which case the granularity is configurable. Agreement For precoding cycling, the precoder set consists of [2 or 4] existing precoders for 2Tx antennas and 4 existing precoders for 4Tx antennas. FFS: The precoder set is configured or not.

RAN2#103bis started the discussion on the feature from RAN2 point of view which continued also in RAN2#104 and reached the following agreements [3]:

RAN2#103bis agreements: - eNB enables CRS for improving channel estimation on MPDCCH via broadcasted RRC signalling. RAN2#104 agreements: - CRS for improving channel estimation on MPDCCH is configured via SIB2. This agreement can be revisited if RAN2 identifies that it would be more reasonable to configure it via SIB1-BR. It is FFS whether configuration would implicitly mean to enable the feature. - UEs report capability to indicate support of using CRS for improving MPDCCH channel estimation. - It is FFS if enabling/disabling is needed for the UE.

In this contribution we discuss the RAN1 progress and how to proceed from RAN2 point of view. This paper is a revision of R2-1900741, where Proposal 1 and the preceding paragraph are new.

# 2	Discussion

## 2.1	General observations

So far, the demodulation of MPDCCH is only based on DMRS. For Rel-16, agreements have been made in RAN1 and RAN2 to make it possible to improve the MPDCCH channel estimation by enabling the possibility to use CRS for MPDCCH demodulation. Depending on the type of the UE, i.e. BL or non-BL, and details of the mapping between DMRS and CRS, gains can be expected for both high and low SNR levels resulting in fewer number of repetitions needed for successful MPDCCH decoding. This results in less time needed for the UE to monitor MPDCCH and, consequently, improvements in power consumption, provided that the potentially increased load of processing the two types of reference signals is not too costly.

This feature aims both idle and connected mode so that UEs can use CRS for improved MPDCCH channel estimation both when they are in connected mode and for example during the RA procedure prior to entering the connected mode.

## 2.2	RAN1 progress

Most of the RAN1 agreements are related to L1 implementation aspects of this feature which are out of the scope of this paper. Few agreements have been made regarding signalling and configuration aspects.

According to the RAN1 and RAN2 agreements the following information is broadcasted in SIB2:

- Enable/disable bit (FFS if the presence of configuration information enables implicitly the feature)

- Power offset between CRS and DMRS antenna ports of MPDCCH in both connected and idle mode

- Configuration of CRS for MPDCCH for UEs in idle mode

In UE-specific signalling the following information should be included:

- Configuration of CRS for MPDCCH for UEs in connected mode

Even though precise information on the size of these objects is not defined, it is reasonable to assume that the total number of bits required is in the order of few tens of bits for SIB2 and few bits for UE-specific signalling.

The intention of the feature is to help UEs by improving the channel estimation making MPDCCH reception more efficient, thus if the power offset is signalled in SI the UE may use this regardless of separate indication on enabling or disabling the feature. Same principle can also apply to connected mode, where the UE may use the offset signalled for idle, even if there is no dedicated or separate configuration for connected mode. If dedicated configuration for a UE in connected is used, then the presence of such configuration would indicate to UE to override the idle mode configuration.

It seems there is no need to have a separate enable/disable bit for this feature. Separate enable/disable bit would make sense in the case RAN1 defines a default configuration, which could then be enabled with such bit.

Separate enable/disable bit is not specified for this feature. This can be revisited depending on further RAN1 agreements.

Many general aspects of the feature are still under discussion in RAN1 (i.e. there are many FFSs still being discussed), thus beyond above, there is not much RAN2 should address at this point of time.

Most signalling and configuration details are still being discussed in RAN1.

RAN2 should wait for further progress in RAN1 before agreeing on further details on the information to be transmitted in the extended messages.

## 2.3	Configuration aspects

Currently all the configurations related to MPDCCH are spread into different messages and IEs based on which function is associated with a particular search space as shown in the list below. For simplicity configurations related to SC-MTCH and SC-MCCH are omitted [4].

- SIB1

  - Downlink frequency hopping parameters for MPDCCH/PDSCH of paging, MPDCCH/PDSCH of RAR/Msg4 and unicast MPDCCH/PDSCH

- SIB2

  - Configuration for CSS for Paging

  - Configuration for CSS for RAR, Msg3 and Msg4 for EDT access

  - Configuration for CSS for RAR, Msg3 and Msg4 for non-EDT access

- RRCConnectionReconfiguration

  - Information relevant to mobility

  - Reconfiguration of UE-SS (RadioResourceConfigDedicated)

- RRCConnectionSetup

  - Configuration of UE-SS (RadioResourceConfigDedicated)

In connected mode it should be possible to configure MPDCCH, during first connection, through RRCConnectionSetup, whereas RRCConnectionReconfiguration may be used to update the configuration once connected. Both messages contain the IE RadioResourceConfigDedicated defining UE-SS configuration. This IE can be modified to include the new configurations.

RadioResourceConfigDedicated IE is extended to provide MPDCCH configuration for connected mode.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Most signalling and configuration details are still being discussed in RAN1.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Separate enable/disable bit is not specified for this feature. This can be revisited depending on further RAN1 agreements.

Proposal 2	RAN2 should wait for further progress in RAN1 before agreeing on further details on the information to be transmitted in the extended messages.

Proposal 3	RadioResourceConfigDedicated IE is extended to provide MPDCCH configuration for connected mode.

# References

RP-190770, Additional MTC enhancements for LTE, Shenzhen, China, March 2019

R1-1903661, RAN1 agreements for Rel-16 Additional MTC Enhancements for LTE, Athens, Greece, March 2019

R2-1902405, RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC, Athens, Greece, March 2019

TS 36.331, v15.4.0