3GPP TSG-RAN WG2 Meeting #105bis	R2-1904167

Xi’an, China, 8th-12th Apr 2019

Agenda item:	11.6.4.1.2

Source: 	Huawei, HiSilicon

Title: 	RACH-less handover for NTN

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss RACH-less handover for NTN.

Discussion

In the RAN2 #104 meeting, it was agreed that the details of satellite ephemeris can be captured in the TR. The satellite ephemeris could provide the location of satellites in any time. According to [1], the targeted usage-scenarios with satellite access are pedestrian, stationary in building, on board vehicle, high speed train or airplanes. In those scenarios, most of the transmission between satellite and UEs will be LoS transmission. Then, if UE location is available and satellite ephemeris is told to UE, UE could estimate the timing advance value to its serving cell based on its location and the serving satellite location.

Observation 1: UE could estimate the timing advance value to its serving cell based on its location and the satellite ephemeris.

The following table shows the delay constraints of different NTN scenarios by referring to [2].

| NTN scenarios | A | B | C1 | C2 | D1 | D2 |
| --- | --- | --- | --- | --- | --- | --- |
|  | GEO transparent payload | GEO regenerative payload | LEO transparent payload |  | LEO regenerative payload |  |
| Satellite altitude | 35 786 km |  | 600 km |  |  |  |
| Relative speed of Satellite wrt earth | negligible |  | 7.56 km per second |  |  |  |
| Min elevation for both feeder and service links | 10° for service link and 5° for feeder |  |  |  |  |  |
| Typical Min / Max NTN beam foot print diameter (note 1) | 100 km / 1000 km |  | 50 km / 500 km |  |  |  |
| Maximum Round Trip Delay on the radio interface between the gNB and the UE | 541.75 ms (Worst case) | 270.57 ms | 28.41 ms |  | 12.88 ms |  |
| Minimum Round Trip Delay on the radio interface between the gNB and the UE | 477.14 ms | 238.57 ms | 8 ms |  | 4 ms |  |
| Maximum Delay variation as seen by the UE (note 2) | Negligible |  | Up to +/- 40 µs/sec (Worst case) |  | Up to +/- 20 µs/sec |  |

According to this table, the RTT of NTN is much higher than terrestrial network, especially for GEO scenario. In the legacy handover mechanism, after receiving RRC reconfiguration from source gNB, UE will perform contention-free RACH in the target gNB to obtain TA value of target cell. If the legacy handover mechanism is adopted in NTN, the handover interruption latency will very long due to the long RTT. RACH-less handover is discussed in LTE with the condition that UE has same TA value in source gNB and target gNB. According to observation 1, RACH-less handover mechanism could be used in NTN due to the fact that UE could estimate the TA value of target gNB after obtaining its location and satellite ephemeris. In this way, handover interruption latency could be reduced by at least one RTT.

Observation 2: RACH-less handover mechanism could work in NTN when UE could estimate its TA value of target gNB.

Figure 1 Legacy handover mechanism

Figure 2 RACH-less handover mechanism

Hence, RAN2 should study supporting RACH-less handover mechanism in NTN from the perspective of reducing handover interruption latency.

Proposal 1: RAN2 should consider supporting RACH-less handover mechanism in NTN.

In order to ensure RACH-less handover mechanism working well in NTN, the satellite ephemeris needs to be informed to UEs. Furthermore, network should know whether UE owns the capacity to estimate its TA value to decide to configure RACH resource or UL-Grant in RRC reconfiguration. Therefore, it is proposed that satellite ephemeris should be informed to UEs and UE reports its capacity of estimating TA to network.

Proposal 2: in order to ensure RACH-less handover mechanism working well in NTN, satellite ephemeris should be informed to UEs and UE should report its capacity of estimating TA to network.

Conclusion

In this contribution, we discussed RACH-less handover for NTN, and we get the following proposals:

Observation 1: UE could estimate the timing advance value to its serving cell based on its location and the satellite ephemeris.

Observation 2: RACH-less handover mechanism could work in NTN when UE could estimate its TA value of target gNB.

Proposal 1: RAN2 should consider supporting RACH-less handover mechanism in NTN.

Proposal 2: in order to ensure RACH-less handover mechanism working well in NTN, satellite ephemeris should be informed to UEs and UE should report its capacity of estimating TA to network.

References

[1] RP-190710, Study on solutions for NR to support non-terrestrial networks (NTN)

[2] 3GPP TR 38.811: “Study on New Radio (NR) to support non terrestrial networks”