3GPP TSG-RAN WG2 Meeting #105bis	                                            R2-1904405

Xi’an, China, 8th – 12th April 2019	                          Resubmission of R2-1901341

Agenda Item:	11.2.2.1

Souce:	Huawei, HiSilicon

Title:	Consideration on SI for NR-U

Document for:	Discussion and decision

# 1 Introduction

In RAN2#103 bis [1], it was agreed that:

Agreements: - RAN2 should focus on scheduling of SIB at this stage and the detailed design of MIB and SIB1 are pending in RAN1. - More opportunities for SI transmission in time domain may be needed to relieve LBT impact.

In this contribution, we further discuss the SI issues for NR-U and provide our proposals.

# 2 Discussion

### 2.1 On-demand SI

In Rel-15 NR, msg1 and msg3-based random access are supported for SI request. We believe this is also applicable to NR-U. Note 2-step RACH based on demand SI request has been agreed in previous meeting.

Proposal 1: Both msg1 and msg3-based random access for SI request should be supported for NR-U as baseline.

As for dedicated RRC signalling based on demand SI request mentioned in [2], there are lots of discussions during Rel-15 NR. Finally RAN2 did not agree to introduce dedicated RRC signalling based on demand SI request and it should not be discussed in NR-U until it has been introduced in NR licensed cell.

Proposal 2: Dedicated RRC signalling based SI request in RRC_CONNECTED state should be discussed in licensed cell firstly.

In [3], it is proposed to introduce an indication on the broadcast duration of requested SI. There have also been lots discussions on this during Rel-15 NR. However it has not been agreed. The final result was that the network can broadcast the requested SI during a modification period after the SI request and indication is not needed. We believe it should not be discussed in NR-U until it has been introduced for NR licensed cell.

Proposal 3: RAN2 not to discuss the indication on the broadcast duration of the requested SI until it has been supported for NR-licensed cell.

### 2.2 SI occasions in time domain

RAN2 agreed [1] that “more opportunities for SI transmission in time domain may be needed to relieve LBT impact”. Several solutions can be available to provide with more opportunities for SI transmission: the first is reducing SI period and another is extending SI window length.

Proposal 4: reducing SI period/extending SI window length should be considered.

In Rel-15 NR, to identify which SI message is to be transmitted, each SI message is associated with a SI window and the SI windows of different SI messages are not overlapped in time domain as showed in figure 1.

Figure 1: SI window for different SI message

However, reducing SI period/extending SI window length may result that the network cannot configure the SI windows of different SI message to be not overlapped in time domain.

Observation 1: reducing SI period/extending SI window length may result that the network cannot configure the SI windows of different SI messages to be non-overlapping in time domain.

This issues has been discussed during previous RAN2 meeting and no agreement is achieved. Two options are available to address the issue caused by introducing of reducing SI period/extending SI window:

- Option 1: multiple SI messages are associated with a SI window and different SI window does not overlap in time domain; - Option 2: a mechanism is introduced to distinguish which SI message is to be transmitted, e.g. using different SI-RNTI for different SI messages or an explicit indication is introduced in DCI format.

Option 1 is simple. The network can transmit multiple SI messages associated with the same SI window into a MAC PDU or different MAC PDUs. Option 2 is difficult and may impact the PHY. We prefer to option 1.

Proposal 5: Multiple SI messages can be associated with a SI window and different SI windows do not overlap in time domain.

# 3 Conclusion

Based on the discussions above, we have the following observations and proposals:

Observation 1: reducing SI period/extending SI window length may result that the network cannot configure the SI windows of different SI messages to be non-overlapping in time domain.

Proposal 1: Both msg1 and msg3-based random access for SI request should be supported for NR-U as baseline.

Proposal 2: Dedicated RRC signalling in RRC_CONNECTED state should be discussed in licensed cell firstly.

Proposal 3: RAN2 not to discuss the duration of the broadcast of requested SI until it has been supported for NR-licensed cell.

Proposal 4: reducing SI period/extending SI window length should be considered.

Proposal 5: Multiple SI messages can be associated with a SI window and different SI windows do not overlap in time domain.

# 4 References

- RAN2#103bis chairman note.

- R2-1815047, Dedicated RRC signaling for request and delivery of on-demand SI in NR-U, Ericsson.

- R2-1815049, Indication of duration of broadcast of requested on-demand SI in NR-U, Ericsson.