3GPP TSG-RAN WG2 #105bis	Tdoc R2-1905135

Xi’an, China, 8th April – 12th April 2019

Agenda Item:	11.14

Source:	Ericsson

Title:	Overview on RAN2 aspects for CLI

Document for:	Information

# Background

During the Rel-14 NR SI and subsequent Rel-15 WI, flexible duplexing in general and dynamic TDD was in particular was studied, and Rel-15 NR currently supports flexible frame structure which enables both semi-static and dynamic TDD operation. In the context of both dynamic TDD as well as semi-static TDD with misaligned TDD configurations between neighbouring cells, cross-link interference (CLI) can occur. How to mitigate or avoid such CLI was studied in previous releases and various schemes was enumerated in [2]. Certain CLI mitigation schemes require UE-side CLI measurements as input and the introduction of such measurements was also discussed in Rel-15. However, the work on flexible duplex and CLI measurements was downprioritized by the RAN plenary in 2017 and scoped out of the NR WI.

At RAN#80, a WI on CLI and RIM was approved. Furthermore, at, RAN#82, the WID [1] was updated and contain the following objectives:

The detailed objectives for cross-link interference mitigation to support flexible resource adaptation for unpaired NR cells are:

- Specify cross-link interference measurements and reporting at a UE (e.g., CLI-RSSI and/or CLI-RSRP) [RAN1, RAN2, RAN4]

- Specify network coordination mechanism(s) including at least exchange of intended DL/UL configuration [RAN1, RAN3]

- Perform coexistence study to identify conditions of coexistence among different operators in adjacent channels [RAN4]

  - Target no or very minimal impact on RF requirement

Note: Measurement and coordination mechanisms should be applicable to IAB nodes.

In RAN1 Ad-Hoc meeting 1901 in Taipei, it was agreed that both RSSI and SRS-RSRP for CLI measurement is supported, and the following agreements that are related to RAN2 were reached:

Agreement The following CLI measurements are supported: - SRS-RSRP: - Linear average of the power contributions of the SRS to be measured over the configured resource elements within the considered measurement frequency bandwidth in the time resources in the configured measurement occasions - RSSI: - The linear average of the total received power observed only in certain OFDM symbols of measurement time resource(s), in the measurement bandwidth, over the configured resource elements for measurement by the UE Agreement For SRS-RSRP measurement report, L3 measurement reporting is applied Agreement For CLI-RSSI, L3 measurement reporting is applied - FFS: Whether SRS-RSRP measurement report and CLI-RSSI measurement report can be configured together for a UE - Whether UE should be required to perform SRS-RSRP measurement and CLI-RSSI measurement simultaneously Agreement For L3 measurement reporting of RSSI/SRS-RSRP, the specification detail on the reporting and triggering mechanism is discussed in RAN2 - Send an LS to RAN2 (LGE, Hyunsoo) Agreement - From a specification perspective, the measurement/report configuration of CLI-RSSI and SRS-RSRP are independent. - FFS: UE capability discussion

In this paper, we give an overview on the UE-side cross-link interference measurement and reporting and touch upon some RAN2-related issues.

# Discussion

Cross-link interference (CLI) occurs when the transmission directions of neighbouring cells in the network are not aligned, either due to the neighbouring cells applying different semi-static TDD patterns or due to the use of dynamic TDD in at least one of the neighbouring cells. For example, Cell A may be scheduling UL while Cell B is scheduling DL, which causes the UL transmissions of UE(s) in Cell A to interfere with the DL reception of UE(s) in Cell B (so called UE-to-UE interference). In addition, the UL reception of TRP A is interfered by the DL transmission of TRP B (so called TRP-to-TRP interference). This is illustrated in Figure 1 below. Such cross-link interference (UE-to-UE interference or TRP-to-TRP interference) can potentially be significantly larger than that the conventional inter-cell (TRP-to-UE or UE-to-TRP) interference, which risks deteriorating the network performance.

Figure 1: Illustration of cross-link interference

The higher interference level can be due to the following reasons:

- A larger transmission power is applied by the interfering node than by the intended transmitting node

- A more directive antenna pattern, which is also directed at the receiving node, is used by the interfering node than what would have been used by the intended transmitting node

- The path loss between the receiving node and the interfering node is lower than the path loss between the receiving node and the intended transmitting node

Here we focus on UE-to-UE interference and out of the 3 listed reasons for the increased interference level, the most significant is the difference in pathloss between the cross-link and the downlink. This can for instance happen when the victim UE is on the cell edge and the aggressor UE, i.e. the UE causing the interference, is in close proximity to the victim UE, which is commonly referred to as a near-far effect. The link between the victim and aggressor UE may in some cases be much stronger than the link between victim UE and its serving gNB, which could potentially have a large impact on the performance.

Due to near-far effect, the UE-to-UE interference can in some instances be very large

There are several ways of mitigating the effect of the UE-to-UE Cross Link Interference, most importantly through interference avoidance and improved link adaption, which are actions that may be done through implementation. But in order to apply procedures to mitigate the UE-to-UE CLI there is a need to perform measurements to detect the occurrence as it is likely that this interference would only be subject to a subset of UEs, i.e UEs on the cell edge. There have thus been a need to introduce measurement schemes to detect the UE-to-UE CLI before the network can employ measures to mitigate the effect.

Measurement schemes are needed to detect the UE-to-UE Cross-Link Interference.

During the RAN1 WI [1] methods for enabling L1 measurement of Cross-Link Interference have been introduced.

## Discussion on layer 3 UE-to-UE CLI measurements

RRC already provides a framework for measuring and reporting measurements in RRC_CONNECTED mode. The measurement reporting scheme is seen in Figure 2.

Figure 2. Measurement reporting

According to TS 38.331 the measurement and reporting configuration includes a general set of 5 parameters:

1.	Measurement objects: A list of objects on which the UE shall perform the measurements.

2.	Reporting configurations: A list of reporting configurations where there can be one or multiple reporting configurations per measurement object. Each reporting configuration consists of the following:

3.	Measurement identities: A list of measurement identities where each measurement identity links one measurement object with one reporting configuration. By configuring multiple measurement identities, it is possible to link more than one measurement object to the same reporting configuration, as well as to link more than one reporting configuration to the same measurement object. The measurement identity is also included in the measurement report that triggered the reporting, serving as a reference to the network.

4.	Quantity configurations: The quantity configuration defines the measurement filtering configuration used for all event evaluation and related reporting, and for periodical reporting of that measurement. For NR measurements, the network may configure up to 2 quantity configurations with a reference in the NR measurement object to the configuration that is to be used. In each configuration, different filter coefficients can be configured for different measurement quantities, for different RS types, and for measurements per cell and per beam.

5.	Measurement gaps: Periods that the UE may use to perform measurements.

As the measurement framework is typically defined by measuring on serving and neighbouring resources, such as SS/PBCH blocks(s) and/or CSI-RS resources, there is a need to consider how this framework can be adapted to use the L1 measurements for CLI defined by RAN1[1].

There is a need to consider what type of resources that shall be measured for UE-to-UE CLI and how to incorporate them in the RRC reporting framework.

For the measurement reporting configuration there is a need to consider the reporting criterion and the quantity configuration. The reporting criterion may be based on a single event, i.e. signal strength or quality of a neighbouring resource is larger than a threshold or be periodical where the UE would report the measurement of the signal strength or quality regardless of whether the measurement quantity is large or not. The measurement quantity that is reported and used in event evaluation also needs to be considered for the UE-to-UE CLI.

There is a need to consider what type measurement quantity and the reporting criterion that is needed for UE-to-UE CLI.

# Conclusion

In section 2 we made the following observations:

Observation 1	Due to near-far effect, the UE-to-UE interference can in some instances be very large

Observation 2	Measurement schemes are needed to detect the UE-to-UE Cross-Link Interference.

Observation 3	There is a need to consider what type of resources that shall be measured for UE-to-UE CLI and how to incorporate them in the RRC reporting framework.

Observation 4	There is a need to consider what type measurement quantity and the reporting criterion that is needed for UE-to-UE CLI.

# References

RP-182864, “Revised WID on Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR”, LG Electronics

TR 38.802, “Study on new radio access technology Physical layer aspects”