3GPP TSG RAN WG2#105bis	R2-1905361

Xi'an, China, 8 – 12, April, 2019

3GPP TSG-RAN WG1 Meeting #96bis 	R1-1905570

# Xi´an, China, 8th – 12th April 2019

Title:	LS on PUR Working Assumptions for NB-IoT and LTE-M

Release:	Release 16

Work Items:	NB_IOTenh3-Core, LTE_eMTC5-Core

Source:	RAN WG1

To:	RAN WG2

Cc:

Contact Person:

#### Name:	Gus Vos

E-mail Address:	gvos@sierrawireless.com

Attachments:	None

1. Overall Description:

In RAN1#96bis for the preconfigured uplink resource (PUR) feature, RAN1 has made the following working assumptions for both NB-IOT and LTE-M:

NB-IoT and LTE-M Working Assumptions

- Working Assumption#1

- In idle mode, updating PUR configurations and/or PUR parameters via L1 signalling after a PUR transmission is supported

FFS: Which PUR configurations and PUR parameters will be signaled via L1

FFS: Definition of PUR configurations and PUR parameters

- The working assumption will be automatically confirmed if for some cases L2/L3 signaling is not needed. If RAN2 decides that L2/L3 signaling is needed for all cases, the working assumption will be reverted.

- Working Assumption#2

- For dedicated PUR

During the PUR search space monitoring, the UE monitors for DCI scrambled with a RNTI assuming that the RNTI is not shared with any other UE

  - Note: It is up to RAN2 to decide how the RNTI is signaled to UE or derived

FFS if the UE monitors any additional RNTI which may be shared with other UEs.

Note: The same RNTI may be used over non-overlapping time and/or frequency resources

- If it is concluded that working assumption #2 feasible, the working assumption #2 will be automatically confirmed.

2. Actions:

To RAN2 group:

ACTION:

#1 RAN1 respectfully asks RAN2 to take the above working assumptions into account in their work.

#2 RAN1 kindly asks RAN2 to determine and inform RAN1 of the feasibility from a RAN2 perspective of the first bullet in Working Assumption #2 for both NB-IOT and LTE-M.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #97 	13 - 17 May 2019    Reno, US

TSG-RAN WG1 Meeting #98 	26 - 30 Aug 2019    Prague, Czechia