3GPP TSG-RAN WG2 Meeting #105bis	R2-1905409

# Xi’an, China, 08 Apr - 12 Apr 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | Draft | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on periodicityAndOffset in ZP-CSI-RS-Resource |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For periodicityAndOffset field in IE ZP-CSI-RS-Resource, it is optional with Cond PeriodicOrSemiPersistent. But the corresponding description of PeriodicOrSemiPersistent is missing, it may lead to some confusion about when this field is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the description of Cond PeriodicOrSemiPersistent in IE ZP-CSI-RS-Resource. - Clarify that network always configures this field for periodic and semi-persistent ZP-CSI-RS resource (as indicated in PDSCH-Config). Impact analysis Impacted 5G architecture options: Standalone, EN-DC Impacted functionality: ZP-CSI-RS Inter-operability: If the UE is implemented according to the CR and the network is not, network may not configure periodicityAndOffset for periodic or semi-persistent ZP-CSI-RS-Resource (as indicated in PDSCH-Config), so UE cannot obtain periodicity and slot offset for periodic/semi-persistent ZP-CSI-RS, it may lead to a reconfiguration failure. If the network is implemented according to the CR and the UE is not, there is no interoperability issue, since the network will provide this periodicityAndOffset configuration when needed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network may not configure periodicityAndOffset for periodic and semi-persistent ZP-CSI-RS-Resource (as indicated in PDSCH-Config), so UE cannot obtain periodicity and slot offset for periodic/semi-persistent ZP-CSI-RS, it may lead to a reconfiguration failure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The baseline specification is draft TS 38.331-f50 |  |  |  |  |  |  |  |  |

CHANGE START

6.3.2	Radio resource control information elements

<unnecessary parts omitted>

#### –	ZP-CSI-RS-Resource

The IE ZP-CSI-RS-Resource is used to configure a Zero-Power (ZP) CSI-RS resource (see TS 38.214 [19], clause 5.1.4.2).

ZP-CSI-RS-Resource information element

-- ASN1START

-- TAG-ZP-CSI-RS-RESOURCE-START

ZP-CSI-RS-Resource ::=              SEQUENCE {

zp-CSI-RS-ResourceId                ZP-CSI-RS-ResourceId,

resourceMapping                     CSI-RS-ResourceMapping,

periodicityAndOffset                CSI-ResourcePeriodicityAndOffset                        OPTIONAL, --Cond PeriodicOrSemiPersistent

...

}

ZP-CSI-RS-ResourceId ::=            INTEGER (0..maxNrofZP-CSI-RS-Resources-1)

-- TAG-ZP-CSI-RS-RESOURCE-STOP

-- ASN1STOP

ZP-CSI-RS-Resource field descriptions

periodicityAndOffset Periodicity and slot offset for periodic/semi-persistent ZP-CSI-RS (see TS 38.214 [19], clause 5.1.4.2). Network always configures this field for periodic and semi-persistent ZP-CSI-RS resource (as indicated in PDSCH-Config).

resourceMapping OFDM symbol and subcarrier occupancy of the ZP-CSI-RS resource within a slot.

zp-CSI-RS-ResourceId ZP CSI-RS resource configuration ID (see TS 38.214 [19], clause 5.1.4.2).

| Conditional Presence | Explanation |
| --- | --- |
| PeriodicOrSemiPersistent | The field is optionally present, Need M, for periodic and semi-persistent ZP-CSI-RS-Resources (as indicated in PDSCH-Config). The field is absent otherwise. |

<unnecessary parts omitted>

CHANGE END