3GPP TSG-RAN WG2 #105bis	R2-1903842

# Xi’an, China, 8h – 12th April 2019						Revision of R2-1902197

Agenda Item:	12.1.13

Source:	Ericsson

Title:	RRC release after EDT for User Plane CIoT EPS optimization

Document for:	Discussion, Decision

# 1	Introduction

In RAN2#105, it was discussed whether AS RAI should be introduced for User Plane CIoT EPS optimization Please see the meeting minutes below for the related discussion:

R2-1902197	RRC release after EDT for User Plane CIoT EPS optimization	Ericsson	discussion	LTE_eMTC4-Core, NB_IOTenh2-Core

- LG supports the proposal in principle, but wonders if this can also be done if NAS RAI information can be provided to the eNB from MME.

- Intel wonders whether segmentation should also be supported considering that it has already been discussed whether more data can be transmitted when EDT is initiated. Ericsson explains that this is not about UL, it is about DL.

- QC explains that when rai-activation is configured, BSR equal to zero means that UE does not have any more data to send or expect ant data in the DL.

- Ericsson agrees and thinks that it would be also useful to have such indication in case the UE expects data in the DL.

- QC thinks that this is a new feature which should not be specified in Rel-15. Ericsson wonders if it is possible to introduce the feature in Rel-16.

- Sierra Wireless supports the proposal in principle and explains that they have a proposal with a similar purpose.

- Gemalto thinks this may be introduced in Rel-16 and RAN2 may have an email discussion to discuss the details.

Noted. The discussion may continue within the context of Rel-16.

In this contribution we discuss further details for RRC release after EDT for User Plane CIoT EPS optimization.

# 2	Discussion

## 2.1 EDT for CP Optimization

Early data transmission (EDT) procedure for Control Plane CIoT EPS optimization is illustrated in Figure 1 below (subclause 7.3b.1 in [1]).

Figure 1: EDT for Control Plane CIoT EPS Optimizations

The eNB may indicate to MME that connection is triggered for EDT with the S1-AP Initial UE message. If downlink data is available, S-GW sends the downlink data to MME. When data is received, MME forwards the data to the eNB via DL NAS Transport procedure. It may trigger Connection Establishment Indication procedure and/or indicate whether further data is expected.

In mobile originated data transport using Control Plane CIoT EPS Optimisation, UE establishes a RRC connection or sends the RRCEarlyDataRequest (EDT) message. The UE may also indicate Release Assistance Information in the NAS PDU (NAS RAI) whether no further uplink or downlink data transmissions are expected, or only a single downlink data transmission (e.g. acknowledgement or response to uplink data) subsequent to this uplink data transmission is expected.

If no downlink data is expected based on the Release Assistance Information from the UE, this means all application layer data exchanges have been completed with the UL data transfer, and if MME is not aware of any pending MT traffic and S1-U bearers are not established, MME may signal the S1-AP UE Context Release Command to the eNB.

If the Release Assistance Information was received with uplink data and it indicated that downlink data was expected, it means downlink packet following the transmission of Release Assistance Information is the last packet of the application layer data exchange. For this case, unless MME is aware of additional pending MT traffic and S1-U bearers are established, MME sends a S1 UE Context Release Command immediately after the S1-AP message including the downlink data encapsulated in NAS PDU as an indication that eNB shall release the RRC connection promptly after successfully sending data to the UE.

If "EDT Session" indication was received, it is also possible that MME includes the ”end indication” for no further data in the S1-AP message including the downlink data encapsulated in NAS PDU.

MME has access to information, e.g., MT data/signalling awareness and NAS RAI, to make the decision to send the S1 UE Context Release Command to the eNB.

## 2.2 EDT for UP Optimization

Early data transmission (EDT) procedure for User Plane CIoT EPS optimization is illustrated in Figure 2 below (subclause 7.3b.2 in [1]).

Figure 2: EDT for User Plane CIoT EPS Optimizations

The UE sends the RRCConnectionResumeRequest message to the eNB, including its Resume ID, establishment cause, and an authentication token. The UE resumes all SRBs and DRBs, derives new security keys using the NextHopChainingCount provided in the RRCConnectionRelease message of the previous connection and re-establishes the AS security. The user data is ciphered and transmitted on DTCH multiplexed with the RRCConnectionResumeRequest message on CCCH. However, it is not possible for the UE to provide an indication similar to the Release Assistance Information in NAS PDU whether no further uplink or downlink data transmissions are expected, or only a single downlink data transmission (e.g. acknowledgement or response to uplink data) subsequent to this uplink data transmission is expected.

There is no RAI in AS layer to indicate whether no further UL/DL data transmissions or a single DL data transmission are expected.

The MME confirms the UE context resume to the eNB and it is aware of any MT-signalling/data that may have been received previously. But MME may not be aware of the service and does not know if there will be DL data triggered at this point.

If no further data is expected from the S-GW, eNB can initiate the suspension of the S1 connection and the deactivation of the S1-U bearers. Neither the eNB nor the MME has any knowledge if there will be data in the DL to transmit or if DL data will be triggered with the transmission in the UL. Therefore, there is no reason for MME to provide the “pending” flag in S1 suspend response in this case. Note that eNB may also decide to release the UE completely and S1 UE context Release Request may be sent.

For User Plane CIoT EPS optimization, it is not clear how eNB can make the decision after Msg3 (i.e. RRC Connection Resume Request) if the RRC connection can be released/suspended with Msg4 (i.e. RRC Connection Release), or RRC connection shall be established with Msg4 RRC Connection Resume. AS RAI is not reflected in EDT for UP CIoT EPS optimisation. If UE uses the EDT as an implicit AS RAI indication in itself (e.g. for a single UL packet), the flow provided in [1], which shows DL data, is not correct. Furthermore, it is not clear how eNB can distinguish dual UL+DL packet from single packet and from transmissions that will generate multiple packets.

RAI has been introduced in RAN2 implicitly, however the captured mechanism is a pure “UL buffer empty” indication which does not originate with application layer information and it does not support the RAI values defined for NAS. To support eNB to make the RRC connection handling decision as early as possible, AS RAI can be introduced from UE to RAN with values similar to the ones defined for NAS.

Introduce AS RAI from UE to RAN.

The UE may include this IE to inform the network whether

a)   no subsequent DL and UL data transmission is expected; or

b)   a single subsequent DL transmission is expected; or

c)   multiple subsequent data transmissions are expected in the DL and/or UL.

Regarding the 3rd bullet; note that there is a difference between knowing that there will be multiple transmissions, i.e.  (i.e. the eNB should move the UE to connected mode, and not knowing anything. In NAS RAI which has been specified in TS 24.301, no indication would be sent if multiple packets are expected.

AS RAI informs the network whether a) no subsequent DL and UL data transmission is expected; or, b) a single subsequent DL transmission is expected; or c) multiple subsequent data transmissions are expected in the DL and/or UL.

RAN2 to discuss whether AS RAI is provided in MAC or RRC.

# 3	Conclusion

In this contribution RRC release after EDT is discussed for User Plane CIoT EPS optimization. The following observations were made:

Observation 1	MME has access to information, e.g., MT data/signalling awareness and NAS RAI, to make the decision to send the S1 UE Context Release Command to the eNB.

Observation 2	There is no RAI in AS layer to indicate whether no further UL/DL data transmissions or a single DL data transmission are expected.

Based on the discussion in the sections above we propose the following:

Proposal 1	Introduce AS RAI from UE to RAN.

Proposal 2	AS RAI informs the network whether a) no subsequent DL and UL data transmission is expected; or, b) a single subsequent DL transmission is expected; or c) multiple subsequent data transmissions are expected in the DL and/or UL.

Proposal 3	RAN2 to discuss whether AS RAI is provided in MAC or RRC.

# 4	References

TS 36.300, Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2, 15.4.0, 2019 Jan 15.