3GPP TSG-RAN WG2#105bis	R2-1904530

Xi’An, China, 8th April – 12th April 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | num | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Measurements for SCell failure report (38.331) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | n.a. (draft CR to be merged into rapporteur's CR when agreed) |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  |  | Date: |  | 2019-03-28 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When a RLF is detected on a logical channel restricted to SCell(s) the UE should report the SCell-RLF to the network by including also the latest available measurements that then can configure the UE to take the necessary actions (e.g., reconfiguration, addition, or removal of the SCell). |  |  |  |  |  |  |  |  |
| Summary of change: |  | The UE includes in the FailureInformation message the latest available measurements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRC specification will not be clear and potential erroneous behaviour on the UE and network side when an RLF happen on the SCell. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2 Message definitions |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### –	FailureInformation

The FailureInformation message is used to inform the network about a failure detected by the UE.

Signalling radio bearer: SRB1 or SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to network

FailureInformation message

FailureInformation ::=         SEQUENCE {

criticalExtensions                  CHOICE {

failureInformation             FailureInformation-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

FailureInformation-IEs ::=.        SEQUENCE {

failureInfoRLC-Bearer						FailureInfoRLC-Bearer   OPTIONAL,

lateNonCriticalExtension        OCTET STRING                        OPTIONAL,

nonCriticalExtension            FailureInformation-v16xy-IEs        OPTIONAL

}

FailureInformation-v16xy-IEs ::=    SEQUENCE {

measResultsNR                       MeasResultNR              OPTIONAL,

measResultsEUTRA                    MeasResultEUTRA           OPTIONAL,

nonCriticalExtension				SEQUENCE {}               OPTIONAL

}

FailureInfoRLC-Bearer ::=					SEQUENCE {

cellGroupId							CellGroupId,

logicalChannelIdentity				LogicalChannelIdentity,

failureType							ENUMERATED {duplication, spare3, spare2, spare1}

}

-- TAG-FAILUREINFORMATION-STOP

-- ASN1STOP

END OF CHANGES