3GPP TSG-RAN2 Meeting #106	R2-1905577

# Reno, US, 13 May - 17 May 2019

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.323 | CR | 0272 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on HFN maintainance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eV2X-Core |  |  |  |  |  | Date: |  |  | 2018-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In section 5.1.3/4, it is required to maintain HFN for R15 V2X, but it is not feasible since for broadcast traffic (which is the only traffic type for LTE-V2X), where RX UE may meet TX UE after TX UE starts transmission for an unknown time period, it is not possible for RX UE to derive the HFN value. Furthermore, HFN value is actually not used at all for V2X data transmission / reception, i.e., no need for maintainace. Impact Analysis Impacted functionality: HFN maintainace for SL transmission/reception Inter-operability: 1. If the network is implemented according to the CR and the UE is not, there is no inter-operability problem, since HFN maintainance function only relates to sidelink. 2. If the UE is implemented according to the CR and the network is not, there is no inter-operability problem, since HFN maintainance function only relates to sidelink. 3. If one UE is implemented according to the CR and the other UE is not, there is no inter-operability problem, since HFN is not used for SL transmission / reception at all. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove HFN maintainance from section 5.1.3/5.1.4.. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE maintain HFN in an infeasible and meaningless manner. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.3, 5.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

CHANGE START

5.1.3	SL Data Transmission Procedures

For Sidelink transmission of the SLRB for which SL-V2X-TxProfile is not configured or configured as rel14, see TS 36.331 [3], the UE shall follow the procedures in subclause 5.1.1 with following modifications:

-	the requirement for maintaining Next_PDCP_TX_SN is not applicable;

-	determine a PDCP SN ensuring that a PDCP SN value is not reused with the same key;

-	perform ciphering (if configured) as specified in subclause 5.6.1 and 5.6.2;

-	perform the header compression (if configured) if SDU Type is set to 000, i.e. IP SDUs.

For sidelink transmission of the SLRBs for which the indicated SL-V2X-TxProfile is rel15, see TS 36.331 [3], the UE shall follow the procedures in subclause 5.1.1 with following modifications compared to above Sidelink transmission procedure:

-	the requirement for maintaining Next_PDCP_TX_SN is applicable;

-	for the SLRBs associated to packets which have PPPR value lower than the configured PPPR threshold threshSL-Reliability, see TS 36.331 [3], the PDCP entity duplicates the PDCP PDUs, and submits the PDCP PDUs to both associated RLC entities.

For sidelink transmission, the requirement for maintaining TX_HFN is not applicable.

5.1.4	SL Data Reception Procedures

For Sidelink reception, the UE shall follow the procedures in subclause 5.1.2.1.3 with following modifications, except if it receives a PDCP SN which is not "0":

-	the requirement for maintaining Next_PDCP_RX_SN is not applicable;

-	perform the deciphering (if configured) as specified in subclause 5.6.1 and 5.6.2;

-	perform the header decompression (if configured) if SDU Type is set to 000, i.e. IP SDUs.

Otherwise, if the UE receives a PDCP SN which is not "0", the Sidelink reception of the UE shall follow the procedures in subclause 5.1.2.1.4.1 with following modifications compared to above Sidelink reception procedure:

-	the requirement for maintaining Next_PDCP_RX_SN is applicable;

-	perform the re-ordering procedure as specified in subclause 5.1.2.1.4.1.

For sidelink reception, the requirement for maintaining RX_HFN is not applicable.

CHANGE END