3GPP TSG-RAN WG2 Meeting #106												R2-1905762

Reno, USA, 13th May - 17th May 2019 	 							Revision of R2-1903590

Source: 	vivo

Title:  	Discussion on the UE handling for the equal priority of uplink grants

Agenda Item:	11.7.3

Document for:	Discussion and Decision

# Introduction

In the RAN2#105 meeting, RAN2 made the following agreements [1] for the MAC handling of the collided grants:

For cases when MAC prioritizes a grant, MAC prioritizes the grant on which data of the highest priority can be transmitted according to LCP restrictions and priority configured for each LCH.

In this contribution, we discuss how to handle the case that the highest priority of the data to be transmitted in two collided grants are equal.

# Discussion

## Equal highest priority

According to the I-IOT study, the UE could work as a switch which serves multiple URLLC services and eMBB services. From our understanding, the different URLLC services could have different priorities. For example, some I-IOT URLLC services requires even lower packet loss rate than the other URLLC services. And the UE can also have multiple periodical services. Then the UE could have the following LCP restriction configurations:

- Configured grant is configured for logical channel 1 and 2.

- The dynamic grant with short PUSCH duration is configured for logical channel 1 and 3.

Assuming the logical channel priority order for the logical channel 1/2/3 is 1 >2 > 3, when the dynamic grant is collided with the configured grant, the highest priority of the data to be transmitted can be equal.

Observation: The highest priority of the data to be transmitted in two collided grants can be equal.

Here we consider that when the highest priority of the data to be transmitted in two collided grants are equal, the MAC can take further action on the prioritization of the collided grants, by using the following information from the uplink grant:

- Option 1: the second/next highest priority of the data to be transmitted

- Option 2: the uplink grant size

- Option 3: the PUSCH duration

- Option 4: the MCS (e.g. MCS-C-RNTI or MCS-level)

From our understanding, all options listed above have its own benefits, and they are not mutually exclusive with each other. Option 1 can allow higher priority data to be transmitted. Option 2 can allow more data to be transmitted. Option 3 can reduce the transmission latency. Option 4 can provide more reliable transmission. For example, when the PUSCH is scheduled by a PDCCH with CRC scrambled by MCS-C-RNTI, it can be declared that the PUSCH is granted to the URLLC traffic. When mcs-Table in configuredGrantConfig is set to 'qam64LowSE' and the PUSCH is transmitted with configured grant, it can be declared that the configured grant is allocated to the URLLC traffic. However considering the UE complexity, we are not expecting RAN2 to consider all this aspects for the further prioritization in the MAC.

Proposal: RAN2 is kindly requested to discuss which of the following options can be considered by the MAC for the further prioritization of the collided uplink grants when the highest priority of the data to be transmitted is equal:

- Option 1: the second/next highest priority of the data to be transmitted

- Option 2: the uplink grant size

- Option 3: the PUSCH duration

- Option 4: the MCS (e.g. MCS-C-RNTI or MCS-level)

# Conclusions

Based on the analysis given above, we have the following Proposals and Observations：

Observation: The highest priority of the data to be transmitted in two collided grants can be equal.

Proposal: RAN2 is kindly requested to discuss which of the following options can be considered by the MAC for the further prioritization of the collided uplink grants when the highest priority of the data to be transmitted is equal:

- Option 1: the second/next highest priority of the data to be transmitted

- Option 2: the uplink grant size

- Option 3: the PUSCH duration

- Option 4: the MCS (e.g. MCS-C-RNTI or MCS-level)

# Reference

[1] RAN2#105 meeting minutes.