3GPP TSG-RAN WG2 Meeting #106				                            R2-1905950

Reno, USA, 13th May - 17th May 2019			         (was R2-1905212, R2-1904493)

# Agenda Item: 	11.4.8

# Source:	Autotalks, OnStar Europe, Volkswagen AG

# Title:	Motivation for logical UE split in V2X applications

# Document for:		Discussion and decision

# Introduction

The Objective of the V2X WI in RP-190766 [1] includes:

-	NR sidelink scheduling by NR Uu and LTE Uu;

-	Sidelink pre-configuration and configuration by NR Uu and LTE Uu.

TR 38.885 [2] or the V2X WI objective discuss the interaction between NR Uu or LTE Uu with the PC5 protocol stack, actually with the PC5 MAC sublayer and the RRC sublayer.

In this contribution we demonstrate that it is needed to logically split the UE into a U-UE, including the Uu protocol layers and a PC5-UE, including the sidelink protocol layers. The motivation is that such a separation is friendlier for deployment and testing and more future proof to technology upgrades. It will also allow the carmakers to select the categories of Uu system independently from the categories of the PC5 system.

The logical split should be applied during the development of NR-V2X.

# Discussion

## V2X as a subsystem

SAE (Society of Automotive Engineers) document J3161 has defined in [3] the communication between two LTE V2X sub-systems in communicating cars. In Fig.1 is shown the On-Board Equipment (OBE) based on SAE description, the main blocks being the PC5 radio system, the GNSS receiver for satellite positioning and the dedicated processor of this ECU (Electronic Control Unit).

ECU further communicates with the Safety Application ECU.

Note that in this diagram the connection with the cellular system is not shown, being considered a different subsystem.

Observation 1: Uu-UE and the PC5-UE are two separate units.

Fig. 1	V2X system in SAE J3161

## C-ITS device may be a combination of a PC5 standalone and a Uu device

Based on draft EC regulations in “COMMISSION DELEGATED REGULATION on C-ITS” [4], the “C-ITS stations installed on vehicles, handheld or alongside the road infrastructure are considered as products that can be placed on the market as stand-alone assemblies or as parts of larger assemblies. The extent to which C-ITS stations to be installed on vehicles comply with the applicable requirements can be tested before or after installation.”

A C-ITS station based on LTE PC5 mode 4 (out-of-coverage) adheres to C-ITS station definition and can be put as such on the market, given that is allowed by regulations.

Observation 2: The definition of C-ITS system in EU V2X regulation assumes an architecture in which the connection to the cellular system is not defined.

Mode 3 C-ITS station based on NR/LTE PC5 could be achieved by connecting the PC5 UEto an Uu UE implementing the functionality according to the WI in [1].

Based on USA and European regulations, the resulting functionality shall be tested.

## Interaction between Uu and PC5

Based on TR 38.885, Uu interface (MAC, RLC, PDCP, RRC layers) will control the PC5 interface, few examples being provided below:

- Scheduling of transmissions over PC5 (dynamic or SPS) where grants are sent through Uu;

- Synchronizing PC5 with Uu;

- Reporting measurements relative to PC5 to Uu scheduler;

- Configuring the PC5 radio resources by Uu.

These interactions between the two radio interfaces will need coordination. It will be needed to design Rel. 16 protocols in a way assuming a logical split between Uu-based UE and PC5-based UE.

Observation 3: It will be needed to design Rel. 16 protocols in a way assuming a logical split between Uu-based UE and PC5-based UE.

## Testing the On-Board V2X module

The testing of the On-Board modules can take place at different moments in time:

-  Compliance with the regulatory V2X requirements and the following certification takes place before putting a V2X module on the market;

-  Each module may be tested for correct operation in the production phase of the module.

A Tester apparatus or a computer-driven tester is needed for all the above operations; the tester is directly connected to the V2X module. The testing procedure needs to be executed with:

- Ability to receive an external configuration for the V2X system not only by Uu, but also via a CPU or WiFi;

- Ability to exhaustively test the V2X system.

To achieve the testing procedure is needed a logical split between the Uu-UE and PC5-UEsuch that the V2X module test by the development and test engineers will be done separately.

Conclusion 1: Separation between the Uu-UE and PC5-UE is needed for development, testing and certification.

### Different evolution paths and configuration options of Uu and PC5

In time, PC5 technology may be improved, within future releases of 3GPP, based on the automotive market requirements, regulatory requirements and new spectrum allocations, on a different timing as compared with the evolution of the Uu interface. Keeping Uu and PC5 as separate entities will allow a reduced time to market for each technology.

Carmaker may use an older version of PC5 to assure interoperability and continuity of service. Carmakers may decide to use different combinations of PC5, for example, Rel-14 PC5 standalone, Rel-16 PC5 standalone or Rel-14+ Rel-16 PC5/Uu, based on the class of the vehicle.

Independent of that, carmakers define their Uu capability. The capability is derived from the communication grade of the vehicle, which is typically classified by carmakers to HIGH (highest available Uu Category), MID and LOW (minimal connectivity requirement). Uu / PC5 mix-and-match flexibility is required. Correct V2X operation is required regardless of Uu configuration.

Observation 4: Older releases of PC5 may be used by the carmakers for many years to come;

Observation 5: Different configurations are needed for PC5 and for Uu without impacting PC5 operation correctness.

### Global profiling

3GPP standardization provides a toolbox for V2X operation. Regional profiles are needed to specify operational parameters. Unfortunately, there is not a single profile used worldwide, but each geography, currently US, Europe and China, is defining its own profile.

The profiling process is complicated and time consuming. A clear separation of functionality of Uu UE and PC5 UE would improve the clarity and visibility of the specification, which in turn would contribute to faster completion of profile writing, to higher accuracy, and with that to a faster C-V2X deployment.

Observation 6: Clear separation of PC5-UE and Uu-UE functionality contributes to clarity and visibility.

Observation 7: Clarity and visibility will simplify, accelerate and enhance quality of global C-V2X profiles.

# Example of UE split in V2X applications

Our UE split example is based on the separation of the Uu and PC5 RATs.

Fig. 2 reflects this approach; U-UE includes the Uu protocol stacks, while PC5-UE is the PC5 (Vehicular) UE including the PC5 protocol stack.

Fig. 2   Split UE architecture

# PC5-UE interaction with a V2X application Server

The Uu connectivity will allow for the provision of the PC5-UE by a V2X application running on a Server with parameters for V2X communications over PC5 reference point, as described in TS 23.287 [5].

Conclusion 2: Uu connectivity will allow for the provision of the PC5-UE by a V2X application running on a Server with parameters for V2X communications over PC5 reference point.

# Inter-RAT operation

The split-UE architecture is highly suitable for inter-RAT operation, as RAT-specific Information Elements can be used for the implementation of the generic procedures on this interface.

Conclusion 3:  The split-UE architecture is highly suitable for inter-RAT operation.

# Standardization efforts

We believe that the split-UE concept will not introduce additional burden in RAN2, if the U-UE and PC5-UE will be considered from start as separate entities.

We propose to adopt the terminology of PC5-UE during RAN2 related discussions and related specifications. This is not a new concept in 3GPP and already exists in LTE specifications for example, NB-IoT UE and BL-UE. Similar to the given examples, this notation is important to emphasize that V2X is not a sub-feature of NR UE but a complete system with its own protocol stack and HW components and specific capabilities.

In addition, a clear logical split will simplify test development in RAN5, so the overall effort will be lowered.

Observation 8: The impact of split-UE approach on standardization efforts will be low, if the Uu UE and the PC5 UE will be considered from start as logical separated entities.

Observation 9: A clear logical split will simplify test development in RAN5, so the overall effort will be lowered.

# Conclusions and proposal

In the above discussion we had a number of Observations which are listed below:

Observation 1: Uu-UE and the PC5-UE are two separate units.

Observation 2: The definition of C-ITS system in EU V2X regulation assumes an architecture in which the connection to the cellular system is not defined.

Observation 3: It will be needed to design Rel. 16 protocols in a way assuming a logical split between Uu-based UE and PC5-based UE.

Observation 4: Older releases of PC5 may be used by the carmakers for many years to come.

Observation 5: Different configurations are needed for PC5 and for Uu without impacting PC5 operation correctness.

Observation 6: Clear separation of PC5-UE and Uu-UE functionality contributes to clarity and visibility.

Observation 7: Clarity and visibility will simplify, accelerate and enhance quality of global C-V2X profiles.

Observation 8: The impact of split-UE approach on standardization efforts will be low, if the Uu UE and the PC5 UE will be considered from start as logical separated entities.

Observation 9: A clear logical split will simplify test development in RAN5, so the overall effort will be lowered.

We derived the following conclusions and proposal:

Conclusion 1: Separation between the Uu-UE and PC5-UE is needed for development, testing, certification and interaction with a V2X application running on a Server.

Conclusion 2: Uu connectivity will allow for the provision of the PC5-UE by a V2X application running on a Server with parameters for V2X communications over PC5 reference point.

Conclusion 3:  The split-UE architecture is highly suitable for inter-RAT operation.

Proposal 1: RAN2 is invited to consider the split-UE approach in its further standardization work on V2X Release 16.

# References

- RP-190766, “New WID on 5G V2X with NR sidelink”, LG Electronics, Huawei

- 3GPP TR 38.885 V2.0.0 (2019-03), “Technical Specification Group RAN; NR; Study on Vehicle-to-Everything”, (Release 16)

- SAE, “On-Board System Requirements for LTE V2X V2V Safety Communications “Cars Network”

- COMMISSION DELEGATED REGULATION (EU) …/... of 13.3.2019 supplementing Directive 2010/40/EU of the European Parliament and of the Council with regard to the deployment and operational use of cooperative intelligent transport systems

- 3GPP TS 23.287 V0.3.0 (2019-04), Technical Specification Group Services and System Aspects; Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services (Release 16)