3GPP TSG RAN WG2 #106	 R2-1906196

Reno, US, 13 – 17 May, 2019

Agenda Item:	12.3.3.2 Stage-3 aspects CHO

Source: 	Ericsson

Title:  	Triggering of Conditional Handover in LTE

Document for:	Discussion and Decision

# Introduction

In the work item for Even further Mobility Enhancements in E-UTRAN [1] one objective is to improve the robustness at handover. In RAN2#104 it was agreed that conditional handover is one solution that should be considered for improving the handover robustness in LTE.

In RAN2#105bis, the following was agreed regarding the CHO configuration in LTE:

Agreements

1:  Existing Ax measurement events can be used for executing CHO. FFS which Ax events can be used.

…

In this contribution configuration of the triggering condition to be monitored for conditional handover and the UE actions upon reception of that configuration are discussed in this paper.

# Discussion

In typical mobility scenarios, the network may configure an RRC_CONNECTED UE to perform measurements in the PCell and neighbour cells with a given frequency and monitor the conditions for an A3-like (neighbour becomes offset better than source) event. When the condition is fulfilled, i.e., when a neighbour cell with a given frequency becomes an offset better than the PCell during a timer to trigger, the UE shall send an A3-like measurement report containing these measurements. That serves as an indication to the network that a given neighbour cell is better than the PCell and is a candidate for that UE to be a handover target.

Network should configure the offset and hysteresis thresholds so that the UE does not send unnecessary reports (which consumes power and increases signalling). At the same time, they should be set conservative so that when the event is triggered when the UL coverage is still good enough so that the report can reach the network and the DL coverage is good enough to enable the UE to possibly receive a handover command. However, that is not always possible in all cases for different reasons e.g. due to the fast change of radio environment in some cases and/or UE high speed, etc. Hence, measurement reports may never be received by the network or, even in the case it is received, network may never succeed to send the HO command to the UE if there are problems in the DL. These two effects are illustrated below.

Figure 1 Measurement report does not reach the network or HO command does not reach the UE (i.e. UE declares RLF and reestablishment is initiated)

Conditional handover aims to address the problem by avoiding the communication with the network when the link may be close to get degraded. In other words, when the network is more certain that the link is in good conditions the UE receives a conditional handover configuration containing the following:

- A triggering condition configuration;

- A target configuration to be used in the target cell after the handover;

In this contribution further details on the triggering condition configuration and the UE actions upon reception are provided.

## Triggering condition for CHO

A3 events are typically used as a mechanism to assist the network with measurements so that it can take educated coverage-triggered mobility decisions. If we assume a very simple handover function, upon the reception of a A3 measurement report for a given trigger quantity selected by the network (e.g. RSRP or RSRQ) for a given neighbour cell the network may decide to handover the UE to that cell. Hence, since CHO aims to address robustness problems, it makes sense to assume for CHO an A3 like condition where neighbour becomes an offset better than serving as the initial triggering condition. And, if we try to have a similar performance in CHO, it makes sense to consider as initial baseline for the triggering condition for CHO similar triggering conditions for an A3 event (as in reportConfigEUTRA). That would consist at least of an offset and at least one trigger quantity and a time to trigger.

A5 events are typically used to assist the network to take inter-frequency mobility decisions. The event is triggered when two conditions are fulfilled at the same time. The first condition is that the PCell/PSCell becomes worse than absolute threshold1 AND the second condition is that a neighbor/SCell becomes better than another absolute threshold2. The fulfillment of the first condition is an indication that the current PCell/PSCell frequency is not acceptable anymore (especially if the network has also configured an A3 event to find potential neighbor in the same frequency and is not receiving measurements reports with better neighbor cells). If the second condition is fulfilled, that indicates to the network that at the same time there are better cells in another frequency.

It has been debated in a previous contribution whether we would need a time to trigger configuration. In the case of measurement reporting triggering, that is used to avoid the triggering of a measurement report driven by a spike in radio conditions that are not stable, to avoid unnecessary signalling and only transmit these measurements when they are really stable. In our view that is even more important in CHO, otherwise we would risk triggering too early conditional handovers which could lead to conditional handover to the wrong cell and/or simultaneous handover ping-pongs.

Discuss how to support triggering of CHO based on multiple trigger quantities:

    - Enhance A3/5 event to be based on multiple trigger quantities;

    - Enable two instances of A3/A5 event (possibly with different quantities) to be linked to the same HO command in CHO configuration.

Trigger quantity

In an A3 event, the network may also configure additional information to be reported, which may be used by the network as an input to the handover function. For example, the network may configure the UE to report both quantities RSRQ and RSRP and not only the trigger quantity. That would allow the network to take a decision not only based on a single trigger quantity.

ReportConfigEUTRA ::=				SEQUENCE {

triggerType							CHOICE {

event								SEQUENCE {

eventA3								SEQUENCE {

a3-Offset							INTEGER (-30..30),

reportOnLeave						BOOLEAN

},

}

},

hysteresis							Hysteresis,

timeToTrigger						TimeToTrigger

},

triggerQuantity						ENUMERATED {rsrp, rsrq},

reportQuantity						ENUMERATED {sameAsTriggerQuantity, both},

A typical measurement configuration is when the network configures two A3 events (or two measId(s) associated to A3, to be more precise) for the same measObject (i.e. same target frequency) but with different trigger quantities and thresholds e.g. one for RSRP and one for RSRQ. That reflects the wish to only handover to a neighbour cell when not only RSRP, but also RSRQ is expected to be better than the PCell. As handover decisions are only made when the criteria based on both trigger quantities are fulfilled, the network may wait for the reception of both types of measurement reports before it takes a handover decision.

To have similar performance in conditional handover, we need to support the triggering of CHO when conditions are fulfilled for more than one trigger quantity (e.g. RSRP and RSRQ). That could be done either by associating the monitoring of two conditions (e.g. two A3-like event configuration) to a single HO command to be executed upon the fulfilment of the condition, or by creating a new condition based on multiple triggers (which can be decided during stage-3 discussions).

Using the current structure of ReportConfigEUTRA as baseline, that could be done by adding a new value for triggerQuantity. for example as follows:

triggerQuantity-r15					ENUMERATED {rsrp, rsrq, rsrpandrsrp},

Support for other Ax events

Triggering conditions should be based on events or combination of events that are mapped to events used as input for handover decisions, also including reported information. In that sense, the agreement we had to support A3/5 event as trigger condition is sufficient to address conditional handover.

Depending on the signaling solution for configuring events linked to CHO, the additional effort to include A1, A2, A4 and A6 events as triggering conditions may not be significant. Future use cases may benefit from that e.g. in case we want to enchain conditional re-configurations where one event leads to another before it may lead to a handover. In that sense, we should either support them in Rel-15 or at least make the framework extendable.

One way to do that is to link an instance of reportConfigEUTRA to a HO command in the CHO configuration so the signaling would be ready for extending that to other Ax events.

In CHO configuration, link at least one instance of Ax event configuration in ReportConfigEUTRA to a HO command.

Cell and target frequency

For each measurement to be reported (e.g. for A3 event, proposed as starting baseline for CHO triggering), the UE is configured with a measurement identifier, a report configuration and a measurement object (that indicates the target frequency and the neighbour cell).

For conditional handover triggering condition configuration, in addition to the previous parameters discussed in P1-P4, the UE also requires information about the neighbour cell to be monitored, such as:

- A neighbour cell identifier to be monitored;

- A neighbour frequency of the cell identifier;

Triggering condition configuration for CHO needs a neighbour cell identifier and neighbour frequency information (e.g. ARFCN).

Time to trigger

Even though we have agreed to support A3/5 event as the trigger condition for CHO, perhaps not all fields are applicable, especially parameters related to the triggering once measurements are available such as hysteresis, time to trigger, etc. Time to trigger configuration, in the case of measurement reporting triggering, is used to avoid the triggering of a measurement report driven by a spike in radio conditions that are not stable, to avoid unnecessary signaling and only transmit these measurements when they are really stable. In our view that is even more important in CHO, otherwise we would risk triggering too early conditional handovers which could lead to conditional handover to the wrong cell and/or simultaneous handover ping-pongs.

Time to trigger configuration is applicable when configuring A3/5 event as CHO trigger conditions. FFS whether hysteresis is also applicable.

# Conclusion

RAN2 is kindly asked to discuss the following proposals:

- Discuss how to support triggering of CHO based on multiple trigger quantities:

    - Enhance A3/5 event to be based on multiple trigger quantities;

    - Enable two instances of A3/A5 event (possibly with different quantities) to be linked to the same HO command in CHO configuration.

In CHO configuration, link at least one instance of Ax event configuration in ReportConfigEUTRA to a HO command.

Triggering condition configuration for CHO needs a neighbour cell identifier and neighbour frequency information (e.g. ARFCN).

Time to trigger configuration is applicable when configuring A3/5 event as CHO trigger conditions. FFS whether hysteresis is also applicable.

# References

- RP-181544, Work Item on Even further Mobility Enhancements in E-UTRAN, China Telecom, RAN#81, September 2018