3GPP TSG RAN WG2 #106	 R2-1906215

Reno, US, 13 – 17 May, 2019	(Revision of R2-1903520)

Agenda Item:	11.9.3.1 Conditional handover

Source: 	Ericsson

Title:  	Conditional handover failures in NR

Document for:	Discussion and Decision

# Introduction

In the work item for NR Mobility Enhancements [1], one objective is to improve the robustness at handover. In RAN2#105bis it has been agreed to support Conditional handover (CHO) in NR. This contribution addresses the different failures cases that may occur in CHO related procedures.

# Discussion

## Conditional Handover

In conditional handover the network configures the UE with triggering conditions for when a handover should be executed. When the conditions are fulfilled, the UE executes the handover without any further order from the network. The advantage of the procedure is that the HO Command may be provided to the UE at an earlier stage before the radio conditions have become poor, which increases the chance of a successful transmission of the message. The basic signalling flow for conditional handover is shown in Figure 1.

Figure 1: Conditional handover

CHO Configuration to the UE

In NR, the source receives the configuration prepared by target and provides to the UE in an RRCReconfiguration message containing a reconfigurationWithSync field with instructions for the UE to access the target cell e.g. RACH parameters, cell identity, frequency, SMTC configuration, etc. In CHO, similar information needs to be provided to the UE for a target candidate. In addition, the UE also needs the triggering condition associated to it and further measurement configuration e.g. an A3 event like configuration and a measurement object associated, so that the provided dedicated RRC configuration is only applied if/when the condition is fulfilled.

In RAN2#105bis in Xi’an, that has been agreed as follows:

Agreements

. ..

d/ The baseline operation for Conditional HO procedure assumes HO command type of message contains HO triggering condition(s) and dedicated RRC configuration(s). UE accesses the prepared target when the relevant condition is met.

. . .

Below we discuss different error cases that may occur.

UE is unable to comply with triggering condition configuration within CHO configuration(s)

In NR, a reconfiguration failure is declared in case the UE is unable to comply with (part of) an RRC configuration. As the agreements above says, CHO contains the triggering conditions configuration(s) and possibly an associated measurement configuration (e.g. measurement object) to be applied by the UE upon reception of the message, and another part that shall only be applied upon fulfilment of the triggering conditions.

Hence, upon receiving a CHO configuration, the UE shall evaluate if it is able to comply with each triggering condition configuration(s) and/or associated measurement configuration since these shall be applied upon reception of the message.

Upon receiving a CHO configuration, the UE evaluates if it is able to comply with each triggering condition configuration (and associated measurement configuration).

If the CHO configuration only contains one triggering condition in the CHO configuration (including measurement configuration associated, like a measurement object), and the UE is not able to comply with it, the UE declares a reconfiguration failure, similarly to the inability to comply with a measurement configuration in Rel-15. Then, it continues using the configuration used prior to the reception of the CHO configuration and initiates an autonomous action such as re-establishment or NAS recovery. If the CHO configuration contains multiple triggering condition in the CHO configuration, and the UE is able to comply with at least one of them, the UE would in principle not have to declare a failure and could simply indicate to the network which triggering configurations the UE was not able to comply with e.g. in a reconfiguration complete message sent from the UE to the network to acknowledge that CHO was configured. However, that may not be a likely event, especially if the triggering conditions are set by the source. Hence, a simpler solution would be to follow the same principle as in the case of a single failed configuration and take autonomous actions such as RRC re-establishment or NAS recovery (upon entering IDLE and informing higher layers of a failure).

The CHO configurations contain at least dedicated RRC configurations prepared by target, with content equivalent to an RRCReconfiguration including reconfigurationWithSync. In both NR and LTE, this is considered sensitive information which is only provided after security is activated. A paper discussing the security aspects of CHO configurations has been provided in [X] where the same principle is proposed i.e. CHO configuration is only provided after security is activated. If that principle is agreed, as in the case when the UE is unable to comply with an RRCReconfiguration, the UE should initiate connection re-establishment.

If the UE is not able to comply with at least one triggering conditions configurations in CHO configuration the UE declares a CHO failure and initiates RRC re-establishment.

UE is unable to comply with dedicated RRC configuration(s) upon CHO execution

As discussed above, a CHO configuration also contains an RRCReconfiguration (with a reconfigurationWithSync) for each target cell candidate and associated to a triggering condition. The RRCReconfiguration per target is only applied when the associated triggering condition is fulfilled. Hence, it is not obvious whether the UE shall verify the compliance of each RRCReconfiguration per target upon reception of a CHO configuration.

A first alternative is if the UE verifies the compliance of each RRCReconfiguration upon reception of the CHO configuration. If the CHO configuration is for a single target candidate (i.e. it only contains a single RRCReconfiguration), and If the UE is able to comply with it, the UE monitors the condition and if the condition is fulfilled the UE is certain that there will be no compliance error upon applying the RRCReconfiguration. If the configuration is not compliant, the UE could indicate that to the network e.g. in an acknowledgement message to the CHO configuration. Main benefit of this solution is that the UE does not need to monitor a trigger condition unless that is associated to a compliant RRCReconfiguration, which may be found out as early as possible. Another advantage is that the UE does not have to perform an autonomous action (e.g. re-establishment) since the source gNodeB is still under control of the UE and the UE the CHO configurations are not meant to be provided under very poor radio conditions like in the legacy handover.

However, a possible drawback is if the CHO configuration is for multiple target candidates. In that case there will be multiple RRCReconfiguration(s) and, verifying the compliance of that upon reception of the CHO configuration means verifying the compliance of each RRCReconfiguration (or equivalent) per target candidate, which increases the processing delay when configuring CHO (and possibly delays the start of the monitoring of trigger conditions).

A second alternatively is if UE only verifies the compliance of the RRCReconfiguration(s) in CHO upon the fulfilment of an associated triggering conditions i.e. when one of these are to be applied, as in the case of a legacy handover. The advantage of this solution is the reduced processing delay (more relevant for the case of multiple target candidates) for CHO configuration. However, it may lead to cases where the UE unnecessarily monitors triggering conditions associated to RRCReconfiguration that are non-compliant.

As the inability to comply with an RRCReconfiguration is not a very typical case, the second alternative seems more reasonable to be assumed as baseline since in most cases the processing delay may be reduced.

UE is only required to verify the compliance of the RRCReconfiguration(s) in CHO upon the fulfilment of an associated triggering conditions.

According to P1-P3, if all triggering conditions and measurement configurations associated are compliant the UE monitors the triggering conditions upon receiving the CHO configuration. If one of the triggering conditions are fulfilled, the UE performs the CHO execution which should be similar to a handover execution i.e. It consists of starting the timer T304 (or equivalent) and applying the stored RRCReconfiguration with reconfigurationWithSync message associated to the target cell candidate for which the trigger condition has been fulfilled.

When applying the message, which would be equivalent to having received at that moment an RRCReconfiguration with reconfigurationWithSync, the UE may be unable to comply with (part of) the configuration prepared by that target candidate.

Upon CHO execution, UE may be unable to comply with a dedicated configuration from target candidate.

Timer T304 expires upon CHO execution

In NR reconfiguration with sync, the RRCReconfiguration with reconfigurationWithSync contains a value for timer T304, that is started upon the reception of the message. Then, if the timer expires before the UE can access the target and send the RRCReconfigurationComplete according to target’s configuration, the UE declares a handover failure.

When the CHO trigger condition is fulfilled, and the UE executes CHO, the also UE starts an equivalent timer T304, configured in the stored reconfigurationWithSync associated to the selected target cell candidate. As in reconfiguration with sync procedure, before the UE can access the target candidate cell fulfilling the CHO trigger condition and being selected by the UE (in case multiple triggering cells), the timer T304 may expiry.

Upon CHO execution, timer T304 may expire before the UE can access the selected target cell candidate.

RLF declaration while CHO is being monitored

In NR, RLF is declared e.g. upon the timer T310 expires. That timer is started when the upper layers detect that the downlink quality is going below acceptable levels, which is controlled by indications provided by lower layers.

While the UE is monitoring CHO triggering conditions, as the network would likely configure the trigger conditions to be close to when the radio conditions of the PCell may not be so good any longer, RLF may be declared while the UE is monitoring CHO triggering conditions.

Timer T310 may expire while UE is monitoring CHO trigger conditions.

UE actions upon RLF declaration while CHO is being monitored, expiry of timer T304 in CHO or non-compliance with a dedicated RRC configuration(s) upon CHO execution

In NR, as in LTE, these different cases would lead respectively to a reconfiguration failure (in case the UE would not comply with a dedicated configuration issued by a target cell candidate), handover failure (in case the timer T304 expires after the UE tries to access a target cell candidate) and RLF declaration (in case timer T310 expires while the UE is monitoring CHO triggering conditions). And, upon declaration of any of these failures, the UE would first select a suitable cell to only then transmit an RRCReestablishmentRequest.

But then, in the case of CHO, if the UE selects a cell for which it has a stored RRCReconfiguration with reconfigurationWithSync, which is possible in the cases above, it would be much simpler, faster and efficient to apply the stored RRCReconfiguration with reconfigurationWithSync and complete the CHO compared to reverting the configuration and transmit an RRCReestablishment request.

Upon T304 expiry, T310 expiry or non-compliance of a dedicated RRC configuration of a target cell in CHO, the UE executes CHO if it selects a cell for which it has a stored CHO configuration.

And, as there is no requirement concerning which cell the UE shall select, the UE may prioritize to select a suitable cell for which the UE has a stored RRCReconfiguration with reconfigurationWithSync.

Upon CHO failure, the UE performs cell selection and prioritizes cells for which it has a stored CHO configuration.

# Summary

RAN2 is kindly asked to discuss the following proposals:

- Upon receiving a CHO configuration, the UE evaluates if it is able to comply with each triggering condition configuration (and associated measurement configuration).

- If the UE is not able to comply with at least one triggering conditions configurations in CHO configuration the UE declares a CHO failure and initiates RRC re-establishment.

- UE is only required to verify the compliance of the RRCReconfiguration(s) in CHO upon the fulfilment of an associated triggering conditions.

- Upon T304 expiry, T310 expiry or non-compliance of a dedicated RRC configuration of a target cell in CHO, the UE executes CHO if it selects a cell for which it has a stored CHO configuration.

- Upon CHO failure, the UE performs cell selection and prioritizes cells for which it has a stored CHO configuration.

# References

- RP-181433, Work Item on NR Mobility Enhancements, Intel, RAN#80, La Jolla, USA, May 21 – May 25 2018 September 2018.

- R2-1906211, Triggering of conditional handover in NR, Ericsson

- R2-1906210, Configuration of conditional handover in NR, Ericsson

- R2-1906212, Conditonal handover execution in NR, Ericsson