3GPP TSG-RAN WG2 Meeting#106          							R2-1906267

Reno, USA, 13th – 17th May 2019

Source: 			ZTE Corporation, Sanechips

Title: 	Extend C-RNTI in NTN

Agenda item:		11.6.3.1

Document for: 	Discussion and Decision

- Introduction

In this contribution, we share some consideration on extension of C-RNTI used in NTN.

- Discussion

- RNTI: Cell radio network temporary identifier, which is used to identify a UE in a certain cell.

In NR, as shown in Table 7.1-1 from TS38.321 [1], the C-RNTI is a 16 bit value which can be used to identify up to 65519 UEs in a certain cell.

| Value (hexa-decimal) | RNTI |
| --- | --- |
| 0000 | N/A |
| 0001–FFEF | RA-RNTI, Temporary C-RNTI, C-RNTI, MCS-C-RNTI, CS-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, INT-RNTI, SFI-RNTI, and SP-CSI-RNTI |
| FFF0–FFFD | Reserved |
| FFFE | P-RNTI |
| FFFF | SI-RNTI |

Considering the typical beam footprint size of GEO and non-GEO shown in Table 4.1-1 from TR38.821 [2] with the assumption that one footprint covers the area of a cell, the cell size in GEO and non-GEO will be 31400km2 ~785000km2 and 7850km2 ~ 196250km2, respectively.

Table 4.1-1: Types of NTN platforms

| Platforms | Altitude range | Orbit | Typical beam footprint size |
| --- | --- | --- | --- |
| Low-Earth Orbit (LEO) satellite | 300 – 1500 km | Circular around the earth | 100 – 500 km |
| Medium-Earth Orbit (MEO) satellite | 7000 – 25000 km |  | 100 – 500 km |
| Geostationary Earth Orbit (GEO) satellite | 35 786 km | notional station keeping position fixed in terms of elevation/azimuth with respect to a given earth point | 200 – 1000 km |
| UAS platform (including HAPS) | 8 – 50 km (20 km for HAPS) |  | 5 - 200 km |
| High Elliptical Orbit (HEO) satellite | 400 – 50000 km | Elliptical around the earth | 200 – 1000 km |

If the existing C-RNTI is used in NTN, the density of the active users which can be identified by the C-RNTI is 0.08~2.09 UE/km2 for GEO and 0.33~8.35 UE/km2 for Non-GEO, respectively.

Although we are still discussing the detailed user density and activity factor for each usage scenario, the active user density of IOT connectivity has already been agreed: 400 UE/km2 * 1.00% = 4.00 UE/km2. Taking the all the use scenarios into consideration, the active user density will be far more than 4.00 UE/km2.

For Non-GEO case, the existing C-RNTI may be sufficient. While for GEO case, the existing C-RNTI is not sufficient even for IOT connectivity only.

Observation 1: The existing 16-bit C-RNTI in NR is not sufficient to identify active UEs in a GEO cell.

Proposal 1: The C-RNTI size should be extended to identify active UEs in a certain NTN cell.

Since the size of the C-RNTI is defined in RAN1 and has both RAN1 and RAN2 impacts, an LS should be sent to RAN1:

Proposal 2: An LS is sent to RAN1 to ask them to discuss the extension of C-RNTI and define the size of the extended C-RNTI in NTN [3].

- Conclusion and proposals

With the above analysis, we have the following observation and proposals:

Observation 1: The existing 16-bit C-RNTI in NR is not sufficient to identify active UEs in a GEO cell.

Proposal 1: The C-RNTI size should be extended to identify active UEs in a certain NTN cell.

Proposal 2: An LS should be sent to RAN1 to ask them to discuss the extension of C-RNTI and define the size of the extended C-RNTI in NTN [3].

- Reference

- TS38.321, Medium Access Control (MAC) protocol specification

- TR38.821, Solutions for NR to support non-terrestrial networks (NTN)

- R2-1906268_[DRAFT] LS on extension of C-RNTI in NTN(FS_NR_NTN_solutions)