3GPP RAN WG2 Meeting #106	R2-1906407
Reno, USA, 13th -17th May, 2019	Revision of R2-1903555

Agenda Item:	11.7.3

Source:	InterDigital

Title:	Resource Conflicts for Data-only Intra-UE Prioritization

Document for:	Discussion, Decision

# Introduction

A work item on NR Industrial IoT was agreed in [1], with NR intra-UE prioritization/multiplexing as one of the main objectives. In RAN2#105, the following was agreed:

RAN2 shall study resource conflicts between multiple active configured grants, in addition to Scenarios 2 and 3, part of UL data-data prioritization.

UE prioritization of a grant when there is at most one dynamic grant in the set of conflicting grants (scenario 2 and CG/CG collision) shall be addressed. MAC specifies currently the UE prioritization of such cases, and modifications to MAC would be required.

For cases when MAC prioritizes a grant, MAC prioritizes the grant on which data of the highest priority can be transmitted according to LCP restrictions and priority configured for each LCH.

This contribution addresses enhancements for an NR I-IoT UE supporting multiple services of different QoS requirements, with focus on intra-UE prioritization for transmission and retransmission.

# Discussion

I-IoT devices support mixed traffic types of varying latency and reliability requirements, possibly concurrently for a given UE. A UE may have multiple uplink grants available for transmission of data, possibly each for traffic with different priority levels, which transmissions could overlap in the time domain.

## Prioritization between new transmission and retransmission

Discussions on UL intra-UE data-data prioritization have focused on prioritization between new transmissions. However, there can be scenarios where the UE has received a grant for a retransmission that conflicts with another resource on which new data of high priority could be transmitted. For example, the UE may receive a dynamic grant for retransmission of a TB (for which the NDI has not been toggled) that conflicts in time with a configured grant on which new data of higher priority can be transmitted.

The agreed prioritization rule in MAC for conflicts between grants intended for new transmissions rely on LCP configured priorities and mapping restrictions, whereby the grant on which data of the highest priority can be transmitted is prioritized. When the UL resources in conflict involve a retransmission, LCP is not applicable for the grant on which a retransmission is indicated.

Proposal 1: 	Prioritization between a grant for a new transmission conflicting with a grant for a retransmission is supported as part of the intra-UE UL data-data prioritization.

When at least one of the grants in conflict is for a retransmission, the UE may determine the priority of the grant for the retransmission based on:

- HARQ process state priority:

The UE can save the priority that was initially associated with the HARQ process for the new transmission. Such priority may then be re-used when MAC determines a resource conflict between the grant for the re-transmitted TB and another UL grant on which new data can be transmitted. Consequently, the UE may keep the priority associated with the data in the TB as part of the state for each ongoing HARQ process and update the priority whenever the NDI is toggled.

- Indication of priority level in grant:

The UE may rely on a gNB-assigned priority level for the two resources in conflict. For example, if the priority indicated for the dynamic grant on which a TB is retransmitted is higher that the priority assigned for the conflicting configured grant on which a new transmission can be made, the UE prioritizes the dynamic grant. The UE prioritizes the configured grant otherwise. Details of such gNB-assigned priority are discussed further in the following section.

Proposal 2: 	The UE keeps the priority initially associated with the HARQ process for the new transmission as part of the state of the corresponding HARQ process.

Proposal 3: 	The UE determines the priority of a grant for a retransmission using the priority kept in the corresponding HARQ process state, when at least one of the grants in conflict is for a retransmission.

Proposal 4: 	If supported, the priority of any grants, including those for a retransmission, may be derived from the gNB-indicated priority-level for the grant.

## Priority level indication

NR supports transmissions with different numerologies and/or PUSCH durations for the same UE. While there is a relationship between the HARQ timeline and the QoS provided by the scheduler, how such is provided is also impacted by other elements such as selected transmission parameters (MCS, PRB allocation, etc.), link adaptation, cell load and multiplexing of transmissions between UEs. There is no necessary restriction or direct correspondence between the QoS associated with a DRB and a specific PUSCH duration, numerology, and/or grant type.

A scheduler implementation could benefit from scheduling eMBB traffic using shorter TTIs. For example, this may be used to shorten the slow start phase for small TCP transfers, or simply to maximize resource allocation in a cell configured with bandwidth parts of different numerologies. This may be better achieved by efficiently multiplexing users at the smallest (e.g. mini-slot, or slot) time granularity even for some eMBB traffic (e.g. short data bursts and/or small PDU size). It may thus be challenging to network and scheduling implementations to optimize physical resource usage when supporting all NR service types concurrently on a given carrier.

Consequently, it is beneficial if RRC could configure each LCH with one or more priority levels for the purpose of scheduling, which can be then considered for the purpose of prioritization between grants. Further, the gNB may dynamically indicate a priority level for a dynamic grant and configure (e.g. by RRC signaling) a priority level for each configured grant.

Proposal 5: 	RRC supports configuring a LCH with one or more priority level values.

Proposal 6: 	RRC supports configuring a priority level per configured grant.

From the perspective of the network, each priority level could correspond to a scheduling strategy associated to the transmission of a transport block. A priority level may also be indicated for an UL grant on which a TB is retransmitted, which can be indicated in cases such grant conflicts with another grant.

From the UE MAC’s perspective, for a new transmission, MAC would use the priority level indicated by the gNB for a given UL grant to determine which LCH(s) to consider when constructing the transport block using. The mapping procedure would then be performed by the UE without any knowledge of the underlying physical layer characteristics or actual scheduling strategy from the gNB. From the UE MAC’s perspective, for a re-transmission,

Proposal 7: 	The MAC entity multiplexes data only from LCH(s) configured with the priority level value matching the value associated with the UL grant.

Given it is within the scope of RAN1 to determine how the UE receives the priority level indication for a dynamic grant for a new transmission. RAN2 may send an LS to RAN1. For example, a priority level can be explicitly signaled or encoded in a DCI signaling. Alternatively, the priority level of a dynamic grant can be inferred based on the type of MCS table signaled (e.g. for selection between a set of two values), which can be based on receiving a DCI scrambled by the MCS-C-RNTI or receiving a specific DCI format on a specific UE-specific search space.

## Impact of Intra-UE prioritization on MAC

Depending on the outcome of intra-UE prioritization, MAC may drop an already built MAC PDU, e.g. when the another overlapping PUSCH or SR is prioritized over the grant intended for the PDU. Further, a MAC PDU may have already been generated and delivered to PHY before MAC determines that the PUSCH for the MAC PDU is de-prioritized. For such de-prioritized PDU, the UE may drop the PDU altogether, e.g. when the PUSCH transmission has not started or if the PDU is prioritized or preempted by another uplink transmission at the physical layer.

However, from a behavioural perspective, such dropped PDU is no different than a PDU dropped due LBT failure in NR-U. In NR-U, a PDU dropped due to LBT failure that was intended for transmission on a configured grant can be transmitted on the next configured grant occasion. Further, a PDU dropped due to LBT failure on a dynamic grant is only transmitted when a retransmission grant is received, as in R15, given the network is aware that the transmission was not made. The same UE behaviour should apply for a PDU dropped due to intra-UE prioritization.

Observation 1:	From a behavioral perspective, a PDU dropped due to intra-UE prioritization is no different than a PDU dropped due LBT failure in NR-U.

Proposal 8: 	A MAC PDU dropped due to intra-UE prioritization that was intended for transmission on a configured grant can be transmitted on the next configured grant occasion.

Observation 2:	For a MAC PDU dropped due to intra-UE prioritization that was intended for transmission on a dynamic grant, the UE may retransmit the PDU when it receives a retransmission grant, as for any retransmission in R15.

Another impact of dropped PDUs is possible delay of reporting buffer status or power headroom for time-sensitive traffic. Given BSR and PHR MAC CEs are cancelled upon inclusion in the MAC PDU, these MAC CEs are not automatically regenerated on another available grant. Given importance of such reports to scheduling TSC traffic, the UE should be able to regenerate such MAC CEs on another upcoming available grant or the prioritized grant.

Proposal 10: 	MAC regenerates BSR and PHR MAC CEs included in a PDU dropped due to intra-UE prioritization on the prioritized transmission.

# Summary and Proposals

This contribution addresses enhancements for an NR I-IoT UE supporting multiple services of different QoS requirements, with focus on intra-UE prioritization between a new transmission and a retransmission. RAN2 should discuss the above and agree to the following proposals:

Proposal 1: 	Prioritization between a grant for a new transmission conflicting with a grant for a retransmission is supported as part of the intra-UE UL data-data prioritization.

Proposal 2: 	The UE keeps the priority initially associated with the HARQ process for the new transmission as part of the state of the corresponding HARQ process.

Proposal 3: 	The UE determines the priority of a grant for a retransmission using the priority kept in the corresponding HARQ process state, when at least one of the grants in conflict is for a retransmission.

Proposal 4: 	If supported, the priority of any grants, including those for a retransmission, may be derived from the gNB-indicated priority-level for the grant.

Proposal 5: 	RRC supports configuring a LCH with one or more priority level values.

Proposal 6: 	RRC supports configuring a priority level per configured grant.

Proposal 7: 	The MAC entity multiplexes data only from LCH(s) configured with the priority level value matching the value associated with the UL grant.

Observation 1:	From a behavioral perspective, a PDU dropped due to intra-UE prioritization is no different than a PDU dropped due LBT failure in NR-U.

Proposal 8: 	A MAC PDU dropped due to intra-UE prioritization that was intended for transmission on a configured grant can be transmitted on the next configured grant occasion.

Observation 2:	For a MAC PDU dropped due to intra-UE prioritization that was intended for transmission on a dynamic grant, the UE may retransmit the PDU when it receives a retransmission grant, as for any retransmission in R15.

Proposal 10: 	MAC regenerates BSR and PHR MAC CEs included in a PDU dropped due to intra-UE prioritization on the prioritized transmission.

# References

RP-190728, New WID: Support of NR Industrial Internet of Things (IoT), Nokia, Nokia Shanghai Bell

R2-1902232, “Report Breakout Session NR UP, IAB, NR-U, IIOT”, Vice Chairman (Mediatek)