3GPP TSG-RAN2 Meeting #106 	R2-1906745

# Reno, USA,  13 – 17 May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1066 | rev | - | Current version: | 15.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to PWS reception |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN2#102 meeting, the following agreements were made. According to the agreements, the UE is not required to switch its active DL BWP to the initial DL BWP to receive SI updates if the UE is not configured common search space on the active DL BWP. However, the current text would require the UE to switch its active DL BWP to the initial DL BWP if the UE is not provided with a searchSpaceOtherSystemInformation on the active BWP. R2-1809006 Email Discussion [101bis#44] [NR] – SI Reception in Connected Mode Samsung discussion Agreements 1 On SI modification indication, MIB contents are not re-acquired by UEs' in RRC_CONNECTED 2 MIB does not need to be provided via dedicated signalling, as all the required information can be provided to the UE via servingCellConfigCommon. 3: If the UE’s active DL BWP is not configured with common search space then UE is not required to acquire SI updates from broadcast 4: NW may provide updated SI by dedicated signalling (Can be used at least for the case if the UE’s active DL BWP is not configured with common search space.) 5 RRCReconfiguration message is used to provide SIBs through dedicated 6 RRCReconfiguration can perform SIB updates (other than for SIB1 parameters that are provided via servingCellConfigCommon) without reconfiguration with sync 7 No need for additional requests from the UE to the NW to determine which SIBs to be provided in dedicated signalling. 8 Any SIB (including PWS SIBs) can be provided to the UE through dedicated signaling. 9: How long updated SI is broadcast in UE’s active DL BWP, when SI-RNTI and P-RNTI common search space is configured, is left to network implementation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The UE acquires PWS if the UE capable of PWS receives a Short Message indicating PWS and the UE is provided with searchSpaceOtherSystemInformation on the active DL BWP. Impact analysis Impacted 5G architecture options: Standalone Impacted functionality: PWS reception in RRC_CONNECTED Inter-operability: If the UE does not implement the CR, the UE in RRC_CONNECTED receiving a PWS indication switches its active DL BWP to the initial DL BWP if the UE is not provided with a searchSpaceOtherSystemInformation on the active DL BWP. The communication between the UE and the network on the active DL BWP is interrupted due to the BWP switching. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The communication between the UE and the network on the active DL BWP may be interrupted due to that the UE switches to the initial DL BWP to receive PWS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 5.2.2.2.2	SI change indication and PWS notification

A modification period is used, i.e. updated SI (other than for ETWS and CMAS) is broadcasted in the modification period following the one where SI change indication is transmitted. The modification period boundaries are defined by SFN values for which SFN mod m = 0, where m is the number of radio frames comprising the modification period. The modification period is configured by system information. The UE receives indications about SI modifications and/or PWS notifications using Short Message transmitted with P-RNTI over DCI (see clause 6.5). Repetitions of SI change indication may occur within preceding modification period.

UEs in RRC_IDLE or in RRC_INACTIVE shall monitor for SI change indication in its own paging occasion every DRX cycle. UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

ETWS or CMAS capable UEs in RRC_IDLE or in RRC_INACTIVE shall monitor for indications about PWS notification in its own paging occasion every DRX cycle. ETWS or CMAS capable UEs in RRC_CONNECTED shall monitor for indication about PWS notification in any paging occasion at least once every defaultPagingCycle if the UE is provided with common search space on the active BWP to monitor paging.

If the UE receives a Short Message, the UE shall:

1>	if the UE is ETWS capable or CMAS capable, the etwsAndCmasIndication bit of Short Message is set, and the UE is provided with searchSpaceOtherSystemInformation on the active BWP:

2> immediately re-acquire the SIB1;

2>	if the UE is ETWS capable and si-SchedulingInfo includes scheduling information for SIB6:

3>	acquire SIB6, as specified in sub-clause 5.2.2.3.2, immediately;

2>	if the UE is ETWS capable and si-SchedulingInfo includes scheduling information for SIB7:

3>	acquire SIB7, as specified in sub-clause 5.2.2.3.2, immediately;

2>	if the UE is CMAS capable and si-SchedulingInfo includes scheduling information for SIB8:

3>	acquire SIB8, as specified in sub-clause 5.2.2.3.2, immediately;

1> if the systemInfoModification bit of Short Message is set:

2>	apply the SI acquisition procedure as defined in sub-clause 5.2.2.3 from the start of the next modification period.