3GPP TSG-RAN WG2 Meeting #106	R2-1906779

# Reno, Nevada, USA, May 13 – 17, 2019									          Revision of R2-1903135

Agenda item:	11.8.2.1

Source: 	Qualcomm Incorporated

Title: 	DL and UL NR Positioning Procedures

Document for:	Discussion and Decision

# 1. 	Introduction

A new work item on "NR Positioning Support" has been agreed at RAN#83 [1]. The objectives include:

Define functional interfaces, signaling and procedures including UE reporting, to support NR RAT-dependent positioning for the NR positioning techniques listed in RAN1 objectives [RAN2]

The RAN1 objectives list the following NR positioning techniques:

-	DL-based positioning:

-	DL Time Difference of Arrival (DL-TDOA);

-	DL Angle of Departure (DL-AoD).

-	UL-based positioning:

- 	UL Time Difference of Arrival (UL-TDOA);

-	UL Angle of Arrival (UL-AoA).

-	Combined DL and UL based positioning:

-	Round-trip time (RTT) with one or more neighbouring gNBs/TRPs (multi-cell RTT).

-	Enhanced Cell-ID (E-CID).

To support the above positioning techniques, the following measurements are supposed to be defined by RAN1 [1]:

-	UE measurements (for serving, reference, and neighbouring cells):

-	DL RSTD (reference signal time difference) measurements.

-	DL RSRP (reference signal received power) measurements.

-	UE RX-TX time difference measurements.

- 	gNB measurements:

-	UL RTOA (relative time of arrival) measurements.

-	UL Angle of Arrival (AoA) measurements.

-	UL RSRP (reference signal received power) measurements.

-	gNB RX-TX time difference measurement.

In this contribution, we discuss the additional NR positioning methods for Rel-16 and propose the NG-RAN positioning procedures applicable for combined DL and UL based positioning methods (i.e., multi-cell RTT). This procedure can be considered as a general case, where downlink-based (e.g., DL-TDOA (OTDOA)) and uplink-based (e.g., UL-TDOA (UTDOA)) could be supported as subsets of the proposed procedure.

We note that in some scenarios, an LMF may need to employ several positioning methods in order to most accurately locate a UE. For example, if a network operator has both unsynchronized small cells with simple (e.g. dipole) antennas and synchronized macro cells with massive antenna arrays, an LMF might use multi-cell RTT and/or ECID for nearby small cells and AOA, AOD, DL-TDOA and/or UL-TDOA for more distant macro cells. Such multi-method positioning of a UE will be more efficient if the different position methods can employ common or overlapping procedures with common signalling messages, since this could reduce both the numbers of messages being exchanged and message content.

# 2. 	Standard UE Positioning Methods

The standard positioning methods supported for NG-RAN access in Release 15 are summarized in Table 4.3.1-1 of TS 38.305 [2] which is repeated below:

Table 1: Supported UE positioning methods in NR Release 15.

| Method | UE-based | UE-assisted, LMF-based | NG-RAN node assisted | SUPL NOTE 6 |
| --- | --- | --- | --- | --- |
| A-GNSS | Yes | Yes | No | Yes (UE-based and UE-assisted) |
| OTDOA Note1, Note 2 | No | Yes | No | Yes (UE-assisted) |
| E-CID Note 3, Note 4 | No | Yes | Yes | Yes for E-UTRA (UE-assisted) |
| Sensor | Yes | Yes | No | No |
| WLAN | Yes | Yes | No | Yes |
| Bluetooth | No | Yes | No | No |
| TBS Note 5 | Yes | Yes | No | Yes (MBS) |
| NOTE 1: This includes TBS positioning based on PRS signals. NOTE 2: In this version of the specification only OTDOA based on LTE signals is supported. NOTE 3: In this version of the specification only E-CID based on LTE signals is supported. NOTE 4: This includes Cell-ID for NR method. NOTE 5: In this version of the specification only for TBS positioning based on MBS signals. NOTE 6: SUPL 2.0 is not defined to support NR. |  |  |  |  |

In Rel-15, the OTDOA positioning method makes use of the measured timing of downlink signals received from multiple TPs, comprising eNBs, ng-eNBs and PRS-only TPs, at the UE [2]. This can be extended in Rel-16 to support measured timing of downlink signals received from gNBs. Therefore, the Rel-16 DL-TDOA technique listed in section 1 above could be supported by extending the existing OTDOA positioning method.

Enhanced Cell ID (E-CID) positioning refers to techniques which use UE and/or NG-RAN radio resource and other measurements to obtain the UE location estimate. E-CID in NR Release 15 supports measurements on LTE signals only (e.g., RSRP, RSRQ), as well as inter-RAT measurements (e.g., GERAN, UTRAN, WLAN measurements). The measurements for NR signals (e.g., SS-RSRP, SS-RSRQ for NR) could be added to the E-CID method in Rel-16.

DL Angle of Departure (DL-AoD in section 1 above) is not explicitly supported in NR Rel-15, and therefore, would require a definition in Rel-16. DL-AoD is based on UE DL RSRP (reference signal received power) measurements. If the RSRP measurements are made on Rel-15 signals, DL-AoD could be included in E-CID positioning.  If the UE RSRP measurements are performed using DL-PRS signals assistance data may be required (similar to OTDOA). The required DL-PRS assistance data could be added to E-CID positioning, or a separate DL-AoD method could be defined in Rel-16. DL-RSRP measurements could also be reported as part of OTDOA positioning.

UL Time Difference of Arrival positioning (UL-TDOA in section 1 above) is based on UL RTOA (relative time of arrival) measurements performed at gNBs using UL SRS transmissions from the target UE. UL-TDOA is supported in LTE as UTDOA positioning, but not in NR Rel-15. Therefore, a new positioning method would have to be defined in NR Rel-16 to support UL RTOA measurements.

UL Angle of Arrival (UL-AoA in section 1 above) is based on gNB angle measurements using UL SRS signals transmitted by the target UE, and therefore, similar to UTDOA. UTDOA and UL-AoA could be combined into an "Uplink Positioning Method" which supports RTOA and UL-AoA (angle) measurements. Note, UTDOA in LTE is also simply referred to as "Uplink" in Table 4.3-1 of TS 36.305 [3].

Multi-cell RTT is based on UE and gNB Rx-Tx Time Difference Measurements for serving and neighbouring cells (see also section 3 below). This requires the UE to measure the DL-PRS signals and the gNBs to measure the UL SRS transmission from the UE.  The measurements at the UE and gNBs are TOA measurements of received signals (DL-PRS at the UE and UL-SRS at the gNBs) which are referred to the corresponding transmit time reference (UL PRS transmission at the UE, and DL PRS transmission at the gNBs).  Therefore, multi-cell RTT can be considered as a combination of DL-TDOA and UL-TDOA (but with different measurement types).

Based on the above discussion, Table 1 above could be modified for Release 16 as follows:

Table 2: Supported UE positioning methods in NR Release 16.

| Method | UE-based | UE-assisted, LMF-based | NG-RAN node assisted | SUPL |
| --- | --- | --- | --- | --- |
| A-GNSS | Yes | Yes | No | Yes (UE-based and UE-assisted) |
| OTDOA Note1 | FFS | Yes | No | Yes |
| E-CID Note 2 | No | Yes | Yes | Yes for E-UTRA (UE-assisted) |
| Sensor | Yes | Yes | No | No |
| WLAN | Yes | Yes | No | Yes |
| Bluetooth | No | Yes | No | No |
| TBS Note 3 | Yes | Yes | No | Yes (MBS) |
| Uplink Positioning Note 4 | No | No | Yes | FFS |
| Multi-Cell RTT | No | Yes | Yes | FFS |
| NOTE 1: This includes TBS positioning based on PRS signals. NOTE 2: This includes DL AoD (FFS). NOTE 3: In this version of the specification only for TBS positioning based on MBS signals. NOTE 4: This includes UTDOA and UL-AoA. |  |  |  |  |

# 3. 	Multi-Cell RTT Positioning

OTDOA and UTDOA positioning methods are based on time-of-arrival (TOA) measurements performed on downlink signals or uplink signals, respectively. Although these methods have been shown to be effective, they have a serious problem in installing and maintaining hardware for very precise BS time synchronization.

Round-Trip-Time (RTT) positioning uses two-way time-of-arrival measurements and requires in principle no time synchronization between BSs. However, a coarse BS time synchronization is desired in order to reduce interference and increase hearability from multiple transmission points. This time synchronization requirement is similar to the TDD synchronization requirements (e.g., micro-seconds level synchronization instead of nano-seconds as in case of OTDOA/UTDOA).

RTT measurements are in principle already supported in LTE but for a serving eNB only (Tadv Type 1 and Type 2 measurements [3], [4]). RTT measurements are also widely used in WLAN for (sub-)metre level positioning [5].

Figure 1 illustrates the principle of obtaining distance information from two-way time-of-arrival measurements (UL and DL measurements). Precise timing (TOA) measurements are required, similar to OTDOA/UTDOA location. RTT ranging measurements also rely on being able to compensate for any delays that the responding device may add in receiving and transmitting a signal. However, multiple responding devices (e.g., base stations) do not need to be precisely synchronized as in e.g., OTDOA/UTDOA location. The position of a device can then be determined based on distance (RTT) measurements to multiple base stations, as illustrated in Figure 2.

Figure 1: Principle of determining distance between two devices using UL and DL measurements.

Figure 2: Principle of multilateration in two dimensions from three base stations.

The measurements to support multi-cell RTT are the UE Rx-Tx Time Difference and gNB Rx-Tx Time Difference Measurement listed in section 1 above. These measurements haven’t been defined by RAN1 yet. If the "Initiating Device" in Figure 1 above is the target UE and the "Responding Device" is a gNB, the UE Rx-Tx Time Difference may correspond to (t3-t0) and the gNB Rx-Tx Time Difference may correspond to (t1-t2), and therefore, the RTT would be UE Rx-Tx Time Difference + gNB Rx-Tx Time Difference.

An example timing diagram with two gNBs is illustrated in Figure 3 below. The upper part of Figure 3 shows the uplink-downlink timing relation for a serving gNB1. According to TS 38.211 [6] clause 4.3.1 an uplink frame i for transmission from the UE starts before the start of the corresponding downlink frame at the UE, where NTA corresponds to the Timing Advance between downlink and uplink and Tc is the basic time unit for NR [6].  is a constant as defined in TS 38.133 Table 7.1.2-2 [7]. The NTA is typically determined such that the UL transmissions from the UEs in a cell are aligned. Therefore, the UE Rx-Tx Time Difference (Rx1 – Tx) ideally corresponds to twice the propagation delay 1 and the gNB Rx-Tx Time Difference would be zero in the example of Figure 3.  The same UE uplink frame i arrives at a neighbour gNB2 the propagation delay 2 later.  gNB2 may have a synchronization offset  relative to the serving gNB1 and records the TOA of the UE uplink frame i RX2. The RTTs for gNB1 and gNB2 in this example could then be determined as:

RTT1 = (UE Rx1 – Tx) + (gNB Rx1 – Tx1)

RTT2 = (UE Rx2 – Tx) + (gNB Rx2 – Tx2).

As mentioned above, the measurements have not been finalized by RAN1 yet. However, the Figure 3 illustrates that the basic UE and gNB measurements are TOA measurements (e.g., UE RX1, RX2; and gNB RX1, RX2). Those measurements can be related to the appropriate TX time so that Rx-Tx time differences can be reported to determine RTT as illustrated in Figure 1. Figure 3 also illustrates that any synchronization offset  between gNBs does not impact the RTT calculation. It may, however, impact the TOA search window centre at a neighbour gNB.

Figure 3: Exemplary uplink-downlink timing relation for two gNBs.

# 4. 	Combined Downlink and Uplink Positioning Procedures

The positioning procedures described in the following are based on the Rel-15 positioning architecture where positioning procedures are controlled and instigated by the LMF in the 5GC.

Figure 4: UL/DL measurement procedure.

Precondition:	The LMF knows (at least) the approximate timing of each involved gNB (e.g., SFN Initialisation Time). A NRPPa procedure (similar to the OTDOA Information Exchange procedure [8]) supporting also unsynchronized gNBs may need to be defined for that purpose.

1.	The AMF requests the location of a target device (e.g., for an NI-LR or 5GC MO-LR/MT-LR).

2.	The LMF may request the positioning capabilities of the target device using the LPP Capability Transfer procedure.

3.	The LMF sends a NRPPa UL PRS Request message to the serving gNB to request UL PRS configuration. The request may include any desired UL PRS configuration (i.e., SRS required to fulfill the QoS) and a time interval for how long this UL PRS configuration is needed.

4.	The serving gNB determines the resources available for UL PRS.

5.	The serving gNB provides the UL PRS configuration to the LMF in a NRPPa UL PRS Response message.

6.	The LMF may determine the gNBs nearby the approximate location (e.g. given by the serving cell) of the target device. If there are no or not sufficient DL-PRS resources configured on these gNBs, the LMF may initiate a NRPPa procedure to configure (or reconfigure) the DL-PRS on the gNBs.

7.	The LMF provides the UL-PRS configuration to the selected gNBs in a NRPPa UL PRS Measurement Request message. The message includes all information required to enable the gNBs to perform the UL measurements. The message may also include a start time when each gNB should expect the UL transmissions from the target device, and a search window for the UL (e.g., RTOA or Rx-Tx) measurements.

8a.	The LMF sends a LPP Provide Assistance Data message to the target device. The message includes any required assistance data for the target device to perform the necessary DL PRS measurements (e.g., cell-IDs, DL-PRS configuration, measurement search window, etc.) as well as any information necessary to transmit UL PRS (e.g., UL PRS configuration).

NOTE:	Alternatively, the UL PRS configuration may be provided in the LPP Request Location Information message.

8b.	The LMF sends a LPP Request Location Information message to request DL PRS mesurements as well as a request to transmit UL PRS according to the UL PRS configuration received in the assistance data (or included in this message). The target device begins the UL PRS transmission at the indicated start time and for the provided duration.

NOTE:	Whether the target device can continue the UL PRS transmission during/after a handover is FFS.

9a.	The target device performs the DL-PRS measurements from all gNBs provided in the assistance data at step 8a.

9b.	Each gNB configured at step 7 measures the UL PRS transmissions from the target device.

10. The target device reports the DL PRS measurements to the LMF in a LPP Provide Location Information message. The message may also include an indication of successful completion of the UL PRS transmission.

11.	Each gNB reports the UL PRS measurements to the LMF in a NRPPa UL PRS Measurement Response message.

12. The LMF determines the RTTs for each gNB for which corresponding UL and DL measurements were provided at steps 10 and 11, calculates the position of the target device, and provides the location estimate to the AMF in a Location Response message.

## DL-only (e.g., OTDOA) or UL-only (e.g., UTDOA, AoA) measurement procedures

From the description above, it can be seen that DL-only (e.g.,  OTDOA) or UL-only (e.g., UTDOA, AoA) positioning can be supported as subsets of the UL/DL measurement procedure shown in Figure 3:

(a)	DL-only positioning: Steps 3, 4, 5, 7, 9b, 11 would not be performed.

(b) UL-only positioning: Steps 6, 9a would not be performed.

Therefore, the procedure introduced in Figure 4 above supports DL, UL, and DL+UL based NR positioning methods. Only the measurements made/reported would necessarily be different for the various methods, for example:

OTDOA:

- DL Reference Signal Time Difference (RSTD), measured at step 9a.

UTDOA:

- UL Relative Time of Arrival (RTOA), measured at step 9b.

UL-AoA:

- UL Angle of Arrival, measured at step 9b.

Multi-Cell RTT:

- UE Rx-Tx Time Difference, measured at step 9a.

- gNB Rx-Tx Time Difference, measured at step 9b.

For downlink E-CID positioning with Rel-15 signals, steps 3-7 and 9b,11 may not be needed.

Note further that while assistance data can also be different (e.g. not needed at the UE for UL positioning), there is a potential to share assistance data that is sent to a UE (e.g. for DL PRS configurations) for DL and DL+UL position methods or that is sent to gNBs (e.g. for UL PRS configuration) for UL and DL+UL position methods. Such sharing of assistance data should be easier to accomplish with procedures and signalling messages (e.g. for LPP [9] and NRPPa [8]) which are common or overlapping for multiple position methods.

# 5. 	Summary

In this contribution we discussed the additional NR positioning methods for Rel-16 and proposed the positioning procedures to support positioning solutions based on combined downlink- and uplink measurements (e.g., to support multi-cell RTT location). It has been shown that these procedures can support downlink-only (e.g., OTDOA) and uplink-only (e.g., UTDOA, UL-AoA) positioning as well using a subset of the proposed messages in the procedures.

The positioning procedures between LMF and UE can be based on LPP [9], which requires appropriate extensions to support the new NR positioning methods. Therefore, it is proposed:

Proposal 1: 	Add the following positioning methods to NR Rel-16:
-	Uplink Positioning, comprising UTDOA and UL-AoA;
-	Multi-Cell RTT positioning;

- DL-AoD. (Note)

Support NR-OTDOA and NR-ECID by extending the corresponding Rel-15 methods.

Note:	It needs to be evaluated whether DL-AoD is a separate method or is combined with NR-OTDOA or NR-ECID.

Proposal 2: 	Support combined downlink- and uplink NR positioning procedures for multi-cell RTT location, where downlink-only (e.g., NR-OTDOA, NR-AoD) and uplink-only (e.g., NR-UTDOA, UL-AoA) solutions are supported as subsets of this combined downlink- and uplink NR positioning procedures.

Proposal 3:	Extend LPP to support DL-only (e.g., NR-OTDOA, NR-AoD), UL-only (e.g., NR-UTDOA, UL-AoA), and DL+UL  (e.g., multi-cell RTT) positioning methods.

# References

[1]	RP-190752, "New WID: NR Positioning Support".

[2]		3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[3]		3GPP TS 36.305: "Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN".

[4]		3GPP TS 36.214: "Physical layer; Measurements".

[5]		IEEE Standard 802.11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications.

[6]	3GPP TS 38.211: "NR; Physical channels and modulation".

[7]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[8]		3GPP TS 38.455: "NR Positioning Protocol A (NRPPa)".

[9]		3GPP TS 36.355: "LTE Positioning Protocol (LPP)".