3GPP TSG-RAN WG2 #106	Tdoc R2-1906857

Reno, US, 13 – 17 May 2019

Agenda Item:	11.7.4

Source:	Ericsson

Title:	On activation and deactivation of primary and secondary RLC entities

Document for:	Discussion, Decision

# 1	Introduction

One of the description objectives for this Work Item is to specify mechanisms relating to dynamic control of how a set or subset of configured RLC entities or legs are used for PDCP duplication. This contribution looks at different aspects of the activation and deactivation of the RLC entities.

In Release 15, PDCP data duplication is configured with a Primary RLC entity (described as “primaryPath” in 38.331) and a Secondary RLC entity which, for DRBs, can be either the MCG or the SCG for the DC case, and one of the two RLCs configured for the CA case. The primary RLC entity has two main purposes. The first one is related to the transmission of PDCP Control PDUs. These PDUs are transmitted only over the primary RLC entity. The second purpose is to specify the RLC entity in which data will be transmitted when DC is configured, and the UE buffer is below a certain configured threshold. When PDCP data duplication is configured and not activated or it becomes deactivated, the purpose of the primary RLC entity is still the same as when DC is configured i.e. to set the primary RLC entity in which PDCP Data and Control PDUs are transmitted. When PDCP data duplication is activated, the purpose of the primary RLC entity is only to set the RLC entity in which PDCP control PDUs are transmitted.

When more than two RLC entities can be configured, it needs to be discussed whether a similar behaviour as in Release 15 or a different behaviour is more beneficial.

# 2	Discussion

### Dynamic activation and deactivation of primary and secondary RLC entities

In [1], different configurations are depicted to support the transmission of multiple copies of a PDCP PDU over up to 4 RLCs with DC-only, CA-only, and the combination of DC with CA. Regardless of the architecture and the number of RLC entities, the UE behaviour needs to be specified, i.e. the RLC entity used, when PDCP data duplication is configured but not activated or becomes deactivated. It also needs to be specified in which RLC entity PDCP Control PDUs are transmitted when PDCP data duplication is activated or deactivated.

In Release 15, when PDCP duplication is not activated, the primary RLC entity carries PDCP Control PDUs and PDCP Data PDUs. When the network configures ul-DataSplitThreshold, the PDCP entity delivers PDCP Data PDUs to the secondary RLC entity when the data in the buffer is above the configured threshold. When PDCP duplication is activated, only the PDCP Data PDUs are duplicated and transmitted over the primary and secondary RLC entities. The PDCP Control PDUs are not duplicated and are solely transmitted over the primary RLC.

In Release 16, up to 4 RLC entities can be configured and they can be activated and deactivated via MAC CEs. Deactivating an RLC entity for PDCP Data duplication means that the PDCP entity does not deliver PDCP Data PDUs to the deactivated entity. Secondary RLC entities may be deactivated but the question is if the primary RLC entity can be deactivated for PDCP data duplication.

To allow full flexibility to decide in which of the configured RLC entities duplicated PDCP Data PDUs are transmitted, we think that the primary RLC entity should also be subject to activation and deactivation for PDCP data duplication. As outlined above, deactivation for PDCP data duplication only applies to PDCP Data PDUs and does not affect to PDCP Control PDUs. PDCP Control PDUs should follow Release 15 behaviour and should be transmitted over the primary RLC entity.

Deactivation for PDCP data duplication of the primary RLC entity is supported. When the primary RLC is deactivated for PDCP data duplication, the PDCP entity does not deliver PDCP Data PDUs to this entity.

Deactivation for PDCP data duplication of secondary RLC entities is supported.

As in Release 15, PDCP Control PDUs are not duplicated and are only transmitted over the primary RLC entity.

### Dynamic change of primary RLC entity (primaryPath)

Dynamically change the primary RLC entity i.e. a secondary RLC entity can become the primary RLC entity, is an enhancement which does not seem necessary. When PDCP duplication is used, the primary RLC entity dictates the RLC entity which will carry PDCP Control PDUs. There are two types of PDCP Control PDUs, PDCP Status Reports, and intersected ROHC feedback. ROHC feedback cannot be considered to have the same reliability requirements as the PDCP Data PDUs configured with duplication. PDCP Status Reports are only needed upon e.g. a handover. PDCP Status Reports will not play any role for URLLC services given the latency requirements. Thus, we think that there is no need to dynamically change the primary RLC entity. The primary RLC entity configured by the network should be enough to carry this data when generated.

While activating or deactivating a certain configuration by MAC CE is common, changing an RRC configuration using a MAC command is something different from a protocol layer modelling and should be avoided.

Primary RLC cannot be dynamically changed through MAC CEs.

### RLF behaviour upon SCG failure and no RLC entity active in MCG

When DC is configured, there can be a situation in which the RLC entities configured in the MCG are deactivated for PDCP duplication (i.e. PDCP does not deliver PDCP Data PDUs to the RLCs in the MCG) and only the RLC entities in the SCG remain active. If an RLF in the SCG is experienced, the RRC will suspend the transmission in the SCG and reset the MAC of the SCG. This unintentionally stops all data transmissions of a PDCP duplication bearer even if it has RLC entities configured in the MCG which are deactivated. This is negative for URLLC services.

In this case, one simple option is to activate an RLC in the MCG of a duplication bearer, while legacy SCG procedure is following. This way, URLLC service is not interrupted, as data can still be transmitted via the MCG.

When no RLC in MCG is activated for PDCP data duplication and the UE experiences RLF in the SCG, the UE should:

  - Activate an RLC belonging to the MCG (if none was active), and/or

  - Only suspend the RLC entity which experienced the RLF if it was due to the max number of RLC retransmissions

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Deactivation for PDCP data duplication of the primary RLC entity is supported. When the primary RLC is deactivated for PDCP data duplication, the PDCP entity does not deliver PDCP Data PDUs to this entity.

Proposal 2	Deactivation for PDCP data duplication of secondary RLC entities is supported.

Proposal 3	As in Release 15, PDCP Control PDUs are not duplicated and are only transmitted over the primary RLC entity.

Proposal 4	Primary RLC cannot be dynamically changed through MAC CEs.

Proposal 5	When no RLC in MCG is activated for PDCP data duplication and the UE experiences RLF in the SCG, the UE should:

a.	Activate an RLC belonging to the MCG (if none was active), and/or

b.	Only suspend the RLC entity which experienced the RLF if it was due to the max number of RLC retransmissions

# References

R2-1906853, Scenarios for dynamic activation, Ericsson, RAN2#106