3GPP TSG-RAN WG2 Meeting #106	R2-1906951

Reno, USA, 13 – 17 May 2019

Agenda item:	11.2.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Beam measurements linked to successful and failed handovers

WID/SID:	FS_LTE_NR_data_collect – Release 16

Document for:	Discussion and Decision

# 1	Introduction

In RAN3 there is ongoing discussion on MRO use case related solutions. During last RAN3#103bis meeting, several companies proposed including in beam measurements in the RLF report or the HO report [1],[2],[3]. However not enough details were provided in terms of which information should be included.

The Successful HO report is proposed in the current TR 37.816 v0.3.0:

#### 5.3.2.5 		Successful HO Report (FFS)

[…] The Successful Handover Report may be included in the Handover Complete message (RRCReconfigurationComplete) transmitted from UE to target NG-RAN node over RRC

In this contribution, we discuss which information should be included in the scope of the Study Item.

# 2	Discussion

During handover in 5G systems using beam forming, the UE is switching between two cells but in particular between a beam in the serving cell and a beam in the serving cell.  The HO preparation is triggered based on cell level quality measurements from the UE (an aggregated form of beam measurements on both the serving and neighbouring cells) even though the UE is served by a single beam at moment in time.

Observation 1: While handover is performed between two cells based on cell level quality measurements, the UE is actually served by beams in serving and target cell side.

Beam level measurements can also be included in the HO request message. This information can be used by the Target cell for allocating dedicated RACH resources for the UE which are then included in the HO command.

Observation 2: Beam level measurements can be used by the target cell for allocation of dedicated RACH resources.

It would therefore be beneficial, for HO preparations and execution optimization, for the network to know as much as possible about the beams involved in the HO process. More specifically we propose introducing the following information in the RLF and HO report, HO failure report and HO confirmation messages i.e. following a failed and a successful handover:

- Last serving beam and/or serving cell beam measurements before executing the HO.

-  due to variations in the channel quality and/or beam management, the last serving beam of the UE on the serving cell may be different that the beam used for sending the HO report and/or the beam used for sending the HO command.

- Beam and/or beam measurement of the beam used by the UE on the target cell (successfully or not). - due to changing radio conditions and UE movement between the moment when the measurement report was sent to the serving cell and the actual moment of executing the HO, that this beam may be different that the one pre-configured by the Target cell. In case of  failure, the target beam refers to the beam in the Recovery cell (i.e. the cell where the UE re-establishes).

Proposal:  Include in HO report and RLF report the following information: Last serving beam and/or serving cell beam measurements before executing the HO and beam and/or beam measurement of the beam used by the UE on the target cell.

# 3	Conclusion

In this document, we made the following proposals and observations:

Observation 1: While handover is performed between two cells based on cell level quality measurements, the UE is actually served by beams in serving and target cell side.

Observation 2: Beam level measurements can be used by the target cell for allocation of dedicated RACH resources.

Proposal:  Include in HO report and RLF report the following information: Last serving beam and/or serving cell beam measurements before executing the HO and beam and/or beam measurement of the beam used by the UE on the target cell.

# References

R3-191870, Successful Handover Report for MRO, 3GPP TSG-RAN WG3 #103bis, Xi'an, China, April 2019

R3-191502, Discussion on Mobility Robust Optimization in 5G system, 3GPP TSG-RAN WG3 #103bis, Xi'an, China, April 2019

R3-191869, Reporting RLM related issues upon handover and access to the target cell, 3GPP TSG-RAN WG3 #103bis, Xi'an, China, April 2019