3GPP TSG-RAN WG2 meeting #106	            R2-1906977

Reno, USA, 13th May– 17th May, 2019

Title:	[Draft] LS to RAN3 on E2E flow control

Release:	Release 16

Work Item:	NR_IAB

Source:	Huawei, HiSilicon [3GPP TSG RAN WG2]

To:	3GPP TSG RAN WG3

Contact person:	Yulong Shi

shiyulong5@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

- Overall description

In the RAN2#105bis meeting, following agreements were achieved for IAB flow control.

- Flow control is supported in both upstream and downstream directions in order to avoid congestion-related packet drops on IAB-nodes and IAB-donor DU.

  - In upstream direction, UL scheduling is considered baseline for hop-by-hop flow control. End-to-end flow control is FFS.

  - In downstream direction, the NR UP protocol is considered baseline for end-to-end flow control. Hop-by-hop flow control is FFS.

For the DL E2E flow control, the NR UP protocol is considered as baseline for DL end-to-end flow control, where DU can provide a DDDS frame which only supports UE DRB granularity to CU-UP by F1-U to handle downlink congestion. Since the current F1-U based mechanism is design for traditional access link, the need for additional enhancements of multi-hop backhaul link should be further clarified and studied.

Question 1: Whether the current F1-U based mechanism is sufficient for IAB DL E2E flow control?

For the UL E2E flow control, UL scheduling is considered as baseline for hop-by-hop flow control and this can be left to IAB node implementation. In order to have a unified design for both DL and UL E2E flow control, whether to introduce E2E flow control mechanism for UL can be decided by RAN3.

Question 2: Whether to introduce the E2E flow control mechanism for IAB UL?

- Actions

RAN2 respectfully asks RAN3 to work on the detailed design of IAB UL and DL E2E flow control mechanisms, taking the above into account.

- Date of next RAN2 meetings

3GPPRAN2#107	26 - 30 August 2019   	Prague   	Czechoslovakia

3GPPRAN2#107bis	14 - 18 October 2019   	China