3GPP TSG-RAN WG2 Meeting #106	R2-1907264

Reno, USA, 13 – 17 of May 2019	revision of R2-1904289

Agenda item:	11.9.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Considerations for failure recovery in NR

WID/SID:	NR_Mob_Enh - Release 16

Document for:	Discussion and Decision

# 1	Introduction

RAN#80 plenary approved a Work Item (WI) titled “NR mobility enhancements” to enhance the basic mobility functions that were introduced in NR Release-15 [1]. One of the objectives of the WI is to:

- To study solution(s) to improve HO/SCG change reliability and robustness especially considering challenges in high/med frequency focusing on the following identified solutions but not limited.

  - Conditional handover

  - Fast handover failure recovery

Having optimizations that minimize the number of handover and radio link failures should be given the highest priority. However, such failures cannot be avoided for all UE speeds and use cases. As such, it is also important to optimize failure recovery to minimize user data interruption time, in case the failure already occurred.

# 2	Overview of failure recovery mechanisms

In [2], we discussed factors that affect failure recovery latency and potential mechanisms to minimize the latency. These discussions focus on failure due to strong interference, which occurs in cell boundaries and is basically a Handover failure if the Radio Link Failure (RLF) is declared while Time to Trigger (TTT) is running. Some of the mechanisms to minimize failure recovery latency are:

- Early activation of SRB1 during RRC Reestablishment procedure [3][4]. Similar to RRC Resume procedure, SRB1 can be activated early such that the RRC Reestablishment message from the network to the UE can be sent using SRB1 during RA message 4. This further implies the RRC Reconfiguration message can also be sent to the UE in RA message 4, either multiplexed with the RRC Reestablishment message or separately. This saves a round trip latency of the air interface, compared to activating SRB1 after RRC Reestablishment Complete is received from UE by the network in RA message 5.

- Proactive provisioning of System Information (SI) of the potential target cells to UE. This reduces the failure recovery latency as system information acquisition of the target cell can be done in a shorter time span, for example, compared to accessing the target through a contention based random access procedure. However, blindly providing the system information of all the neighbouring cells of the serving cell to the UE leads to significant wastage of resources such as RACH resources and signalling overhead to acquire the SI of the neighbour cells. Therefore, it is important to provide only the SI of a selected set of cells to the UE. Such selection might require optimizations to minimize its impact on the network, for example, with the assistance of MRO.

- Availability of the UE AS context in the target cell for recovery. This reduces the time required to fetch UE context from the source cell. As such, if there are cells that have UE AS context and the UE is aware of them, they should be prioritised over cells with no UE AS context. For example, in conditional handover, the UE is provided with the list of candidate cells which have UE AS context received during HO preparation, and it can prioritise such cells for RRC Reestablishment during recovery from a failure.

- Early RLF/HOF declaration to quickly initiate RRC Reestablishment procedure. This is important in situations where the link to the source cell is poor and there is also a target cell for recovery with good signal quality. In fact, timer T312 is introduced for this purpose in LTE [5], which would important to adopt it in NR as well, where it is started if a measurement report is triggered for measurement identity for which T312 is configured (i.e. there is at a target cell with good cell quality) while T310 is running (i.e. the radio link to the source cell is poor). However, T312 may still need further enhancements as it will be discussed in the next section.

Considering the above discussions on different mechanisms to minimize failure recovery latency, the following observations and proposals are made:

Observation 1: Enhancement for early activation of SRB1 is beneficial to speed up failure recovery.

Proposal 1: The network may provide SI of potential target cells for failure recovery to reduce failure recovery latency. However, optimizations to minimize its impact on the network, for example with the assistance of MRO, should be considered.

Proposal 2: UE should prioritise re-establishment towards cells with proactively prepared UE AS context. It is FFS if different T312 timer values can be used for different cells to prioritise and speed up re-establishment towards cells with UE context.

# 3	Timer T312 enhancements

The timer T312 is introduced in LTE to enable early RLF declaration, instead of waiting for timer T310 to expire to declare an RLF [5]. The rationale behind the need for an early declaration of RLF is that if the link to the source cell is poor and there is a target cell with good enough signal quality to trigger a measurement report, the link to the source cell is unlikely to be recovered and the RRC Reestablishment to the target cell is likely to succeed [6]. Therefore, for an early declaration of RLF, at least the following conditions should be fulfilled:

- The radio link to the source cell is poor. In LTE [5], this condition is fulfilled if timer T310 is running, i.e. radio link problem is detected towards the source cell.

- There is a neighbour cell with good signal quality. In LTE [5], this condition is fulfilled upon triggering a measurement report for a measurement identity for which T312 has been configured.

In LTE [5], timer T312 is started if condition 2 is fulfilled after condition 1, see Figure 1, and while condition 1 still holds. If condition 2 is fulfilled while T310 is running, it implies that condition 1 is fulfilled before condition 2 and condition 1 holds when condition 2 is fulfilled. In other words, condition 2 should be fulfilled before timer T310 expires, i.e. T2-T1 should be less than timer T310 value, or before T310 is aborted if the link to the source cell improves. Notice that, if by the time condition 2 is fulfilled the remaining running time of T310 is less than timer T312 value, the RLF can only be declared after T310 expires as T312 would be still running when T310 expires.

Figure 1:When condition 1 is fulfilled before condition 2, e.g. UE triggers a measurement report for a measurement identity associated with timer T312 after a radio link problem is detected towards the source cell, i.e. while timer T310 is running.

It would also be important to enhance timer T312 to enable the possibility of early RLF declaration when condition 2 is met before condition 1, e.g. in a case as shown in Figure 2 [6]. This scenario is particularly relevant to conditional handover, as measurement report to prepare candidate cells, which can also be associated with timer T312, is likely to be triggered before radio link problem is detected towards the source cell. However, it should be ensured that condition 2 still holds when condition 1 is fulfilled, e.g. before starting timer T312.

Figure 2:When condition 1 is fulfilled after condition 2, e.g. UE triggers a measurement report for a measurement identity associated with timer T312 before a radio link problem is detected towards the source cell, i.e. while timer T310 is running.

The UE can verify directly or indirectly whether condition 2 (e.g. the condition that triggered a measurement report for a measurement identity which is associated with timer T312 is still valid) holds when condition 1 (e.g. Timer T310 is running) is fulfilled before starting timer T312. It is especially important to directly check if condition 2 still holds when condition 1 is fulfilled when the time elapsed starting from the fulfilment of condition 2 until condition 1 is fulfilled is large, for example larger than a suitably selected parameter T, T2-T1 >= T. However, the UE might assume that condition 2 still holds when condition 1 is fulfilled if the time elapsed starting from the fulfilment of condition 2 until condition 1 is fulfilled is relatively small, for example less than parameter T, i.e. T2-T1 <= T.

Adopting LTE’s T312 to NR is important. However, it might be useful to discuss whether to adopt T312 with enhancements from the onset or to first adopt the T312 with basic functionalities and work towards adding enhancements later.

Proposal 3: Adopt LTE’s T312 timer to NR. Discuss whether to adopt T312 with enhancements from the onset or to first adopt T312 with basic functionalities and work towards adding enhancements later.

Proposal 4: Timer T312 should be enhanced to include the case where UE triggers a measurement report for a measurement identity which is associated with T312 before a radio link problem towards the source cell is detected, i.e. T310 starts running.

Proposal 5: Timer T312 can be started as early as when T310 starts running if measurement report has already been triggered and the condition for triggering the measurement report still holds at the start of T310.

# 4	Conclusion

The contribution discussed failure recovery for LTE. Several factors that might impact the recovery process are discussed, and the following observations and proposal are made.

Observation 1: Enhancement for early activation of SRB1 is beneficial to speed up failure recovery.

Proposal 1: The network may provide SI of potential target cells for failure recovery to reduce failure recovery latency. However, optimizations to minimize its impact on the network, for example with the assistance of MRO, should be considered.

Proposal 2: UE should prioritise re-establishment towards cells with proactively prepared UE AS context. It is FFS if different T312 timer values can be used for different cells to prioritise and speed up re-establishment towards cells with UE context.

Proposal 3: Adopt LTE’s T312 timer to NR. Discuss whether to adopt T312 with enhancements from the onset or to first adopt T312 with basic functionalities and work towards adding enhancements later.

Proposal 4: Timer T312 should be enhanced to include the case where UE triggers a measurement report for a measurement identity which is associated with T312 before a radio link problem towards the source cell is detected, i.e. T310 starts running.

Proposal 5: Timer T312 can be started as early as when T310 starts running if measurement report has already been triggered and the condition for triggering the measurement report still holds at the start of T310.

# References

- RP-181433, New WID: NR mobility enhancements, 3GPP TSG RAN Meeting #80, La Jolla, USA, May 21-May 25 2018.

- R2-1900611, Considerations on failure recovery in NR, 3GPP TSG-RAN WG2 Meeting #105, 25th of February – 1st of March 2019

- R2-1814322, Enhancements to re-establishment procedure in LTE, 3GPP TSG-RAN WG2#103bis, Chengdu, China, 8th – 12th October 2018

- R2-1805012, Reestablishment procedure for NR, 3GPP TSG RAN WG2 Meeting #101bis, Sanya, China, 16th - 20th April 2018

- 3GPP TS 36.331, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification, December 2018

- R2-1814323, Enhancing the handling of timer T312, 3GPP TSG-RAN WG2#103bis, Chengdu, China, 8th – 12th October 2018