3GPP TSG-RAN WG2 Meeting #106	R2-1907284

Reno, USA, 13 – 17 May 2019

Agenda item:	11.2.1.2

Source:	Charter Communications

Title:	Handling Uplink LBT Failures in NR-U

WID/SID:	NR-U WID - Release 16

Document for:	Discussion and Decision

# 1	Introduction

Operation in unlicensed spectrum requires conformance to regulatory rules. Particularly, listen-before-talk (LBT) rule ensures that operating nodes do not transmit over each other. While LBT is very important for operation in unlicensed bands, it calls for a revision in some of the NR baseline procedures. In NR-Unlicensed WID [1], the implications that LBT brings has been captured in a few of the objectives.

In RAN2 #105bis meeting [2], the following agreements related to the impact of UL LBT success/fail were reached:

- Adopt a mechanism in MAC spec to handle the UL LBT failure, where “consistent” UL LBT failures (at least for UL transmissions of SR, RACH, PUSCH) are used for problem detection

- R2 assumes that the configured grant timer is not started/restarted when configured grant is not transmitted due to LBT failure. PDU overwrite need to be avoided somehow.

- The configured grant timer is not started/restarted when UL LBT fails on PUSCH transmission for grant received by PDCCH addressed to CS-RNTI scheduling retransmission for configured grant

- The configured grant timer is not started/restarted when the UL LBT fails on PUSCH transmission for UL grant received by PDCCH addressed to C-RNTI, which indicates the same HARQ process configured for configured uplink grant

- Upon UL transmission on CG, bwp-InactivityTimer is restarted as today (at LBT success)

- Upon UL transmission on CG, sCellDeactivationTimer is restarted as today (at LBT success)

- dataInactivityTimer should be restarted when any MAC entity attempts transmission of a MAC SDU for DTCH logical channel, or DCCH logical channel regardless UL LBT outcome.

- drx-HARQ-RTT-TimerUL should be started/restarted regardless of the LBT outcome for PUSCH transmission with dynamic grant

- drx-HARQ-RTT-TimerUL should not be started/restarted when LBT fails for PUSCH transmission with configured grant

In this contribution, we express our views regarding potential issues that LBT failures could bring in various MAC procedures in NR-U operation.

# 2	Discussion

There are several MAC procedures that get affected due to possible LBT failure. Particularly, if LBT failures happen persistently, it would trigger timers or counters that eventually may lead to significant delay. The following provides a non-comprehensive list.

## 2.1 Scheduling Request

In the baseline specification, the SR counter and SR prohibit timer are maintained by MAC and depending on their status, an SR transmission may be initiated, prevented, or cancelled. The current SR procedure (SR initiation and prevention of subsequent SR transmission) are designed considering operation in licensed spectrum. For NR-U operation, it is vital to make MAC entity aware of whether PHY has been able to transmit the SR as instructed, it is beneficial to distinguish an LBT failure vs an unsuccessful transmission event. Due to these differences, in RAN2#105bis [2], the following agreements regarding SR procedure were reached:

- SR_COUNTER is increased only when SR is successfully transmitted

- As agreed in the SI phase, the sr-ProhibitTimer shall not prohibit SR transmissions due to SR that was not transmitted due to LBT failure.

## 2.2 Random Access

In the baseline specification, during random access procedure the preamble transmission counter, power ramping counter, RA response window and RA contention resolution timer are maintained by MAC and depending on their status, the random-access procedure progresses differently. Due to the differences in NR and NR-U operation, in RAN2#105bis [2], the following agreements regarding SR procedure were reached:

- The PREAMBLE_TRANSMISSION_COUNTER is not increased if the preamble is not transmitted due to LBT failure

- As earlier agreed, The POWER_RAMPING_COUNTER is not increased if the preamble is not transmitted due to LBT failure. For this purpose, LBT failure indication or equiv. (used for other LBT outcome dependencies in MAC) from PHY is used.

- Actual transmission for MSG1 (LBT success) is used for starting RAR window

- Either a) the ra-ContentionResolutionTimer is started regardless of the LBT outcome of msg3 transmission or b) ra-ContentionResolutionTimer is started only at successful LBT outcome of msg3 transmission + immediately the UE to restart from RACH resource selection if all MSG3 transmissions fail.

Given above latest agreement in RAN2#105b, we suggest that the ra-ContentionResolutionTimer is started only at successful LBT outcome of msg3 transmission. This is motivated by the fact that without transmission of Msg3, the initiation of the timer conveys no purpose.

## 2.3 Common LBT Status Indication

Considering above, an LBT failure may happen in preparation to transmit in PUCCH or PRACH resources. Related to LBT fail/success impact in configured grant (CG) transmission as captured in RAN2 #105b [2] agreements, LBT failure during PUSCH access should also be considered when treating persistent LBT failures. We recognize that occurrence of LBT failure also depends on the LBT category which should be considered appropriately.

An occurrence of LBT failure during access of any of above-listed physical channels should be considered in case a pattern of persistent LBT failures happen. While PHY layer would not be able to recognize such pattern, it can pass such indication to MAC.

Proposal 1: Consider a common “LBT status” signalling from PHY to MAC to enable MAC to recognize persistent LBT failure patterns. Inform RAN1 to accommodate such signalling.

## 2.4 Persistent LBT Failures

A persistent LBT failures during performing SR or RA procedures could lead to trigger RLF. This could happen due to following: 1) congested unlicensed channel due to presence of other NR-U or other technologies nodes, 2) presence of interfering nodes in the vicinity of the UE but hidden to gNB.

It is more straightforward and systematic to have a new mechanism for systematic LBT failure in MAC without changing the baseline procedures. Enabling MAC to recognize persistent LBT failures, appropriate solutions can be considered to address the problem. Ultimately, if persistent LBT failure reaches a threshold the UE may initiate RLF, but before that for a timelier reaction other measures could be considered.

Related to above consequences due to UL LBT failures, in RAN2 #105bis [2], the following agreement was reached:

- Adopt a mechanism in MAC spec to handle the UL LBT failure, where “consistent” UL LBT failures (at least for UL transmissions of SR, RACH, PUSCH) are used for problem detection

To identify a persistent LBT failures it may be required to initiate counter(s) and/or timer(s). A set of counter/timer would help to recognize how often and how persistently LBT failures happens. A related discussion then would be how to maintain these counter(s)/timer(s).

Proposal 2: Enable MAC to initiate and maintain counter(s) and/or timer(s) (FFS) in order to detect the occurrence of a persistent LBT failure.

A further distinction may be required on what LBT category faces persistent failures and what appropriate actions should be performed, such as LBT failure reporting at a later time.

Proposal 3: FFS to enable MAC to detect persistent LBT failures per category and whether additional FFS actions should be taken such as LBT failure reporting at a later time.

# 3	Conclusions

In this contribution, we highlighted our concerns regarding consequences that LBT could bring in several MAC procedures. We have proposed the following:

Proposal 1: Consider a common “LBT status” signalling from PHY to MAC to enable MAC to recognize persistent LBT failure patterns. Inform RAN1 to accommodate such signalling.

Proposal 2: Enable MAC to initiate and maintain counter(s) and/or timer(s) (FFS) in order to detect the occurrence of a persistent LBT failure.

Proposal 3: FFS to enable MAC to detect persistent LBT failures per category and whether additional FFS actions should be taken such as LBT failure reporting at a later time.

# References

RP-182878, “New WID on NR-based Access to Unlicensed Spectrum,” Qualcomm, RAN#82, December 2018

RAN2 #105b Chairman notes, Xi’an, China, RAN2#105b, April 2019