3GPP TSG-RAN WG2#106	Tdoc R2-1907309

Reno, Nevada, USA, 13th – 17th May 2019	Revision of R2-1903895

Agenda Item:	12.3.2.1

Source:	Ericsson

Title:	Data forwarding at reduced handover interruption

Document for:	Discussion, Decision

# Introduction

At the RAN#80 meeting in June 2018, a new Rel-16 WI was approved [1] with the purpose to further enhance LTE mobility by reducing handover interruption time and improve mobility robustness.

At the RAN2#105 meeting it was agreed to specify the “non-split bearer” candidate solution (a.k.a. the enhanced Make-Before-Break (MBB) solution) for the Rel-16 E-UTRA enhancements minimizing the interruption time during mobility.

This contribution addresses the following open issue identified at RAN2#105:

=> FFS How to do data forwarding (early/late, including handling the SN, security, CHO impact).

# Early or late start of DL data forwarding

Even if MBB and RACH-less HO (both concepts introduced in Rel-14) are combined in the same handover procedure, it is still not possible to accomplish seamless mobility, i.e. true 0ms HO interruption time, in all scenarios. In order to meet this target, the UE need to keep its connection with the source cell until it is able to receive and transmit user data in the target cell.

In principle, this means that the source eNB, after sending the RRCConnectionReconfiguration message (Handover Command) to the UE, continues to send DL packets to the UE until the UE is able to receive DL packets from the target eNB. In other words, the network needs to provide the UE with a seamless DL data stream throughout the handover execution phase.

To achieve seamless handover, the network needs to provide the UE with a continuous DL data stream throughout the handover execution phase.

Data forwarding can either be triggered at an early stage in the handover procedure as in a legacy handover (when receiving the HANDOVER REQUEST ACKNOWLEDGE message from the target eNB or at the point when sending the Handover Command to the UE), or data forwarding can be triggered at a later stage in the handover execution phase, e.g. when the UE performs random-access in the target cell. In the latter case the data forwarding is either triggered by the target eNB sending an indication over X2 to the source eNB, or by the UE transmitting an Uu message to the source eNB.

Important to consider here is that the target eNB can start DL transmission immediately when the UE has completed the handover to the target cell. Even if a UE with dual active protocol stacks is potentially able to continuously receive DL data from the source eNB after completing the handover in the target cell, the UE is most likely on the cell edge of the source cell, thus the radio conditions to/from the source cell would not be ideal, causing unnecessary delays/interruptions in the DL data flow to the UE.

Ideally, the UE starts receiving DL data from the target eNB immediately after completing the handover.

Starting data forwarding late in the handover execution phase, e.g. when the UE performs random-access in the target cell, benefits from the fact that a smaller amount of DL PDCP duplications will be sent and buffered in the target eNB compared to an early start of data forwarding. However, a difficult part with the late start of data forwarding is the time aspect since the start of data forwarding in the source eNB relies on a message either sent from the target eNB or from the UE when it accesses the target cell. On top of this, the X2 latency must be considered which in many deployments (using non-ideal backhaul) may exceed 10ms [2].

It means, if data forwarding is triggered by the target eNB sending an indication to the source eNB over X2, the target eNB may experience a round-trip delay beyond 20ms before it starts to receive DL PDCP SDUs from the source eNB.

On the other hand, if data forwarding is triggered by the UE sending a Uu message to the source eNB during the random-access procedure, weak radio conditions to/from the source cell may force the UE to re-send the message multiple times before it is successfully received by the source eNB, thus delaying the start of data forwarding. In even worse radio conditions, the message from the UE may not even be received by the source eNB.

A late start of data forwarding implies less DL PDCP duplications sent to the target eNB. On the other hand, a late start of data forwarding need to take the X2 transmission latency into account.

Starting data forwarding at an early stage (e.g. when sending the Handover Command to the UE) means that more DL PDCP duplications will be sent and buffered in the target eNB compared to a late start of data forwarding. Still, compared to a legacy handover, the number of forwarded and buffered packets in the target eNB at early data forwarding will not exceed the number of buffered packets in the target eNB at a legacy handover.

On the other hand, with the early start of data forwarding the target eNB can always start DL transmission immediately when the UE has completed the handover to the target cell.

With an early start of data forwarding the target eNB can always start DL transmission immediately when the UE has completed the handover to the target cell.

It is clear that the number of DL PDCP duplications buffered in the target eNB will increase with an early start of data forwarding and sending of duplicated packets on the radio interface will have a bad impact to user experience, thus it should be avoided.

However, this can be resolved if a PDCP duplication check is performed in the target eNB before transmitting the DL PDCP PDUs to the UE. The PDCP duplication check in the target eNB is based on the PDCP Status Report sent by the UE together with, or directly after, the transmission of the RRCConnectionReconfigurationComplete message (Handover Complete) in the target cell.

Given the DL PDCP duplication check is done in the target eNB, we think that an early start of data forwarding is more reliable compared to a late start of data forwarding. The gain from a late start of data forwarding (less DL PDCP duplications sent and buffered in the target eNB) is overshadowed by the X2 latency and the obvious risk of an interruption in the DL data stream to the UE.

RAN2 assumes early start of data forwarding when enhanced Make-Before-Break (MBB) is configured. Early start of data forwarding is equivalent to the legacy definition of data forwarding initiation.

# Forwarding of SN and HFN

A simple way for the source eNB to apply data forwarding is to forward all DL PDCP SDUs received from the Serving Gateway (SGW) in sequence to the target eNB, i.e. the source eNB does not apply any form of selective data forwarding.

A benefit of forwarding all DL PDCP SDUs in sequence to the target eNB is that it is sufficient to indicate the PDCP Sequence Number (SN) of the first forwarded DL PDCP SDU since the SNs of the subsequent SDUs will be implicitly known by the target eNB. The PDCP SN of the first SDU could be carried in the "PDCP PDU number" field of the GTP-U extension header (as already possible in a legacy handover), but since also the reference HFN for the COUNT preservation is needed in the target eNB, an alternative solution is to make use of the SN STATUS TRANSFER message.

However, sending the SN STATUS TRANSFER message while data forwarding to/from the UE is still active in the source eNB is an obvious deviation from legacy handover in which case the X2AP message is only sent after the source eNB considers the transmitter/receiver status to be frozen (i.e. when transmission/reception to/from the UE has stopped).

Obviously, if RAN2 considers the SN STATUS TRANSFER message as an adaptable solution for transfer of DL PDCP SN and HFN status to the target eNB when early data forwarding is applied, the conditions for sending the X2AP message when enhanced MBB is configured, need to be discussed and eventually approved in RAN3.

RAN2 to consider making use of SN STATUS TRANSFER message to transfer DL PDCP SN and HFN status to the target eNB when enhanced MBB is configured. Final decision is up to RAN3.

As in legacy, forwarding of DL PDCP SDUs from the source eNB will continue until the “end marker” packet is received from the SGW on the old path as a result of the DL path switch request triggered by the target eNB.

As in a legacy handover, DL data forwarding proceeds until the “end marker” packet is received from the SGW on the old path.

# Data forwarding of UL data

With the enhanced MBB solution, the UE continues to send UL PDCP PDUs to the source eNB after receiving the Handover Command, and until it has completed the handover to the target eNB.

In the source eNB, there are two alternative solutions how to handle the UL PDCP PDUs:

- The source eNB continues to send UL PDCP SDUs to the Serving Gateway (SGW) on the old S1-U path until the “end marker” packet is received from the SGW (as a result of the DL path switch request triggered by the target eNB), or;

- the source eNB starts forwarding UL PDCP SDUs to the target eNB for further transmission to the SGW on the new S1-U path.

If the UL packets are sent directly to the SGW on the old S1-U path (as before the Handover Command is sent) the transmission latency on X2 will of course be avoided. On the other hand, when the UE has stopped its UL transmission in the source cell, “another” SN STATUS TRANSFER message need to be sent to the target eNB (i.e. in addition to the SN STATUS TRANSFER message sent for transfer of DL PDCP SN and HFN status when early data forwarding is supported, see previous section) to transfer the UL PDCP SN receiver status and the HFN receiver status.

Furthermore, if the source eNB has undelivered UL PDCP SDUs (e.g. PDCP SDUs received out of sequence which have not been sent to the SGW on the old S1-U path), the source eNB need to forward these SDUs to the target eNB for in-sequence delivery on the new S1-U path. If not forwarded, the UE may be requested to re-send these packets when accessing the target cell.

If the source eNB after sending Handover Command to the UE, continues to send UL PDCP SDUs to the SGW on the old S1-U path, UL PDCP SN and HFN receiver status need to be transferred to the target eNB when the UE stops transmitting to the source eNB.

If, according to the second alternative above, each UL PDCP SDU is forwarded to the target eNB for further transmission to the SGW on the new S1-U path, the UL PDCP SN receiver status and HFN UL receiver status can be included in the same SN STATUS TRANSFER message carrying the DL PDCP SN and HFN status when early data forwarding is applied (see previous section).

If the source eNB after sending Handover Command to the UE, starts forwarding UL PDCP SDUs to the target eNB for further transmission to the SGW on the new S1-U path, UL and DL SN and HFN status can be included in the same SN STATUS TRANSFER message.

However, one obvious drawback with this solution is that it adds X2 latency to each single packet forwarded to the target eNB. On the other hand, the early transfer of UL PDCP SN and HFN receiver status admits the target eNB to process the UL PDCP PDUs immediately when the UE starts transmitting in the target cell.

On the other hand, “early data forwarding” of UL PDCP SDUs adds X2 latency to each single packet forwarded to the target eNB.

From a latency perspective, sending the UL PDCP SDUs directly to the SGW on the old S1-U path (as before the Handover Command is sent) seems more attractive, However, at some point the source eNB need to transfer UL PDCP SN and HFN status to the target eNB which may cause a short interruption in the UL PDCP processing in the target eNB if the information from the source eNB does not arrive in time.

Due to the latency issue, we have a slight preference to the first alternative (source eNB continues to send UL PDCP SDUs to the Serving Gateway (SGW) on the old S1-U path) but obviously there are pros and cons with both alternatives. Therefore, we propose the following:

As there are pros and cons with both alternatives (continue to send UL PDCP SDUs to the SGW on the old S1-U path or early forwarding of UL PDCP SDUs to the target eNB for further transmission to the SGW on the new S1-U path), RAN2 is requested to further evaluate the two alternatives.

# CHO impact to data forwarding

During the handover robustness discussions at the RAN2#105 meeting, following agreement was reached for packet forwarding:

RAN2 assumes late packet forwarding (i.e. not done immediately when the CHO target cells become prepared) could be more suitable for E-UTRAN CHO when there are multiple candidate target cells. In case of single prepared candidate target cell, early packet forwarding could be considered as an option. Detailed decisions require RAN3 study.

An LS was also sent from the same RAN2 meeting asking RAN3 to study data forwarding mechanisms suitable for Conditional Handover (CHO) in E-UTRAN and in NR [3].

As discussed in section 2, a late start of data forwarding suffers from X2 latency and the obvious risk of an interruption in the DL data flow from the network to the UE. With an early start of data forwarding, this risk is eliminated, thus the target eNB can always start DL transmission immediately when the UE has completed the handover to the target cell. However, when CHO and multiple candidate target cells are configured, early start of data forwarding implies that signaling resources will be used in vain, thus the signaling costs will increase with the number of candidate target cells.

Conditional Handover (CHO) and multiple candidate target cells in combination with early data forwarding implies an increased use of user plane and network signaling resources.

As a trade-off between signaling costs and minimizing DL transmission interruption, the decision to either make use of early or late data forwarding could be taken in the source eNB based on the number of candidate cells configured to the UE.

If for instance enhanced MBB is combined with CHO and only a single candidate cell is configured to the UE, then the source eNB could start early data forwarding as if only enhanced MBB is configured.

But if enhanced MBB is combined with CHO and several candidate cells are configured to the UE, then the source eNB may start data forwarding at a later stage in the handover execution phase, e.g. when indicated from the UE or from the target eNB.

For enhanced MBB combined with CHO, the decision in the source eNB to start early or late data forwarding could be based on the number of candidate cells configured to the UE.

# Conclusion

In this paper we made the following observations:

Observation 1	To achieve seamless handover, the network needs to provide the UE with a continuous DL data stream throughout the handover execution phase.

Observation 2	Ideally, the UE starts receiving DL data from the target eNB immediately after completing the handover.

Observation 3	A late start of data forwarding implies less DL PDCP duplications sent to the target eNB. On the other hand, a late start of data forwarding need to take the X2 transmission latency into account.

Observation 4	With an early start of data forwarding the target eNB can always start DL transmission immediately when the UE has completed the handover to the target cell.

Observation 5	If the source eNB after sending Handover Command to the UE, continues to send UL PDCP SDUs to the SGW on the old S1-U path, UL PDCP SN and HFN receiver status need to be transferred to the target eNB when the UE stops transmitting to the source eNB.

Observation 6	If the source eNB after sending Handover Command to the UE, starts forwarding UL PDCP SDUs to the target eNB for further transmission to the SGW on the new S1-U path, UL and DL SN and HFN status can be included in the same SN STATUS TRANSFER message.

Observation 7	On the other hand, “early data forwarding” of UL PDCP SDUs adds X2 latency to each single packet forwarded to the target eNB.

Observation 8	Conditional Handover (CHO) and multiple candidate target cells in combination with early data forwarding implies an increased use of user plane and network signaling resources.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 assumes early start of data forwarding when enhanced Make-Before-Break (MBB) is configured. Early start of data forwarding is equivalent to the legacy definition of data forwarding initiation.

Proposal 2	RAN2 to consider making use of SN STATUS TRANSFER message to transfer DL PDCP SN and HFN status to the target eNB when enhanced MBB is configured. Final decision is up to RAN3.

Proposal 3	As in a legacy handover, DL data forwarding proceeds until the “end marker” packet is received from the SGW on the old path.

Proposal 4	As there are pros and cons with both alternatives (continue to send UL PDCP SDUs to the SGW on the old S1-U path or early forwarding of UL PDCP SDUs to the target eNB for further transmission to the SGW on the new S1-U path), RAN2 is requested to further evaluate the two alternatives.

Proposal 5	For enhanced MBB combined with CHO, the decision in the source eNB to start early or late data forwarding could be based on the number of candidate cells configured to the UE.

# References

RP-181475, New Work Item on even further Mobility enhancement in E-UTRAN, RAN#80

3GPP TR 36.932, Scenarios and requirements for small cell enhancements for E-UTRA and E-UTRAN

R2-1905474, LS on Conditional Handover in E-UTRAN and NR, RAN2#105bis