3GPP TSG-RAN WG2 #106	 TDoc R2-1907362

# Reno, USA, 13th – 17th May 2019

Agenda Item: 	11.4.6

Source:	Ericsson

Title:	On the use of communication range at access stratum

Document for:	Discussion, Decision

# Introduction

In the latest TS 23.287, the delivery of communication range is defined as following:

TS 23.287 - 5.4.1.1: When groupcast or unicast mode of V2X communication over NR based PC5 is used, a Range parameter is associated with the QoS parameters for the V2X communication. The Range may be provided by V2X application layer or use a default value mapped from the service type based on configuration as defined in clause 5.1.2.1. The Range indicates the minimum distance that the QoS parameters need to be fulfilled. The Range parameter is passed to AS layer together with the QoS parameters for dynamic control.

This paper discusses how access stratum utilizes communication range parameter that associated with a V2X service and delivered from upper layers.

# Discussion

In our view, the actual communication range between the transmitter (TX) UE and the receiver (RX) UE can be either accurately measured or roughly estimated.

To accurately measure the communication distance, the TX UE and the RX UE must know each other’s GPS location at access stratum. On the other hand, that would require TX UE and RX UE to frequently exchange their GPS locations, either by adding it to SCI/data packet or by periodic messaging, which leads to extra overhead.

Alternatively, it is also possible to estimate the distance between the TX UE and RX UE

- Based on received signal power, e.g. lower received power indicates larger distance

- Based on zone ID information shared between TX UE and RX UE

Both approaches, i.e. received signal power based and zone ID based, provide reasonable distance estimation while sharing zone IDs between TX UE and RX UE would require extra signalling/overhead. Therefore, in our view estimating the distance based on received signal power is simpler and more efficient.

At access stratum, distance between TX UE and RX UE can be measured based on exchanged GPS location, or estimated based on received signal power or zone ID.

It is up to RAN1 to determine how communication distance is measured/estimated.

Given a communication range requirement, access stratum may try to fulfil it in a best effort fashion. For instance, communication range associated with a V2X service can be used for power control at TX UE side. This is particularly useful for groupcast communication where no CSIT is available.  For instance, the transmission power can be relatively high if the required communication range is large, such that received signal strength for UEs at the edge of the communication range of the group will not be too weak. Besides, if the required communication range is small, the transmission power can be low to reduce the interference to UEs outside the range.

Communication range can be an input for controlling the transmission power, particularly for groupcast. Details are left to RAN1 discussion.

Besides, it has been widely discussed that communication range can be used to facilitate HARQ procedure in SL groupcast, i.e. RX UEs who fail to receive the packet will send a NACK to the TX UE. The following relevant agreements are made in the recent RAN1 and RAN2 meetings.

RAN1#96 Agreements: - For sidelink groupcast, it is supported to use TX-RX distance and/or RSRP in deciding whether to send HARQ feedback. - Details to be discussed during WI phase, including whether the information on TX-RX distance is explicitly signaled or implicitly derived, whether/how this operation is related to resource allocation, accuracy of distance and/or RSRP, the aspects related to “and/or”, etc. - This feature can be disabled/enabled RAN2#105 Agreements 3: Any UEs configured to receive a group destination Layer 2 ID shall be allowed to receive the groupcast transmission, in regardless of whether it is within or out of the “minimum communication range”.

According to the agreements and to limit the traffic load caused by retransmission, TX UE can be configured to only response, i.e. schedule a retransmission, to those RX UEs that are estimated to be within the communication range. As such, the reliability of communication within the range can be sustained. Since the communication range can be estimated based on received signal power as in Proposal 1, TX UE may filter out HARQ feedbacks on the basis of the energy of the PSFCH and a HARQ feedback is considered only if the energy of the associated PSFCH is above a certain threshold.

TX UE may only consider HARQ feedbacks with energy of the associated PSFCH above a certain threshold. Details are left to RAN1 discussion.

Comparatively, for SL unicast, the extra value provided by communication range requirement is unclear. First of all, it is the channel quality that determines if a certain QoS, e.g. reliability and data rate, can be supported. For instance, it does not make much sense to establish a SL unicast QoS flow if two UEs are within a certain range but with poor channel quality.

For SL unicast, the motivation of introducing communication range parameter is not clear.

Communication range parameter is adopted for SL groupcast but not for SL unicast.

# Conclusions

In section 2 we made the following observations:

Observation 1	At access stratum, distance between TX UE and RX UE can be measured based on exchanged GPS location, or estimated based on received signal power or zone ID.

Observation 2	Communication range can be an input for controlling the transmission power, particularly for groupcast. Details are left to RAN1 discussion.

Observation 3	For SL unicast, the motivation of introducing communication range parameter is not clear.

Based on the discussion in section 1 we propose the following:

Proposal 1	It is up to RAN1 to determine how communication distance is measured/estimated.

Proposal 2	TX UE may only consider HARQ feedbacks with energy of the associated PSFCH above a certain threshold. Details are left to RAN1 discussion.

Proposal 3	Communication range parameter is adopted for SL groupcast but not for SL unicast.

# References