3GPP TSG-RAN WG2 #106	R2-1907469

Reno, USA, 13th – 17th May 2019

Agenda Item:	12.2.6

Source:	Ericsson

Title:	Text Proposal on RACH Report NB-IoT

Document for:	Discussion, Decision

# 1	Introduction

Regarding RACH reporting, the following was agreed in RAN2#105:

- Agreements - For NB-IoT RACH report, range of numberOfPreamblesSent is INTEGER (1..64). - contentionDetected is Boolean. - Initial CEL is included in NB-IoT RACH report. - Working assumption: A Boolean flag indicating EDT fallback (i.e., the UE started with EDT NPRACH resources and went through a fallback to non-EDT NPRACH resources) is included in NB-IoT RACH report. - Take the ASN.1 structure provided in the report (proposal 5) as baseline for NB-IoT running CR. - Initial CEL is included in eMTC RACH report. - Working assumption: A Boolean flag indicating EDT fallback (i.e., the UE started with EDT PRACH resources and went through a fallback to non-EDT PRACH resources) is included in eMTC RACH report. - Take the ASN.1 structure provided in the report (proposal 8) as baseline for eMTC running CR. - RAN2 aims on standardizing a single method of sending NB-IoT RACH report, i.e., avoid multiple solutions. - Proposed solutions should be discussed considering signalling overhead, UE power consumption, security aspects and other reports being considered in other email discussions. - NB-IoT RACH report includes the information from the latest successful RACH procedure. - NB-IoT RACH report information is discarded when the UE goes to IDLE. - FFS how to report

And the following agreements were made in RAN2#105bis:

Agreements - A Boolean flag indicating EDT fallback (i.e., the UE started with EDT NPRACH resources and went through a fallback to non-EDT NPRACH resources) is included in NB-IoT RACH report. - A Boolean flag indicating EDT fallback (i.e., the UE started with EDT PRACH resources and went through a fallback to non-EDT PRACH resources) is included in eMTC RACH report. - At least RACH report for NB-IoT based on UE information request/response procedure is supported. - It is up to the network to decide when to request for the RACH report.

In the highlighted parts above, the contents and the general mechanism of NB-IoT RACH report has been described. In this paper we provide text proposal on RACH reporting for NB-IoT according to the agreements above.

# 2	Discussion

As seen in the above agreements, RAN2 has agreed to use UEInformationRequest and UEInformationResponse procedure for RACH reporting in NB-IoT and the contents of the RACH report has also been agreed. We have provided a text proposal below to incorporate this agreement to TS 36.331 clauses 6.7.1 and 6.7.2. The text proposal contains Including ueInformationRequest into DL-DCCH-Message-NB, including ueInformationResponse into UL-DCCH-Message-NB, defining UEInformationRequest-NB message which contains rach-ReportReq to request the RACH report from UE and defining UEInformationResponse-NB message, which contains the RACH report information as has been agreed.

RAN2 to agree to below text proposal introducing UEInformationRequest and UEInformationRequest for NB-IoT to support RACH reporting.

# 3	Text Proposal

First change

#### –	DL-DCCH-Message-NB

The DL-DCCH-Message-NB class is the set of RRC messages that may be sent from the E-UTRAN to the UE on the downlink DCCH logical channel.

-- ASN1START

DL-DCCH-Message-NB ::= SEQUENCE {

message					DL-DCCH-MessageType-NB

}

DL-DCCH-MessageType-NB ::= CHOICE {

c1						CHOICE {

dlInformationTransfer-r13				DLInformationTransfer-NB,

rrcConnectionReconfiguration-r13		RRCConnectionReconfiguration-NB,

rrcConnectionRelease-r13				RRCConnectionRelease-NB,

securityModeCommand-r13					SecurityModeCommand,

ueCapabilityEnquiry-r13					UECapabilityEnquiry-NB,

rrcConnectionResume-r13					RRCConnectionResume-NB,

ueInformationRequest-r16				UEInformationRequest-NB,

spare1 NULL

},

messageClassExtension	SEQUENCE {}

}

-- ASN1STOP

Next change

#### –	UL-DCCH-Message-NB

The UL-DCCH-Message-NB class is the set of RRC messages that may be sent from the UE to the E-UTRAN on the uplink DCCH logical channel.

-- ASN1START

UL-DCCH-Message-NB ::= SEQUENCE {

message					UL-DCCH-MessageType-NB

}

UL-DCCH-MessageType-NB ::= CHOICE {

c1						CHOICE {

rrcConnectionReconfigurationComplete-r13	RRCConnectionReconfigurationComplete-NB,

rrcConnectionReestablishmentComplete-r13	RRCConnectionReestablishmentComplete-NB,

rrcConnectionSetupComplete-r13				RRCConnectionSetupComplete-NB,

securityModeComplete-r13					SecurityModeComplete,

securityModeFailure-r13						SecurityModeFailure,

ueCapabilityInformation-r13					UECapabilityInformation-NB,

ulInformationTransfer-r13					ULInformationTransfer-NB,

rrcConnectionResumeComplete-r13				RRCConnectionResumeComplete-NB,

ueInformationResponse-r16					UEInformationResponse-NB,

spare7 NULL,

spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

messageClassExtension	SEQUENCE {}

}

-- ASN1STOP

Next change

#### –	UEInformationRequest-NB

The UEInformationRequest-NB is the command used by E-UTRAN to retrieve information from the UE.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

UEInformationRequest message

-- ASN1START

UEInformationRequest-NB	::=				SEQUENCE {

rrc-TransactionIdentifier		RRC-TransactionIdentifier,

criticalExtensions				CHOICE {

c1								CHOICE {

ueInformationRequest-NB-r16			UEInformationRequest-NB-r16-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

UEInformationRequest-NB-r16-IEs ::=	SEQUENCE {

rach-ReportReq-r16					BOOLEAN,

nonCriticalExtension				SEQUENCE {}					OPTIONAL

}

-- ASN1STOP

UEInformationRequest field descriptions

rach-ReportReq This field is used to indicate whether the UE shall report information about the random access procedure.

Next change

#### –	UEInformationResponse-NB

The UEInformationResponse-NB message is used by the UE to transfer the information requested by the E-UTRAN.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

UEInformationResponse-NB message

-- ASN1START

UEInformationResponse-NB	::=		SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE {

ueInformationResponse-NB-r16		UEInformationResponse-NB-r16-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

UEInformationResponse-NB-r16-IEs ::=		SEQUENCE {

rach-Report-NB-r16							SEQUENCE {

numberOfPreamblesSent-r16				NumberOfPreamblesSent-r16,

initialCEL-r16							ENUMERATED {ce0, ce1, ce2},

contentionDetected-r16					BOOLEAN,

edt-Fallback-r16						BOOLEAN

}																OPTIONAL,

nonCriticalExtension					SEQUENCE {}						OPTIONAL

}

NumberOfPreamblesSent-r16 ::=			INTEGER (1..64)

-- ASN1STOP

UEInformationResponse field descriptions

contentionDetected This field is used to indicate that contention was detected for at least one of the transmitted preambles, see TS 36.321 [6].

edt-Fallback This field indicates whether the RACH procedure was started using EDT and then there was fallback to regular PRACH procedure.

initialCEL This field indicates the initial CE-level which was chosen for the first RACH preamble.

numberOfPreamblesSent This field is used to indicate the number of RACH preambles that were transmitted. Corresponds to parameter PREAMBLE_TRANSMISSION_COUNTER in TS 36.321 [6].

End of changes

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to agree to below text proposal introducing UEInformationRequest and UEInformationRequest for NB-IoT to support RACH reporting.