3GPP TSG-RAN WG2 #106	R2-1907554

Reno, USA, May 13th – 17th, 2019                          Revision of R2-1904724

Agenda Item	: 11.7.2.3 (NR_IIOT-Core)

Source	: LG Electronics Inc.

Title	: Discussion on Ethernet header compression

Document for	: Discussion and Decision

Introduction

In RAN2#105bis meeting, the following agreements are made

IETF see no issues for 3GPP to develop and maintain a ROHC profile. Also, it seems feasible in the time frame of Rel-16. We develop Ethernet header compression 100% in 3GPP TS (not by extending ROHC)

With the above agreements, in this contribution, we show our view on which Ethernet field should be compressed and how to update the context for the Ethernet header compression (hereinafter EHC).

Discussion

Which part of the Ethernet field is considered as a compression field for Ethernet header compression

During the SI phase, the majority companies think that only static part of the Ethernet header is considered for compression. In addition, among the static part of the Ethernet header, the PREAMBLE, SFD, and FCS fields do not need to be transmitted to the network as stated in TS 23.501. Then, it would be ok to consider only Destination address, Source address, ETHER TYPE and Q-TAGs for compression.

Proposal 1. The PREAMBLE, SFD, and FCS fields are not transmitted to the network.

Proposal 2. Destination address, Source address, ETHER TYPE and Q-TAGs are considered for compression.

In IEEE 802.3, it is addressed that the minimum Ethernet frame length is 64 bytes. Considering this, if the Ethernet frame length is smaller than 64 bytes, the padding is added in the Ethernet payload. Given that the padding does not have any information, the padding may not be required for transmission via Uu interface. Actually, the padding can be easily identified by checking the length field if the Ethernet header contains the length field.

However, we are not sure how much gain can be achieved by removing the padding. If the Ethernet header is 14 bytes (minimum size for Ethernet header) and the MTU size is 20 bytes (minimum size for IIOT traffic), the gain of the removing padding is maximized. On the other hand, if the Ethernet header (at most 30 bytes) is larger than 14 bytes and the IP header (at most 60 bytes) is contained in the Ethernet payload, the padding may be only one or two bytes. In this case, there would be marginal gain of removing the padding while increasing UE complexity.

In summary, the gain of padding removal is very limited, and we do not prefer to consider the padding in Ethernet payload for compression.

Proposal 3. The padding in Ethernet payload is not considered for compression.

Consideration of context update procedure

In order to successfully perform the compression and decompression, the compressor and decompressor should maintain the same context of the Ethernet header. For this, RAN2 should discuss how to deliver the context from compressor to decompressor.

The straightforward solution is that the compressor transmits the compressed packets only after the compressor is sufficiently confident that the decompressor successfully establishes header context.

The confidence can be achieved by transmitting multiple packets containing the full header information. After transmitting the multiple full header packets, the compressor (optimistically) assumes that the decompressor successfully establishes header context, and can transmit packets with compressed header.

How many full header packets should be transmitted to establish header context is up to compressor implementation. It can be negotiated between compressor and decompressor when EHC is configured.

Once the header context is established in the decompressor, the compressor can send compressed header packet from then on. However, the header context may need to be updated due to e.g. change of static fields. In this case, the compressor should send full header packet again to update the header context.

Proposal 4. The compressor transmits multiple full header packets to establish/update header context in the decompressor.

If feedback channel is available, the decompressor can send feedback to the compressor to inform that the header context is successfully established. In this case, the compressor stops transmitting full header packet when the feedback is received.

Proposal 5. The compressor stops transmitting full header packet when the feedback is received from the decompressor.

RAN2 need to further discuss which information needs to be included in the PDU compressed with EHC. The following parameters can be considered.

- Packet type

  - This parameter is used by the decompressor to identify whether the packet contains full header or compressed header.

- Context Identifier (CID)

  - This parameter is used by the decompressor to identify to which context the packet belongs. For full header packet, the decompressor updates the header context indicated by the CID. For compressed header packet, the decompressor decompresses the packet using the context indicated by the CID.

- Cyclic Redundancy Check (CRC)

  - This parameter is used by the decompressor to verify whether the received full header has any damage. This parameter is included only in the full header packet.

Proposal 6. At least the packet type, CID, and CRC are included in the PDU compressed with EHC.

Conclusion

In this contribution, we show our view on which Ethernet field should be compressed and how to update the context for the Ethernet header compression. Based on the above discussion, we propose followings.

Proposal 1. The PREAMBLE, SFD, and FCS fields are not transmitted to the network.

Proposal 2. Destination address, Source address, ETHER TYPE and Q-TAGs are considered for compression.

Proposal 3. The padding in Ethernet payload is not considered for compression.

Proposal 4. The compressor transmits multiple full header packets to establish/update header context in the decompressor.

Proposal 5. The compressor stops transmitting full header packet when the feedback is received from the decompressor.

Proposal 6. At least the packet type, CID, and CRC are included in the PDU compressed with EHC.