3GPP TSG-RAN WG2 Meeting #106	 R2-1908132

# Reno, USA, 13th to 17th May, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 3982 | rev | 1 | Current version: | 15.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Capture NR agreements in eLTE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_5GCN_connect-Core |  |  |  |  |  | Date: |  |  | 2019-05-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To capture below NR agreements: 1 In R2-1907323: - For RRCConnectionRelease with waitTime, barring alleviation should not be performed triggered by stop of T309 or T302, since T302 is started and it should prevent the alleviation. - For RRCConnectionRelease without waitTime, barring alleviation should be performed triggered by stop of T309 or T302. 2 In R2-1908317 to apply (delta configuration with) the received rrc-InactiveConfig, it is not clear whether the UE uses rrc-InactiveConfig,in the UE configuration or in the UE inactive AS context. 3 In R2-1906079, clarify “The network does not configure rrc_InactiveConfig when the network redirect the UE to an inter-RAT carrier frequency.” 4 In R2-1908453, for RNAU the resumeCause is set to emergency if an emergency service is ongoing. 5 In R2-1908318, When the UE enters RRC_IDLE from RRC_INACTIVE, if stored, the UE discards the cell reselection priority information provided by the dedicated signalling, and if running, the UE stops T320; 6 in R2-1908319, Add a requirement that UE ignores all fields in the RRC release message when received without security except wait timer. 7 In R2-1908338, clarify that in case of T302 expiry all access categories are alleviated for which T390 is not running 8 in R2-1908340, in 5.3.16.2, specify that UE shall inform upper layer the emergency service is barred if access attempt for the access category '2' is failed. 9 In R2-1908113, the RRCResume procedure is updated to ensure that resumeCause field is set for all NAS triggered access attempts, regardless if an acess category and access identity(ies) are provided or not. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1 In 5.3.8.3, 5.3.8.7, 5.3.12, Move the stop of T309 or T302 after the checking whether RRCConnectionRelease with waitTime; (R2-1907323) 2 R2-19083171) remove the storage of rrc-InactiveConfig,in inactive AS context in 5.3.8.7 2) release rrc-InactiveConfig,when going to RRC_IDLE (5.3.12) 3) release the rrc-InactiveConfig,except the ran-NotificationAreaInfo upon reception of RRCConnectionResume (5.3.3.4a) 3 In 6.2.2, clarify “The network does not configure rrc_InactiveConfig when the network redirect the UE to an inter-RAT carrier frequency.” (R2-1906079) 4 In 5.3.3.2, for RNAU the resumeCause is set to emergency if an emergency service is ongoing. (R2-1908453) 5 R2-1908318 - When the UE enters RRC_IDLE from RRC_INACTIVE, if stored, the UE discards the cell reselection priority information provided by the dedicated signalling, and if running, the UE stops T320 (5.3.12) - In the timer description in 7.3.1, stop condition of T320 was modified. 6 in 5.3.8.3, add a requirement that UE ignores all fields in the RRC release message when received without security except wait timer. (R2-1908319) 7 In 5.3.16.4, clarify that in case of T302 expiry all access categories are alleviated for which T309 is not running (R2-1908338) 8 in 5.3.16.2, specify that UE shall inform upper layer the emergency service is barrd if access attempt for the access category '2' is failed. (R2-1908340) 9 in 5.3.3.2, the RRCResume procedure is updated to ensure that resumeCause field is set for all NAS triggered access attempts, regardless if an acess category and access identity(ies) are provided or not. (R2-1908113) Impact Analysis: Impacted functionality: E-UTRA/5GC related features. Inter-operability: See impact analysis from individual NR CRs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | See analysis from individual NR CRs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.2, 5.3.3.4a, 5.3.8.3, 5.3.8.7, 5.3.12, 5.3.16.2, 5.3.16.4, 6.2.2, 7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 5.3.3.2	Initiation

The UE initiates the procedure when upper layers request establishment or resume of an RRC connection while the UE is in RRC_IDLE or when upper layers request resume of an RRC connection or RRC layer requests resume of an RRC connection for, e.g. RNAU or reception of RAN paging while the UE is in RRC_INACTIVE.

Except for NB-IoT, upon initiation of the procedure, if the UE is connected to EPC, the UE shall:

1>	if SystemInformationBlockType2 includes ac-BarringPerPLMN-List and the ac-BarringPerPLMN-List contains an AC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]):

2>	select the AC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers;

2>	in the remainder of this procedure, use the selected AC-BarringPerPLMN entry (i.e. presence or absence of access barring parameters in this entry) irrespective of the common access barring parameters included in SystemInformationBlockType2;

1>	else

2>	in the remainder of this procedure use the common access barring parameters (i.e. presence or absence of these parameters) included in SystemInformationBlockType2;

1>	if SystemInformationBlockType2 contains acdc-BarringPerPLMN-List and the acdc-BarringPerPLMN-List contains an ACDC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35]):

2>	select the ACDC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers;

2>	in the remainder of this procedure, use the selected ACDC-BarringPerPLMN entry for ACDC barring check (i.e. presence or absence of access barring parameters in this entry) irrespective of the acdc-BarringForCommon parameters included in SystemInformationBlockType2;

1>	else:

2>	in the remainder of this procedure use the acdc-BarringForCommon (i.e. presence or absence of these parameters) included in SystemInformationBlockType2 for ACDC barring check;

1>	if upper layers indicate that the RRC connection is subject to EAB (see TS 24.301 [35]):

2>	if the result of the EAB check, as specified in 5.3.3.12, is that access to the cell is barred:

3>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that EAB is applicable, upon which the procedure ends;

1>	if upper layers indicate that the RRC connection is subject to ACDC (see TS 24.301 [35]), SystemInformationBlockType2 contains BarringPerACDC-CategoryList, and acdc-HPLMNonly indicates that ACDC is applicable for the UE:

2>	if the BarringPerACDC-CategoryList contains a BarringPerACDC-Category entry corresponding to the ACDC category selected by upper layers:

3>	select the BarringPerACDC-Category entry corresponding to the ACDC category selected by upper layers;

2>	else:

3>	select the last BarringPerACDC-Category entry in the BarringPerACDC-CategoryList;

2>	stop timer T308, if running;

2>	perform access barring check as specified in 5.3.3.13, using T308 as "Tbarring" and acdc-BarringConfig in the BarringPerACDC-Category as "ACDC barring parameter";

2>	if access to the cell is barred:

3>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring is applicable due to ACDC, upon which the procedure ends;

1>	else if the UE is establishing the RRC connection for mobile terminating calls:

2>	if timer T302 is running:

3>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile terminating calls is applicable, upon which the procedure ends;

1>	else if the UE is establishing the RRC connection for emergency calls:

2>	if SystemInformationBlockType2 includes the ac-BarringInfo:

3>	if the ac-BarringForEmergency is set to TRUE:

4>	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to TS 22.011 [10] and TS 23.122 [11]:

NOTE 1:	ACs 12, 13, 14 are only valid for use in the home country and ACs 11, 15 are only valid for use in the HPLMN/ EHPLMN.

5>	if the ac-BarringInfo includes ac-BarringForMO-Data, and for all of these valid Access Classes for the UE, the corresponding bit in the ac-BarringForSpecialAC contained in ac-BarringForMO-Data is set to one:

6>	consider access to the cell as barred;

4>	else:

5>	consider access to the cell as barred;

2>	if access to the cell is barred:

3>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication, upon which the procedure ends;

1>	else if the UE is establishing the RRC connection for mobile originating calls:

2>	perform access barring check as specified in 5.3.3.11, using T303 as "Tbarring" and ac-BarringForMO-Data as "AC barring parameter";

2>	if access to the cell is barred:

3>	if SystemInformationBlockType2 includes ac-BarringForCSFB or the UE does not support CS fallback:

4>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating calls is applicable, upon which the procedure ends;

3>	else (SystemInformationBlockType2 does not include ac-BarringForCSFB and the UE supports CS fallback):

4>	if timer T306 is not running, start T306 with the timer value of T303;

4>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating calls and mobile originating CS fallback is applicable, upon which the procedure ends;

1>	else if the UE is establishing the RRC connection for mobile originating signalling:

2>	perform access barring check as specified in 5.3.3.11, using T305 as "Tbarring" and ac-BarringForMO-Signalling as "AC barring parameter";

2>	if access to the cell is barred:

3>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating signalling is applicable, upon which the procedure ends;

1>	else if the UE is establishing the RRC connection for mobile originating CS fallback:

2>	if SystemInformationBlockType2 includes ac-BarringForCSFB:

3>	perform access barring check as specified in 5.3.3.11, using T306 as "Tbarring" and ac-BarringForCSFB as "AC barring parameter";

3>	if access to the cell is barred:

4>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating CS fallback is applicable, due to ac-BarringForCSFB, upon which the procedure ends;

2>	else:

3>	perform access barring check as specified in 5.3.3.11, using T306 as "Tbarring" and ac-BarringForMO-Data as "AC barring parameter";

3>	if access to the cell is barred:

4>	if timer T303 is not running, start T303 with the timer value of T306;

4>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating CS fallback and mobile originating calls is applicable, due to ac-BarringForMO-Data, upon which the procedure ends;

1>	else if the UE is establishing the RRC connection for mobile originating MMTEL voice, mobile originating MMTEL video, mobile originating SMSoIP or mobile originating SMS:

2>	if the UE is establishing the RRC connection for mobile originating MMTEL voice and SystemInformationBlockType2 includes ac-BarringSkipForMMTELVoice; or

2>	if the UE is establishing the RRC connection for mobile originating MMTEL video and SystemInformationBlockType2 includes ac-BarringSkipForMMTELVideo; or

2>	if the UE is establishing the RRC connection for mobile originating SMSoIP or SMS and SystemInformationBlockType2 includes ac-BarringSkipForSMS:

3>	consider access to the cell as not barred;

2>	else:

3>	if establishmentCause received from higher layers is set to mo-Signalling (including the case that mo-Signalling is replaced by highPriorityAccess according to TS 24.301 [35] or by mo-VoiceCall according to the subclause 5.3.3.3):

4>	perform access barring check as specified in 5.3.3.11, using T305 as "Tbarring" and ac-BarringForMO-Signalling as "AC barring parameter";

4>	if access to the cell is barred:

5>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating signalling is applicable, upon which the procedure ends;

3>	if establishmentCause received from higher layers is set to mo-Data (including the case that mo-Data is replaced by highPriorityAccess according to TS 24.301 [35] or by mo-VoiceCall according to the subclause 5.3.3.3):

4>	perform access barring check as specified in 5.3.3.11, using T303 as "Tbarring" and ac-BarringForMO-Data as "AC barring parameter";

4>	if access to the cell is barred:

5>	if SystemInformationBlockType2 includes ac-BarringForCSFB or the UE does not support CS fallback:

6>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating calls is applicable, upon which the procedure ends;

5>	else (SystemInformationBlockType2 does not include ac-BarringForCSFB and the UE supports CS fallback):

6>	if timer T306 is not running, start T306 with the timer value of T303;

6>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring for mobile originating calls and mobile originating CS fallback is applicable, upon which the procedure ends;

Upon initiation of the procedure, if the UE is connected to 5GC, the UE shall:

1>	if the upper layers provide an Access Category and one or more Access Identities upon requesting establishment of an RRC connection:

2>	perform the unified access control procedure as specified in 5.3.16 using the Access Category and Access Identities provided by upper layers;

3>	if the access attempt is barred, the procedure ends;

1>	if the resumption of the RRC connection is triggered by response to NG-RAN paging:

2>	select '0' as the Access Category;

2>	perform the unified access control procedure as specified in 5.3.16 using the selected Access Category and one or more Access Identities provided by upper layers;

3>	if the access attempt is barred, the procedure ends;

1>	else if the resumption of the RRC connection is triggered by upper layers:

2>	if the upper layers provide an Access Category and one or more Access Identities:

3>	perform the unified access control procedure as specified in 5.3.16 using the Access Category and Access Identities provided by upper layers;

4>	if the access attempt is barred, the procedure ends;

2>	set the resumeCause in accordance with the information received from upper layers;

1>	else if the resumption of the RRC connection is triggered due to an RNAU:

2>	if an emergency service is ongoing:

3>	select '2' as the Access Category;

3> set the resumeCause to emergency;

2>	else:

3>	select '8' as the Access Category;

2>	perform the unified access control procedure as specified in 5.3.16 using the selected Access Category and one or more Access Identities to be applied as specified in TS 24.501 [95];

3>	if the access attempt is barred:

4>	set the variable pendingRnaUpdate to 'TRUE';

4>	the procedure ends;

Except for NB-IoT, upon initiating the procedure, if connected to EPC or 5GC, the UE shall:

1>	if the UE is resuming an RRC connection from a suspended RRC connection or from RRC_INACTIVE:

2>	if the UE is resuming an RRC connection from a suspended RRC connection:

3>	if the UE was configured with EN-DC:

4>	perform EN-DC release, as specified in TS 38.331 [82], clause 5.3.5.10;

2>	release the MCG SCell(s), if configured, in accordance with 5.3.10.3a;

2>	release powerPrefIndicationConfig, if configured and stop timer T340, if running;

2>	release reportProximityConfig and clear any associated proximity status reporting timer;

2>	release obtainLocationConfig, if configured;

2>	release idc-Config, if configured;

2>	release sps-AssistanceInfoReport, if configured;

2>	release measSubframePatternPCell, if configured;

2>	release the entire SCG configuration, if configured, except for the DRB configuration (as configured by drb-ToAddModListSCG);

2>	release naics-Info for the PCell, if configured;

2>	release the LWA configuration, if configured, as described in 5.6.14.3;

2>	release the LWIP configuration, if configured, as described in 5.6.17.3;

2>	release bw-PreferenceIndicationTimer, if configured and stop timer T341, if running;

2>	release delayBudgetReportingConfig, if configured and stop timer T342, if running;

2>	release ailc-BitConfig, if configured;

2>	release uplinkDataCompression, if configured;

NOTE 1a:	The parameters and configurations are released from the UE Inactive AS context if the UE is resuming an RRC connection from RRC_INACTIVE.

1>	apply the default physical channel configuration as specified in 9.2.4;

1>	apply the default semi-persistent scheduling configuration as specified in 9.2.3;

1>	apply the default MAC main configuration as specified in 9.2.2;

1>	apply the CCCH configuration as specified in 9.1.1.2;

1>	apply the timeAlignmentTimerCommon included in SystemInformationBlockType2;

1>	start timer T300;

1>	if the UE is resuming an RRC connection from a suspended RRC connection:

2>	initiate transmission of the RRCConnectionResumeRequest message in accordance with 5.3.3.3a;

1>	else if the UE is resuming an RRC connection from RRC_INACTIVE:

2>	stop T380, if running;

2>	set the variable pendingRnaUpdate to 'FALSE';

2>	initiate transmission of the RRCConnectionResumeRequest message in accordance with 5.3.3.3a;

1>	else:

2>	if stored, discard the UE AS context and resumeIdentity;

2>	if the UE is initiating CP-EDT in accordance with conditions in 5.3.3.1b:

3>	initiate transmission of the RRCEarlyDataRequest message in accordance with 5.3.3.3b;

2>	else:

3>	initiate transmission of the RRCConnectionRequest message in accordance with 5.3.3.3;

NOTE 2:	Upon initiating the connection establishment procedure, the UE is not required to ensure it maintains up to date system information applicable only for UEs in RRC_IDLE state or UEs in RRC_INACTIVE. However, the UE needs to perform system information acquisition upon cell re-selection.

For NB-IoT, upon initiation of the procedure, the UE shall:

1>	if the UE is establishing or resuming the RRC connection for mobile originating exception data; or

1>	if the UE is establishing or resuming the RRC connection for mobile originating data; or

1>	if the UE is establishing or resuming the RRC connection for delay tolerant access; or

1>	if the UE is establishing or resuming the RRC connection for mobile originating signalling;

2>	perform access barring check as specified in 5.3.3.14;

2>	if access to the cell is barred:

3>	inform upper layers about the failure to establish the RRC connection or failure to resume the RRC connection with suspend indication and that access barring is applicable, upon which the procedure ends;

1>	apply the default physical channel configuration as specified in 9.2.4;

1>	apply the default MAC main configuration as specified in 9.2.2;

1>	apply the CCCH configuration as specified in 9.1.1.2;

1>	start timer T300;

1>	if the UE is establishing an RRC connection:

2>	if stored, discard the UE AS context and resumeIdentity;

2>	if the UE is initiating CP-EDT in accordance with conditions in 5.3.3.1b:

3>	initiate transmission of the RRCEarlyDataRequest message in accordance with 5.3.3.3b;

2>	else:

3>	initiate transmission of the RRCConnectionRequest message in accordance with 5.3.3.3;

1>	else if the UE is resuming an RRC connection:

2>	release schedulingRequestConfig, if configured;

2>	initiate transmission of the RRCConnectionResumeRequest message in accordance with 5.3.3.3a;

NOTE 3:	Upon initiating the connection establishment or resumption procedure, the UE is not required to ensure it maintains up to date system information applicable only for UEs in RRC_IDLE state. However, the UE needs to perform system information acquisition upon cell re-selection.

NOTE 4:	For EDT, upon initiating the connection establishment or resumption procedure, it is up to UE implementation whether to continue cell re-selection related measurements as well as cell re-selection evaluation and, if the conditions for cell re-selection are fulfilled, whether to perform cell re-selection as specified in 5.3.3.5.

#### 5.3.3.4a	Reception of the RRCConnectionResume by the UE

The UE shall:

1>	stop timer T300;

1>	if T309 is running:

2>	stop timer T309 for all access categories;

2>	perform the actions as specified in 5.3.16.4.

1>	stop T380 if running;

1>	except if the RRCConnectionResume is received in response to an RRCConnectionResumeRequest for EDT:

2>	if resuming an RRC connection from a suspended RRC connection:

3>	restore the PDCP state and re-establish PDCP entities for SRB2, if configured with E-UTRA PDCP, and for all DRBs that are configured with E-UTRA PDCP;

3>	if drb-ContinueROHC is included:

4>	indicate to lower layers that stored UE AS context is used and that drb-ContinueROHC is configured;

4>	continue the header compression protocol context for the DRBs configured with the header compression protocol;

3>	else:

4>	indicate to lower layers that stored UE AS context is used;

4>	reset the header compression protocol context for the DRBs configured with the header compression protocol;

3>	discard the stored UE AS context and resumeIdentity;

2>	else if the RRCConnectionResume message includes the fullConfig (for resuming an RRC connection from RRC_INACTIVE):

3>	perform the radio configuration procedure as specified in 5.3.5.8;

2>	else (for resuming an RRC connection from RRC_INACTIVE):

3>	restore the physical layer configuration, the MAC configuration, the RLC configuration and the PDCP configuration from the stored UE Inactive AS context;

3>	if drb-ContinueROHC is included:

4>	indicate to lower layers that drb-ContinueROHC is configured;

3>	discard the stored UE Inactive AS context;

3>	release the rrc-InactiveConfig, except ran-NotificationAreaInfo;

1>	else:

2>	discard the stored UE AS context and resumeIdentity;

1>	perform the radio resource configuration procedure in accordance with the received radioResourceConfigDedicated and as specified in 5.3.10;

NOTE 1:	When performing the radio resource configuration procedure, for the physical layer configuration and the MAC Main configuration, the restored RRC configuration from the stored UE AS context is used as basis for the reconfiguration.

1>	if the received RRCConnectionResume message includes the sk-Counter:

2>	perform key update procedure as specified in TS 38.331 [82], clause 5.3.5.8;

1>	if the received RRCConnectionResume message includes the nr-RadioBearerConfig1:

2>	perform radio bearer configuration as specified in TS 38.331 [82], clause 5.3.5.6;

1>	if the received RRCConnectionResume message includes the nr-RadioBearerConfig2:

2>	perform radio bearer configuration as specified in TS 38.331 [82], clause 5.3.5.6;

1>	except if the RRCConnectionResume is received in response to an RRCConnectionResumeRequest for EDT:

2>	resume SRB2 and all DRBs, if any, including RBs configured with NR PDCP;

1>	if stored, discard the cell reselection priority information provided by the idleModeMobilityControlInfo or inherited from another RAT;

1>	if stored, discard the dedicated offset provided by the redirectedCarrierOffsetDedicated;

1>	if the RRCConnectionResume message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	if T302 is running:

2>	stop timer T302;

2>	if the UE is connected to 5GC:

3>	perform the actions as specified in 5.3.16.4;

1>	stop timer T303, if running;

1>	stop timer T305, if running;

1>	stop timer T306, if running;

1>	stop timer T308, if running;

1>	perform the actions as specified in 5.3.3.7;

1>	stop timer T320, if running;

1>	stop timer T350, if running;

1>	perform the actions as specified in 5.6.12.4;

1>	stop timer T360, if running;

1>	stop timer T322, if running;

1>	if the RRCConnectionResume is received in response to an RRCConnectionResumeRequest for EDT or an RRCConnectionResumeRequest from RRC_INACTIVE:

2>	ignore the nextHopChainingCount value indicated in the RRCConnectionResume message;

1>	else:

2>	if resuming an RRC connection from a suspended RRC connection:

3>	update the KeNB key based on the KASME key to which the current KeNB is associated, using the nextHopChainingCount value indicated in the RRCConnectionResume message, as specified in TS 33.401 [32];

3>	store the nextHopChainingCount value;

3>	derive the KRRCint key associated with the previously configured integrity algorithm, as specified in TS 33.401 [32];

3>	request lower layers to verify the integrity protection of the RRCConnectionResume message, using the previously configured algorithm and the KRRCint key;

3>	if the integrity protection check of the RRCConnectionResume message fails:

4>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other', upon which the procedure ends;

3>	derive the KRRCenc key and the KUPenc key associated with the previously configured ciphering algorithm, as specified in TS 33.401 [32];

3>	configure lower layers to resume integrity protection using the previously configured algorithm and the KRRCint key immediately, i.e., integrity protection shall be applied to all subsequent messages received and sent by the UE;

3>	configure lower layers to resume ciphering and to apply the ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE;

1>	enter RRC_CONNECTED;

1>	indicate to upper layers that the suspended RRC connection has been resumed;

1>	stop the cell re-selection procedure;

1>	consider the current cell to be the PCell;

1>	set the content of RRCConnectionResumeComplete message as follows:

2>	set the selectedPLMN-Identity to the PLMN selected by upper layers (see TS 23.122 [11], TS 24.301 [35] for E-UTRA/EPC and TS 24.501 [95] for E-UTRA/5GC) from the PLMN(s) included in the plmn-IdentityList in SystemInformationBlockType1;

2>	set the dedicatedInfoNAS to include the information received from upper layers;

2>	except for NB-IoT:

3>	if resuming an RRC connection from a suspended RRC connection:

4>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

5>	include rlf-InfoAvailable;

4>	if the UE has MBSFN logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

5>	include logMeasAvailableMBSFN;

4>	else if the UE has logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

5>	include logMeasAvailable;

4>	if the UE has Bluetooth logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

5>	include logMeasAvailableBT;

4>	if the UE has WLAN logged measurements available and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

5>	include logMeasAvailableWLAN;

4>	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

5>	include connEstFailInfoAvailable;

4>	include the mobilityState and set it to the mobility state (as specified in TS 36.304 [4]) of the UE just prior to entering RRC_CONNECTED state;

4>	if the SIB2 contains idleModeMeasurements, and the UE has IDLE mode measurement information available in VarMeasIdleReport:

5>	include the idleMeasAvailable;

4>	stop T331, if running;

4>	if the UE has flight path information available:

5>	include flightPathInfoAvailable;

3>	if the UE supports storage of mobility history information and the UE has mobility history information available in VarMobilityHistoryReport:

4>	include mobilityHistoryAvail;

2>	for NB-IoT:

3>	if the UE supports serving cell idle mode measurements reporting and servingCellMeasInfo is present in SystemInformationBlockType2-NB:

4>	set the measResultServCell to include the measurements of the serving cell;

NOTE 2:	The UE includes the latest results of the serving cell measurements as used for cell selection/ reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 36.133 [16].

1>	submit the RRCConnectionResumeComplete message to lower layers for transmission;

1>	the procedure ends.

#### 5.3.8.3	Reception of the RRCConnectionRelease by the UE

The UE shall:

1>	except for NB-IoT, BL UEs or UEs in CE, delay the following actions defined in this sub-clause 60 ms from the moment the RRCConnectionRelease message was received or optionally when lower layers indicate that the receipt of the RRCConnectionRelease message has been successfully acknowledged, whichever is earlier;

1>	for BL UEs or UEs in CE, delay the following actions defined in this sub-clause 1.25 seconds from the moment the RRCConnectionRelease message was received or optionally when lower layers indicate that the receipt of the RRCConnectionRelease message has been successfully acknowledged, whichever is earlier;

1>	for NB-IoT, delay the following actions defined in this sub-clause 10 seconds from the moment the RRCConnectionRelease message was received or optionally when lower layers indicate that the receipt of the RRCConnectionRelease message has been successfully acknowledged, whichever is earlier.

NOTE:	For BL UEs, UEs in CE and NB-IoT, when STATUS reporting, as defined in TS 36.322 [7], has not been triggered and the UE has sent positive HARQ feedback (ACK), as defined in TS 36.321 [6], the lower layers can be considered to have indicated that the receipt of the RRCConnectionRelease message has been successfully acknowledged.

1>	stop T380, if running;

1>	if the RRCConnectionRelease message is received in response to an RRCConnectionResumeRequest for EDT:

2>	indicate to upper layers that the suspended RRC connection has been resumed;

2>	discard the stored UE AS context and resumeIdentity;

2>	stop timer T300;

2>	stop timer T302, if running;

2>	stop timer T303, if running;

2>	stop timer T305, if running;

2>	stop timer T306, if running;

2>	stop timer T308, if running;

2>	perform the actions as specified in 5.3.3.7;

2>	stop timer T320, if running;

2>	stop timer T322, if running;

1>	if the security is not activated and if UE is connected to 5GC:

2> ignore any field included in RRCConnectionRelease message except waitTime;

2>	perform the actions upon leaving RRC_CONNECTED or RRC_INACTIVE as specified in 5.3.12 with the release cause 'other' upon which the procedure ends;

1>	if the RRCConnectionRelease message includes redirectedCarrierInfo indicating redirection to geran; or

1>	if the RRCConnectionRelease message includes idleModeMobilityControlInfo including freqPriorityListGERAN:

2>	if AS security has not been activated; and

2>	if upper layers indicate that redirect to GERAN without AS security is not allowed:

3>	ignore the content of the RRCConnectionRelease;

3>	perform the actions upon leaving RRC_CONNECTED or RRC_INACTIVE as specified in 5.3.12, with release cause 'other', upon which the procedure ends;

1>	if AS security has not been activated:

2>	ignore the content of redirectedCarrierInfo, if included and indicating redirection to nr;

2>	ignore the content of idleModeMobilityControlInfo, if included and including freqPriorityListNR;

2>	if the UE ignores the content of redirectedCarrierInfo or of idleModeMobilityControlInfo:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other', upon which the procedure ends;

1>	if the RRCConnectionRelease message includes redirectedCarrierInfo indicating redirection to eutra and if UE is connected to 5GC:

2>	if cn-Type is included:

3>	after the cell selection, indicate the available CN Type(s) and the received cn-Type to upper layers;

NOTE 1:	Handling the case if the E-UTRA cell selected after the redirection does not support the core network type specified by the cn-Type, is up to UE implementation.

1>	if the RRCConnectionRelease message includes the idleModeMobilityControlInfo:

2>	store the cell reselection priority information provided by the idleModeMobilityControlInfo;

2>	if the t320 is included:

3>	start timer T320, with the timer value set according to the value of t320;

1>	else:

2>	apply the cell reselection priority information broadcast in the system information;

1>	if the RRCConnectionRelease message includes the measIdleConfig:

2>	clear VarMeasIdleConfig and VarMeasIdleReport;

2>	store the received measIdleDuration in VarMeasIdleConfig;

2>	start T331 with the value of measIdleDuration;

2>	if the measIdleConfig contains measIdleCarrierListEUTRA:

3>	store the received measIdleCarrierListEUTRA in VarMeasIdleConfig;

3>	start performing idle mode measurements as specified in 5.6.20;

NOTE 2:	If the measIdleConfig does not contain measIdleCarrierListEUTRA, UE may receive measIdleCarrierListEUTRA as specified in 5.2.2.12.

1>	for NB-IoT, if the RRCConnectionRelease message includes the redirectedCarrierInfo:

2>	if the redirectedCarrierOffsetDedicated is included in the redirectedCarrierInfo:

3>	store the dedicated offset for the frequency in redirectedCarrierInfo;

3>	start timer T322, with the timer value set according to the value of T322 in redirectedCarrierInfo;

1>	if the releaseCause received in the RRCConnectionRelease message indicates loadBalancingTAURequired:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'load balancing TAU required';

1>	else if the releaseCause received in the RRCConnectionRelease message indicates cs-FallbackHighPriority:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'CS Fallback High Priority';

1>	else:

2>	if the waitTime is present:

3>	start timer T302, with the timer value set according to the waitTime;

3>	inform the upper layer that access barring is applicable for all access categories except categories '0' and '2';

2>	if the extendedWaitTime is present; and

2>	if the UE supports delay tolerant access or the UE is a NB-IoT UE:

3>	forward the extendedWaitTime to upper layers;

2>	if the extendedWaitTime-CPdata is present and the NB-IoT UE only supports the Control Plane CIoT EPS optimisation:

3>	forward the extendedWaitTime-CPdata to upper layers;

2>	if the releaseCause received in the RRCConnectionRelease message indicates rrc-Suspend:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC suspension';

2>	else if rrc-InactiveConfig is included:

3>	perform the actions upon entering RRC_INACTIVE as specified in 5.3.8.7;

2>	else:

3>	perform the actions upon leaving RRC_CONNECTED or RRC_INACTIVE as specified in 5.3.12, with release cause 'other';

#### 5.3.8.4	T320 expiry

The UE shall:

1>	if T320 expires:

2>	if stored, discard the cell reselection priority information provided by the idleModeMobilityControlInfo or inherited from another RAT;

2>	apply the cell reselection priority information broadcast in the system information;

#### 5.3.8.5	T322 expiry

The UE shall:

1>	if T322 expires:

2>	discard the redirectedCarrierOffsetDedicated provided in RRCConnectionRelease message;

#### 5.3.8.6	UE actions upon receiving the expiry of DataInactivityTimer

Upon receiving the expiry of DataInactivityTimer from lower layers while in RRC_CONNECTED, the UE shall:

1>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'RRC connection failure';

#### 5.3.8.7	UE actions upon entering RRC_INACTIVE

Upon entering RRC_INACTIVE, the UE shall:

1>	reset MAC and release the default MAC configuration if any;

1>	stop all timers that are running except T302, T309, T320 and T325;

1>	re-establish RLC entities for all SRBs and DRBs;

1>	if the RRCConnectionRelease message is including the waitTime:

2>	start timer T302, with the timer value set according to the waitTime;

2>	inform the upper layer that access barring is applicable for all access categories except categories '0' and '2';

1>	if T309 is running:

2>	stop timer T309 for all access categories;

2>	perform the actions as specified in 5.3.16.4.

1>	apply the received rrc-InactiveConfig;

1>	if the RRCConnectionRelease message was received in response to an RRCConnectionResumeRequest:

2>	in the stored UE Inactive AS context:

3>	replace the KeNB and KRRCint keys with the current KeNB and KRRCint keys;

3>	replace the C-RNTI with the temporary C-RNTI which the UE has used to receive the RRCConnectionRelease message;

3>	replace the cellIdentity with the cellIdentity of the PCell at the time the UE has received the RRCConnectionRelease message;

3>	replace the previously stored physical cell identity with the physical cell identity of the PCell at the time the UE has received the RRCConnectionRelease message;

1>	else:

2>	in the UE Inactive AS Context, the current KeNB and KRRCint keys, the ROHC state, the C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell, and all other parameters configured;

1>	if the periodic-RNAU-timer is included:

2>	start timer T380, with the timer value set to the periodic-RNAU-timer;

1>	suspend all SRB(s) and DRB(s), except SRB0;

1>	indicate PDCP suspend to lower layers of all DRBs;

1>	indicate the suspension of the RRC connection to upper layers;

1>	enter RRC_INACTIVE and perform procedures as specified in TS 36.304 [4], clause 5.2.7;

Upon selecting to an inter-RAT cell or switching to another CN type, the UE shall:

1>	perform the actions upon leaving RRC_INACTIVE as specified in 5.3.12, with release cause 'other';

### 5.3.12	UE actions upon leaving RRC_CONNECTED or RRC_INACTIVE

Upon leaving RRC_CONNECTED or RRC_INACTIVE, the UE shall:

1>	reset MAC;

1>	if the UE is leaving RRC_INACTIVE:

2>	if going to RRC_IDLE was not triggered by reception of the RRCConnectionRelease message:

3>	stop the timer T320, if running;

3>	if stored, discard the cell reselection priority information provided by the idleModeMobilityControlInfo;

1> else if T302 is running:

2>	stop timer T302;

2> if the UE is connected to 5GC:

3>	perform the actions as specified in 5.3.16.4;

1>	if T309 is running:

2>	stop timer T309 for all access categories;

2>	perform the actions as specified in 5.3.16.4.

1>	stop all timers that are running except T302, T320, T322, T325, T330, T331;

1>	if leaving RRC_CONNECTED was triggered by suspension of the RRC:

2>	re-establish RLC entities for all SRBs and DRBs, including RBs configured with NR PDCP;

2>	store the UE AS Context including the current RRC configuration, the current security context, the PDCP state including ROHC state, C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell;

2>	store the following information provided by E-UTRAN:

3>	the resumeIdentity;

3>	the nextHopChainingCount, if present. Otherwise discard any stored nextHopChainingCount that does not correspond to stored key KRRCint;

3>	the drb-ContinueROHC, if present. Otherwise discard any stored drb-ContinueROHC;

2>	suspend all SRB(s) and DRB(s), including RBs configured with NR PDCP, except SRB0;

2>	indicate the suspension of the RRC connection to upper layers;

2>	configure lower layers to suspend integrity protection and ciphering;

NOTE 1:	Except for UP-EDT, ciphering is not applied for the subsequent RRCConnectionResume message used to resume the connection and an integrity check is performed by lower layers, but merely upon request from RRC.

1>	else:

2>	upon leaving RRC_INACTIVE:

3>	discard the UE Inactive AS context;

3>	discard the KeNB, the KRRCenc key, the KRRCint and the KUPenc key;

2>	release the rrc-InactiveConfig, if configured;

2>	release all radio resources, including release of the MAC configuration, the RLC entity and the associated PDCP entity and SDAP (if any) for all established RBs;

2>	indicate the release of the RRC connection to upper layers together with the release cause;

1>	if leaving RRC_CONNECTED was not triggered by reception of the MobilityFromEUTRACommand message, inter-RAT cell reselection while the UE is in RRC_INACTIVE or by selecting an inter-RAT cell while T311 was running:

2>	if timer T350 is configured:

3>	start timer T350;

3>	apply rclwi-Configuration if configured, otherwise apply the wlan-Id-List corresponding to the RPLMN included in SystemInformationBlockType17;

2>	else:

3>	release the wlan-OffloadConfigDedicated, if received;

3>	if the wlan-OffloadConfigCommon corresponding to the RPLMN is broadcast by the cell:

4>	apply the wlan-OffloadConfigCommon corresponding to the RPLMN included in SystemInformationBlockType17;

4>	apply steerToWLAN if configured, otherwise apply the wlan-Id-List corresponding to the RPLMN included in SystemInformationBlockType17;

2>	enter RRC_IDLE and perform procedures as specified in TS 36.304 [4], clause 5.2.7;

1>	else:

2>	release the wlan-OffloadConfigDedicated, if received;

NOTE 2:	BL UEs or UEs in CE verifies validity of SI when released to RRC_IDLE.

1>	release the LWA configuration, if configured, as described in 5.6.14.3;

1>	release the LWIP configuration, if configured, as described in 5.6.17.3;

#### 5.3.16.2	Initiation

Upon initiation of the procedure, the UE shall:

1>	if T309 is running for the Access Category:

2>	consider the access attempt as barred;

1>	else if timer T302 is running and the Access Category is neither '2' nor '0':

2>	consider the access attempt as barred;

1>	else:

2>	if the Access Category is '0':

3>	consider the access attempt as allowed;

2>	else if SystemInformationBlockType25 is not broadcasted:

3>	consider the access attempt as allowed;

2>	else:

3>	if SystemInformationBlockType25 includes uac-BarringPerPLMN-List and the uac-BarringPerPLMN-List contains an UAC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers (see TS 24.501 [95]):

4>	select the UAC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers;

4>	in the remainder of this procedure, use the selected UAC-BarringPerPLMN entry (i.e. presence or absence of access barring parameters in this entry) irrespective of the uac-BarringForCommon included in SystemInformationBlockType25;

3>	else if SystemInformationBlockType25 includes uac-BarringForCommon:

4>	in the remainder of this procedure use the uac-BarringForCommon (i.e. presence or absence of these parameters) included in SystemInformationBlockType25;

3>	else:

4>	consider the access attempt as allowed;

3>	if uac-BarringForCommon is applicable or the uac-AC-BarringListType indicated that uac-ExplicitAC-BarringList is used:

4>	if the corresponding UAC-BarringPerCatList contains a UAC-BarringPerCat entry corresponding to the Access Category:

5>	select the UAC-BarringPerCat entry;

5>	if the uac-BarringInfoSetList contain a UAC-BarringInfoSet entry corresponding to the uac-barringInfoSetIndex in the UAC-BarringPerCat:

6>	select the UAC-BarringInfoSet entry;

6>	perform access barring check for the Access Category as specified in 5.3.16.5, using the UAC-BarringInfoSet as "UAC barring parameter";

5>	else:

6>	consider the access attempt as allowed;

4>	else:

5>	consider the access attempt as allowed;

3>	else if the uac-AC-BarringListType indicated that uac-ImplicitAC-BarringList is indicated:

4>	select the uac-BarringInfoSetIndex corresponding to the Access Category in the uac-ImplicitACBarringList;

4>	if the uac-BarringInfoSetList contain the UAC-BarringInfoSet entry corresponding to the selected uac-BarringInfoSetIndex:

5>	select the UAC-BarringInfoSet entry;

5>	perform access barring check for the Access Category as specified in 5.3.16.5, using the UAC-BarringInfoSet as "UAC barring parameter";

4>	else:

5>	consider the access attempt as allowed;

3>	else:

4>	consider the access attempt as allowed;

1>	if the access barring check was requested by upper layers:

2>	if the access attempt is considered as barred:

3>	if timer T302 is running:

4> if timer T309 is running for Access Category '2':

5>	inform the upper layer that access barring is applicable for all access categories except categories '0', upon which the procedure ends;

4> else

5>	inform the upper layer that access barring is applicable for all access categories except categories '0' and '2', upon which the procedure ends;

3>	else:

4>	inform upper layers that the access attempt for the Access Category is barred, upon which the procedure ends;

2>	else:

3>	inform upper layers that the access attempt for the Access Category is allowed, upon which the procedure ends;

1>	else:

2>	the procedure ends;

#### 5.3.16.3	Void

#### 5.3.16.4	T302, T309 expiry or stop (Barring alleviation)

If the UE is connected to 5GC, the UE shall:

1>	if timer T302 expires or is stopped:

2> for each Access Category for which T309 is not running:

3>	consider the barring for this Access Category to be alleviated:

1>	else if timer T309 corresponding to an Access Category other than '2' expires or is stopped, and if timer T302 is not running;

2>	consider the barring for this Access Category to be alleviated;

1>	else if timer T309 corresponding to the Access Category '2' expires or is stopped:

2>	consider the barring for this Access Category to be alleviated;

1>	When barring for an access category is considered being alleviated:

2>	if the Access Category was informed to upper layers as barred:

3>	inform upper layers about barring alleviation for the Access Category;

2>	if barring is alleviated for Access Category '8':

3>	perform actions specified in 5.3.17;

### 6.2.2	Message definitions

/*****omit unrelated parts*****/

#### –	RRCConnectionRelease

The RRCConnectionRelease message is used to command the release of an RRC connection, or to complete an UP-EDT procedure.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

RRCConnectionRelease message

-- ASN1START

RRCConnectionRelease ::=			SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE {

rrcConnectionRelease-r8				RRCConnectionRelease-r8-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionRelease-r8-IEs ::=		SEQUENCE {

releaseCause						ReleaseCause,

redirectedCarrierInfo				RedirectedCarrierInfo				OPTIONAL,	-- Need ON

idleModeMobilityControlInfo			IdleModeMobilityControlInfo			OPTIONAL,	-- Need OP

nonCriticalExtension				RRCConnectionRelease-v890-IEs		OPTIONAL

}

RRCConnectionRelease-v890-IEs ::=	SEQUENCE {

lateNonCriticalExtension			OCTET STRING (CONTAINING RRCConnectionRelease-v9e0-IEs)	OPTIONAL,

nonCriticalExtension				RRCConnectionRelease-v920-IEs		OPTIONAL

}

-- Late non critical extensions

RRCConnectionRelease-v9e0-IEs ::= SEQUENCE {

redirectedCarrierInfo-v9e0			RedirectedCarrierInfo-v9e0			OPTIONAL,	-- Cond NoRedirect-r8

idleModeMobilityControlInfo-v9e0	IdleModeMobilityControlInfo-v9e0	OPTIONAL,	-- Cond IdleInfoEUTRA

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- Regular non critical extensions

RRCConnectionRelease-v920-IEs ::=	SEQUENCE {

cellInfoList-r9					CHOICE {

geran-r9						CellInfoListGERAN-r9,

utra-FDD-r9						CellInfoListUTRA-FDD-r9,

utra-TDD-r9						CellInfoListUTRA-TDD-r9,

...,

utra-TDD-r10					CellInfoListUTRA-TDD-r10

}															OPTIONAL,	-- Cond Redirection

nonCriticalExtension			RRCConnectionRelease-v1020-IEs		OPTIONAL

}

RRCConnectionRelease-v1020-IEs ::=	SEQUENCE {

extendedWaitTime-r10				INTEGER (1..1800)		OPTIONAL,	-- Need ON

nonCriticalExtension			RRCConnectionRelease-v1320-IEs				OPTIONAL

}

RRCConnectionRelease-v1320-IEs::=	SEQUENCE {

resumeIdentity-r13					ResumeIdentity-r13				OPTIONAL,	-- Need OR

nonCriticalExtension				RRCConnectionRelease-v1530-IEs	OPTIONAL

}

RRCConnectionRelease-v1530-IEs ::=	SEQUENCE {

drb-ContinueROHC-r15				ENUMERATED {true}			OPTIONAL,	-- Cond UP-EDT

nextHopChainingCount-r15			NextHopChainingCount		OPTIONAL,	-- Cond UP-EDT

measIdleConfig-r15					MeasIdleConfigDedicated-r15	OPTIONAL,	-- Need ON

rrc-InactiveConfig-r15				RRC-InactiveConfig-r15		OPTIONAL,	-- Need OR

cn-Type-r15							ENUMERATED {epc,fivegc}		OPTIONAL,	-- Need OR

nonCriticalExtension				RRCConnectionRelease-v1540-IEs			OPTIONAL

}

RRCConnectionRelease-v1540-IEs ::=	SEQUENCE {

waitTime							INTEGER (1..16)		OPTIONAL, -- Cond 5GC

nonCriticalExtension				SEQUENCE {}				OPTIONAL

}

ReleaseCause ::=				ENUMERATED {loadBalancingTAUrequired,

other, cs-FallbackHighPriority-v1020, rrc-Suspend-v1320}

RedirectedCarrierInfo ::=			CHOICE {

eutra								ARFCN-ValueEUTRA,

geran								CarrierFreqsGERAN,

utra-FDD							ARFCN-ValueUTRA,

utra-TDD							ARFCN-ValueUTRA,

cdma2000-HRPD						CarrierFreqCDMA2000,

cdma2000-1xRTT						CarrierFreqCDMA2000,

...,

utra-TDD-r10						CarrierFreqListUTRA-TDD-r10,

nr-r15								CarrierInfoNR-r15

}

RedirectedCarrierInfo-v9e0 ::=			SEQUENCE {

eutra-v9e0								ARFCN-ValueEUTRA-v9e0

}

RRC-InactiveConfig-r15::=		SEQUENCE {

fullI-RNTI-r15					I-RNTI-r15,

shortI-RNTI-r15					ShortI-RNTI-r15,

ran-PagingCycle-r15				ENUMERATED {	rf32, rf64, rf128, rf256}	OPTIONAL,	--Need OR

ran-NotificationAreaInfo-r15	RAN-NotificationAreaInfo-r15		OPTIONAL,	--Need ON

periodic-RNAU-timer-r15			ENUMERATED {min5, min10, min20, min30, min60,

min120, min360, min720}		OPTIONAL,	--Need OR

nextHopChainingCount-r15		NextHopChainingCount		OPTIONAL,	--Cond INACTIVE

nonCriticalExtension			SEQUENCE{}		OPTIONAL

}

RAN-NotificationAreaInfo-r15	::= CHOICE {

cellList-r15				PLMN-RAN-AreaCellList-r15,

ran-AreaConfigList-r15		PLMN-RAN-AreaConfigList-r15

}

PLMN-RAN-AreaCellList-r15	::=	SEQUENCE (SIZE (1..maxPLMN-r15)) OF PLMN-RAN-AreaCell-r15

PLMN-RAN-AreaCell-r15	::=		SEQUENCE {

plmn-Identity-r15				PLMN-Identity	OPTIONAL,

ran-AreaCells-r15				SEQUENCE (SIZE (1..32)) OF CellIdentity

}

PLMN-RAN-AreaConfigList-r15	::=	SEQUENCE (SIZE (1..maxPLMN-r15)) OF PLMN-RAN-AreaConfig-r15

PLMN-RAN-AreaConfig-r15	::=	SEQUENCE {

plmn-Identity-r15			PLMN-Identity	OPTIONAL,

ran-Area-r15				SEQUENCE (SIZE (1..16)) OF	RAN-AreaConfig-r15

}

RAN-AreaConfig-r15	::=	SEQUENCE {

trackingAreaCode-5GC-r15	TrackingAreaCode-5GC-r15,

ran-AreaCodeList-r15		SEQUENCE (SIZE (1..32)) OF RAN-AreaCode-r15	OPTIONAL	--Need OR

}

CarrierFreqListUTRA-TDD-r10 ::=			SEQUENCE (SIZE (1..maxFreqUTRA-TDD-r10)) OF ARFCN-ValueUTRA

IdleModeMobilityControlInfo ::=		SEQUENCE {

freqPriorityListEUTRA				FreqPriorityListEUTRA			OPTIONAL,		-- Need ON

freqPriorityListGERAN				FreqsPriorityListGERAN			OPTIONAL,		-- Need ON

freqPriorityListUTRA-FDD			FreqPriorityListUTRA-FDD		OPTIONAL,		-- Need ON

freqPriorityListUTRA-TDD			FreqPriorityListUTRA-TDD		OPTIONAL,		-- Need ON

bandClassPriorityListHRPD			BandClassPriorityListHRPD		OPTIONAL,		-- Need ON

bandClassPriorityList1XRTT			BandClassPriorityList1XRTT		OPTIONAL,		-- Need ON

t320								ENUMERATED {

min5, min10, min20, min30, min60, min120, min180,

spare1}						OPTIONAL,		-- Need OR

...,

[[	freqPriorityListExtEUTRA-r12		FreqPriorityListExtEUTRA-r12		OPTIONAL		-- Need ON

]],

[[	freqPriorityListEUTRA-v1310			FreqPriorityListEUTRA-v1310			OPTIONAL,		-- Need ON

freqPriorityListExtEUTRA-v1310		FreqPriorityListExtEUTRA-v1310		OPTIONAL		-- Need ON

]],

[[	freqPriorityListNR-r15				FreqPriorityListNR-r15		OPTIONAL		-- Need ON

]]

}

IdleModeMobilityControlInfo-v9e0 ::=	SEQUENCE {

freqPriorityListEUTRA-v9e0			SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityEUTRA-v9e0

}

FreqPriorityListEUTRA ::=			SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityEUTRA

FreqPriorityListExtEUTRA-r12 ::=		SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityEUTRA-r12

FreqPriorityListEUTRA-v1310 ::=			SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityEUTRA-v1310

FreqPriorityListExtEUTRA-v1310 ::=		SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityEUTRA-v1310

FreqPriorityEUTRA ::=				SEQUENCE {

carrierFreq							ARFCN-ValueEUTRA,

cellReselectionPriority				CellReselectionPriority

}

FreqPriorityEUTRA-v9e0 ::=			SEQUENCE {

carrierFreq-v9e0					ARFCN-ValueEUTRA-v9e0		OPTIONAL	-- Cond EARFCN-max

}

FreqPriorityEUTRA-r12 ::=				SEQUENCE {

carrierFreq-r12							ARFCN-ValueEUTRA-r9,

cellReselectionPriority-r12				CellReselectionPriority

}

FreqPriorityEUTRA-v1310 ::=				SEQUENCE {

cellReselectionSubPriority-r13				CellReselectionSubPriority-r13		OPTIONAL		-- Need ON

}

FreqPriorityListNR-r15 ::=		SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityNR-r15

FreqPriorityNR-r15 ::=			SEQUENCE {

carrierFreq-r15						ARFCN-ValueNR-r15,

cellReselectionPriority-r15			CellReselectionPriority,

cellReselectionSubPriority-r15		CellReselectionSubPriority-r13		OPTIONAL		-- Need OR

}

FreqsPriorityListGERAN ::=			SEQUENCE (SIZE (1..maxGNFG)) OF FreqsPriorityGERAN

FreqsPriorityGERAN ::=				SEQUENCE {

carrierFreqs						CarrierFreqsGERAN,

cellReselectionPriority				CellReselectionPriority

}

FreqPriorityListUTRA-FDD ::=		SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF FreqPriorityUTRA-FDD

FreqPriorityUTRA-FDD ::=			SEQUENCE {

carrierFreq							ARFCN-ValueUTRA,

cellReselectionPriority				CellReselectionPriority

}

FreqPriorityListUTRA-TDD ::=		SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF FreqPriorityUTRA-TDD

FreqPriorityUTRA-TDD ::=			SEQUENCE {

carrierFreq							ARFCN-ValueUTRA,

cellReselectionPriority				CellReselectionPriority

}

BandClassPriorityListHRPD ::=		SEQUENCE (SIZE (1..maxCDMA-BandClass)) OF BandClassPriorityHRPD

BandClassPriorityHRPD ::=			SEQUENCE {

bandClass							BandclassCDMA2000,

cellReselectionPriority				CellReselectionPriority

}

BandClassPriorityList1XRTT ::=	SEQUENCE (SIZE (1..maxCDMA-BandClass)) OF BandClassPriority1XRTT

BandClassPriority1XRTT ::=			SEQUENCE {

bandClass							BandclassCDMA2000,

cellReselectionPriority				CellReselectionPriority

}

CellInfoListGERAN-r9 ::=		SEQUENCE (SIZE (1..maxCellInfoGERAN-r9)) OF CellInfoGERAN-r9

CellInfoGERAN-r9 ::=				SEQUENCE {

physCellId-r9						PhysCellIdGERAN,

carrierFreq-r9						CarrierFreqGERAN,

systemInformation-r9				SystemInfoListGERAN

}

CarrierInfoNR-r15	::= SEQUENCE {

carrierFreq-r15					ARFCN-ValueNR-r15,

subcarrierSpacingSSB-r15			ENUMERATED {kHz15, kHz30, kHz120, kHz240},

smtc-r15							MTC-SSB-NR-r15				OPTIONAL		-- Need OP

}

CellInfoListUTRA-FDD-r9 ::=			SEQUENCE (SIZE (1..maxCellInfoUTRA-r9)) OF CellInfoUTRA-FDD-r9

CellInfoUTRA-FDD-r9 ::=				SEQUENCE {

physCellId-r9						PhysCellIdUTRA-FDD,

utra-BCCH-Container-r9				OCTET STRING

}

CellInfoListUTRA-TDD-r9 ::=			SEQUENCE (SIZE (1..maxCellInfoUTRA-r9)) OF CellInfoUTRA-TDD-r9

CellInfoUTRA-TDD-r9 ::=				SEQUENCE {

physCellId-r9						PhysCellIdUTRA-TDD,

utra-BCCH-Container-r9				OCTET STRING

}

CellInfoListUTRA-TDD-r10 ::=		SEQUENCE (SIZE (1..maxCellInfoUTRA-r9)) OF CellInfoUTRA-TDD-r10

CellInfoUTRA-TDD-r10 ::=			SEQUENCE {

physCellId-r10						PhysCellIdUTRA-TDD,

carrierFreq-r10						ARFCN-ValueUTRA,

utra-BCCH-Container-r10				OCTET STRING

}

-- ASN1STOP

RRCConnectionRelease field descriptions

carrierFreq or bandClass The carrier frequency (UTRA, E-UTRA, and NR) and band class (HRPD and 1xRTT) for which the associated cellReselectionPriority is applied. For NR, the ARFCN-ValueNR corresponds to a GSCN value as specified in TS 38.101 [85].

carrierFreqs The list of GERAN carrier frequencies organised into one group of GERAN carrier frequencies.

cellInfoList Used to provide system information of one or more cells on the redirected inter-RAT carrier frequency. The system information can be used if, upon redirection, the UE selects an inter-RAT cell indicated by the physCellId and carrierFreq (GERAN and UTRA TDD) or by the physCellId (other RATs). The choice shall match the redirectedCarrierInfo. In particular, E-UTRAN only applies value utra-TDD-r10 in case redirectedCarrierInfo is set to utra-TDD-r10.

cellList Indicates a list of cells configured as RAN area. For each element, in the absence of plmn-Identity the UE considers the registered PLMN. Total number of cells across all PLMNs does not exceed 32.

cn-Type The cn-Type is used to indicate that the UE is redirected from 5GC to EPC or 5GC when redirectedCarrierInfo indicates E-UTRA frequency.

drb-ContinueROHC This field indicates whether to continue or reset the header compression protocol context for the DRBs configured with the header compression protocol. Presence of the field indicates that the header compression protocol context continues when UE initiates UP-EDT in the same cell, while absence indicates that the header compression protocol context is reset.

extendedWaitTime Value in seconds for the wait time for Delay Tolerant access requests.

freqPriorityListX Provides a cell reselection priority for each frequency, by means of separate lists for each RAT (including E-UTRA). The UE shall be able to store at least 3 occurrences of FreqsPriorityGERAN. If E-UTRAN includes freqPriorityListEUTRA-v9e0 and/or freqPriorityListEUTRA-v1310 it includes the same number of entries, and listed in the same order, as in freqPriorityListEUTRA (i.e. without suffix). Field freqPriorityListExt includes additional neighbouring inter-frequencies, i.e. extending the size of the inter-frequency carrier list using the general principles specified in 5.1.2. EUTRAN only includes freqPriorityListExtEUTRA if freqPriorityListEUTRA (i.e without suffix) includes maxFreq entries. If E-UTRAN includes freqPriorityListExtEUTRA-v1310 it includes the same number of entries, and listed in the same order, as in freqPriorityListExtEUTRA-r12.

idleModeMobilityControlInfo Provides dedicated cell reselection priorities. Used for cell reselection as specified in TS 36.304 [4]. For E-UTRA and UTRA frequencies, a UE that supports multi-band cells for the concerned RAT considers the dedicated priorities to be common for all overlapping bands (i.e. regardless of the ARFCN that is used).

measIdleConfig Indicates a one-shot measurement configuration to be stored and used by the UE while in RRC_IDLE.

periodic-RNAU-timer Refers to the timer that triggers the periodic RNAU procedure in UE. Value min5 corresponds to 5 minutes, value min10 corresponds to 10 minutes and so on.

ran-Area Indicates whether TA code(s) or RAN area code(s) are used for the RAN notification area. The network uses only TA code(s) or RAN area code(s) to configure a UE. Total number of TACs across all PLMNs does not exceed 16. Total number of RAN-AreaCode across all PLMNs does not exceed 32.

ran-NotificationAreaInfo Network ensures that the UE in RRC_INACTIVE always has a valid ran-NotificationAreaInfo.

ranAreaConfigList Indicates a list of RAN area codes or RA code(s) as RAN area. For each element, in the absence of plmn-Identity the UE considers the registered PLMN.

ran-pagingCycle Refers to the UE specific cycle for RAN-initiated paging. Value rf32 corresponds to 32 radio frames, rf64 corresponds to 64 radio frames and so on.

redirectedCarrierInfo The redirectedCarrierInfo indicates a carrier frequency (downlink for FDD) and is used to redirect the UE to an E-UTRA or an inter-RAT carrier frequency, by means of the cell selection upon leaving RRC_CONNECTED as specified in TS 36.304 [4]. The value geran can only be included after successful security activation when UE is connected to 5GC.

releaseCause The releaseCause is used to indicate the reason for releasing the RRC Connection. The cause value cs-FallbackHighPriority is only applicable when redirectedCarrierInfo is present with the value set to utra-FDD, utra-TDD or utra-TDD-r10. E-UTRAN should not set the releaseCause to loadBalancingTAURequired or to cs-FallbackHighPriority if the extendedWaitTime is present. The network should not set the releaseCause to loadBalancingTAURequired if the UE is connected to 5GC.

rrc-InactiveConfig Indicates configuration for the RRC_INACTIVE state. The network does not configure this field when the UE is redirected to an inter-RAT carrier frequency.

smtc The SSB periodicity/offset/duration configuration of the redirected target NR frequency. It is based on the timing reference of EUTRAN PCell. If the field is absent, the UE uses the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing

subcarrierSpacingSSB Indicate subcarrier spacing of SSB of redirected target NR frequency. Only the values 15 or 30 (<6GHz), 120 kHz or 240 kHz (>6GHz) are applicable.

systemInformation Container for system information of the GERAN cell i.e. one or more System Information (SI) messages as defined in TS 44.018 [45], table 9.1.1.

t320 Timer T320 as described in section 7.3. Value minN corresponds to N minutes.

utra-BCCH-Container Contains System Information Container message as defined in TS 25.331 [19].

waitTime Wait time value in seconds.

| Conditional presence | Explanation |
| --- | --- |
| 5GC | The field is optionally present, Need ON, if the UE is connected to 5GC; otherwise the field is not present. |
| EARFCN-max | The field is mandatory present if the corresponding carrierFreq (i.e. without suffix) is set to maxEARFCN. Otherwise the field is not present. |
| IdleInfoEUTRA | The field is optionally present, Need OP, if the IdleModeMobilityControlInfo (i.e. without suffix) is included and includes freqPriorityListEUTRA; otherwise the field is not present. |
| INACTIVE | The field is mandatory present in this release. |
| NoRedirect-r8 | The field is optionally present, Need OP, if the redirectedCarrierInfo (i.e. without suffix) is not included; otherwise the field is not present. |
| Redirection | The field is optionally present, Need ON, if the redirectedCarrierInfo is included and set to geran, utra-FDD, utra-TDD or utra-TDD-r10; otherwise the field is not present. |
| UP-EDT | The field is optionally present, Need ON, if the UE supports UP-EDT and releaseCause is set to rrc-Suspend; otherwise the field is not present. |

### 7.3.1	Timers (Informative)

| Timer | Start | Stop | At expiry |
| --- | --- | --- | --- |
| T300 NOTE1 | Transmission of RRCConnectionRequest or RRCConnectionResumeRequest or RRCEarlyDataRequest | Reception of RRCConnectionSetup, RRCConnectionReject or RRCConnectionResume or RRCEarlyDataComplete or RRCConnectionRelease for UP-EDT, cell re-selection and upon abortion of connection establishment by upper layers | Perform the actions as specified in 5.3.3.6 |
| T301 NOTE1 | Transmission of RRCConnectionReestabilshmentRequest | Reception of RRCConnectionReestablishment or RRCConnectionReestablishmentReject message as well as when the selected cell becomes unsuitable | Go to RRC_IDLE |
| T302 | Reception of RRCConnectionReject while performing RRC connection establishment or reception of RRCConnectionRelease including waitTime | Upon entering RRC_CONNECTED and upon cell re-selection, or upon reception of RRCEarlyDataComplete or RRCConnectionRelease for UP-EDT or upon reception of RRCConnectionReject message for E-UTRA/5GC. | Inform upper layers about barring alleviation as specified in 5.3.3.7 |
| T303 | Access barred while performing RRC connection establishment for mobile originating calls | Upon entering RRC_CONNECTED and upon cell re-selection, or upon reception of RRCEarlyDataComplete or RRCConnectionRelease for UP-EDT | Inform upper layers about barring alleviation as specified in 5.3.3.7 |
| T304 | Reception of RRCConnectionReconfiguration message including the MobilityControl Info or reception of MobilityFromEUTRACommand message including CellChangeOrder | Criterion for successful completion of handover within E-UTRA, handover to E-UTRA or cell change order is met (the criterion is specified in the target RAT in case of inter-RAT) | In case of cell change order from E-UTRA or intra E-UTRA handover, initiate the RRC connection re-establishment procedure; In case of handover to E-UTRA, perform the actions defined in the specifications applicable for the source RAT. |
| T305 | Access barred while performing RRC connection establishment for mobile originating signalling | Upon entering RRC_CONNECTED and upon cell re-selection, or upon reception of RRCEarlyDataComplete or RRCConnectionRelease for UP-EDT | Inform upper layers about barring alleviation as specified in 5.3.3.7 |
| T306 | Access barred while performing RRC connection establishment for mobile originating CS fallback. | Upon entering RRC_CONNECTED and upon cell re-selection, or upon reception of RRCEarlyDataComplete or RRCConnectionRelease for UP-EDT | Inform upper layers about barring alleviation as specified in 5.3.3.7 |
| T307 | Reception of RRCConnectionReconfiguration message including MobilityControlInfoSCG | Successful completion of random access on the PSCell, upon initiating re-establishment and upon SCG release | Inform E-UTRAN about the SCG change failure by initiating the SCG failure information procedure as specified in 5.6.13. |
| T308 | Access barred due to ACDC while performing RRC connection establishment subject to ACDC | Upon entering RRC_CONNECTED and upon cell re-selection, or upon reception of RRCEarlyDataComplete or RRCConnectionRelease for UP-EDT | Inform upper layers about barring alleviation for ACDC as specified in 5.3.3.7 |
| T309 | When access attempt is barred at access barring check for an Access Category. The UE shall maintain one instance of this timer per Access Category. | Upon entering RRC_CONNECTED, upon cell (re)selection, upon reception of RRCConnectionRelease, upon change of PCell while in RRC_CONNECTED, or upon reception of MobilityFromEUTRACommand. | Perform the actions as specified in 5.3.16.4. |
| T310 NOTE1 NOTE2 | Upon detecting physical layer problems for the PCell i.e. upon receiving N310 consecutive out-of-sync indications from lower layers | Upon receiving N311 consecutive in-sync indications from lower layers for the PCell, upon triggering the handover procedure and upon initiating the connection re-establishment procedure | If security is not activated and the UE is not a NB-IoT UE that supports RRC connection re-establishment for the Control Plane CIoT EPS optimisation: go to RRC_IDLE else: initiate the connection re-establishment procedure |
| T311 NOTE1 | Upon initiating the RRC connection re-establishment procedure | Selection of a suitable E-UTRA cell or a cell using another RAT. | Enter RRC_IDLE |
| T312 NOTE2 | Upon triggering a measurement report for a measurement identity for which T312 has been configured, while T310 is running | Upon receiving N311 consecutive in-sync indications from lower layers, upon triggering the handover procedure, upon initiating the connection re-establishment procedure, and upon the expiry of T310 | If security is not activated: go to RRC_IDLE else: initiate the connection re-establishment procedure |
| T313 NOTE2 | Upon detecting physical layer problems for the PSCell i.e. upon receiving N313 consecutive out-of-sync indications from lower layers | Upon receiving N314 consecutive in-sync indications from lower layers for the PSCell, upon initiating the connection re-establishment procedure, upon SCG release and upon receiving RRCConnectionReconfiguration including MobilityControlInfoSCG | Inform E-UTRAN about the SCG radio link failure by initiating the SCG failure information procedure as specified in 5.6.13. |
| T320 | Upon receiving t320 or upon cell (re)selection to E-UTRA from another RAT with validity time configured for dedicated priorities (in which case the remaining validity time is applied). | Upon entering RRC_CONNECTED, when PLMN selection is performed on request by NAS, when the UE enters RRC_IDLE from RRC_INACTIVE, or upon cell (re)selection to another RAT (in which case the timer is carried on to the other RAT) , or upon reception of RRCEarlyDataComplete or RRCConnectionRelease for UP-EDT | Discard the cell reselection priority information provided by dedicated signalling. |
| T321 | Upon receiving measConfig including a reportConfig with the purpose set to reportCGI | Upon acquiring the information needed to set all fields of cellGlobalId for the requested cell, upon receiving measConfig that includes removal of the reportConfig with the purpose set to reportCGI and upon detecting that a cell is not broadcasting SIB1. | Initiate the measurement reporting procedure, stop performing the related measurements and remove the corresponding measId |
| T322 NOTE1 | Upon receiving redirectedCarrierOffsetDedicated included in RedirectedCarrierInfo | Upon entering RRC_CONNECTED, when PLMN selection is performed on request by NAS, or upon cell (re)selection to another RAT, or upon reception of RRCEarlyDataComplete or RRCConnectionRelease for UP-EDT | Release redirectedCarrierOffsetDedicated. |
| T325 | Timer (re)started upon receiving RRCConnectionReject message with deprioritisationTimer. |  | Stop deprioritisation of all frequencies or E-UTRA signalled by RRCConnectionReject. |
| T330 | Upon receiving LoggedMeasurementConfiguration message | Upon log volume exceeding the suitable UE memory, upon initiating the release of LoggedMeasurementConfiguration procedure | Perform the actions specified in 5.6.6.4 |
| T331 | Upon receiving RRCConnectionRelease message including measIdleConfig. | Upon receiving RRCConnectionSetup, RRCConnectionResume or, if validityArea is configured, upon reselecting to cell that does not belong to validityArea. | Release the stored VarMeasIdleConfig. |
| T340 NOTE2 | Upon transmitting UEAssistanceInformation message with powerPrefIndication set to normal | Upon initiating the connection re-establishment procedure | No action. |
| T341 NOTE2 | Upon transmitting UEAssistanceInformation message with bw-Preference. | Upon resuming an RRC connection or upon initiating the connection re-establishment procedure | No action. |
| T342 NOTE2 | Upon transmitting DelayBudgetReport message. | Upon initiating the connection re-establishment and connection resume procedures | No action. |
| T350 | Upon entering RRC_IDLE if t350 has been received in wlan-OffloadInfo. | Upon entering RRC_CONNECTED, or upon cell reselection. | Perform the actions specified in 5.6.12.4. |
| T351 | Reception of RRCConnectionReconfiguration message including the associationTimer in WLAN-MobilityConfig. | Upon successful connection to WLAN, upon WLAN connection failure, upon leaving RRC_CONNECTED, upon triggering the handover procedure, or upon initiating the connection re-establishment procedure. | Perform WLAN Connection Status Reporting specified in 5.6.15.2. |
| T360 | Upon performing the redistribution target selection as specified in TS 36.304 [4]. | Upon entering RRC_CONNECTED, upon receiving a Paging message including redistributionIndication; upon reselecting a cell not belonging to the redistribution target. | Stop considering a frequency or cell to be redistribution target, and perform the redistribution target selection if the condition specified in TS 36.304 [4] is met. |
| T370 | Upon receiving SL-DiscConfig including a discSysInfoToReportConfig set to setup. | Upon initiating the transmission of SidelinkUEInformation including discSysInfoReportFreqList, upon receiving SL-DiscConfig including discSysInfoToReportConfig set to release, upon handover and re-establishment. | Release discSysInfoToReportConfig. |
| T314 NOTE2 | Upon early detecting physical layer problems for the PCell i.e. upon receiving N310 consecutive "early-out-of-sync" indications from lower layers. | Upon receiving N311 consecutive in-sync indications from lower layers for the PCell, upon triggering the handover procedure and upon initiating the connection re-establishment procedure | Initiate the UE Assistance Information procedure to report early detection of physical layer problems in accordance with 5.6.10. |
| T315 NOTE2 | Upon detecting physical layer improvements of the PCell i.e. upon receiving N311 consecutive "early-in-sync" indications from lower layers. | Upon receiving N310 consecutive "early-out-of-sync" indications from lower layers for the PCell. | Initiate the UE Assistance Information procedure to report detection of physical layer improvements in accordance with 5.6.10. |
| T343 NOTE2 | Upon transmitting UEAssistanceInformation message with RLM-Report including earlyOutOfSync. | Upon initiating the connection re-establishment procedure | No action. |
| T344 NOTE2 | Upon transmitting UEAssistanceInformation message with RLM-Report including earlyInSync. | Upon initiating the connection re-establishment procedure | No action. |
| T345 | Upon transmitting UEAssistanceInformation message with overheatingAssistance | Upon initiating the connection re-establishment procedure | No action. |
| T380 | Upon reception of periodic-RNAU-timer in RRCConnectionRelease. | Upon reception of RRCConnectionResume, RRCConnectionRelease or RRCConnectionSetup. | Initiate the RAN notification area update procedure |
| NOTE1: Only the timers marked with "NOTE1" are applicable to NB-IoT. NOTE2: The behaviour as specified in 7.3.2 applies. |  |  |  |

End Text Proposal Change