3GPP TSG-RAN WG2 Meeting #106	R2-1908143

Reno, Nevada, USA, 13th – 17th May 2019

Agenda Item:	11.12.3

Source: 	MediaTek Inc.

Title:  	TP for UE energy saving

Document for:	Discussion and decision

# Introduction

R2-1906963 TP for UE energy saving MediaTek Inc. pCR Rel-16 37.816 0.3.0 FS_LTE_NR_data_collect => Remove the solution description part => With this change TP is agreed in R2-1908143

The TP is included below.

# Text Proposal for TR 37.816

5.x UE energy saving

5.x.1 Use case description

High-end mobile devices with 5G modem are expected to feature advanced components e.g. display, processors, graphic engine, etc. Added up, these render power management to be very challenging. In other words, power efficiency is critical for every single component, in particular for the modem itself, regardless of the power source used (e.g. battery capacity and technology).

Battery lifetime is an important criterion directly affecting user experience – no matter how good the other performance aspects of a device may be, if its battery lifetime is poor, user experience will be greatly impacted. To ensure a smooth migration from 4G to 5G, the battery lifetime should thus be, under similar criteria, on par or better using 5G than 4G.

Therefore, network and UE vendors have a common interest in ensuring that the 5G NR ecosystem is able to maximize battery lifetime for all its users in all conditions. This means network and UE ought to cooperate to ensure lower baseline power and maintain better efficiency with all data rates, as shown below.

Figure 1: UE power variation with data rate

3GPP has started an effort, i.e. power saving SI, to achieve this target. From the study, as shown on Figure 2, for different traffic types, roughly less than 36% of the total power consumption occurs in sleep state (~RRC_Idle mode). Said otherwise, reducing the power consumption in RRC_Connected mode is the primary target. As the UE behavior in RRC_Connected mode is controlled by the network, the joint role of the UE and the network to reduce power consumption in this state is clear.

Figure 2: Power consumption distribution for different traffic models

For 5G/NR design UE energy efficiency is listed as one of the key performance indicators in 3GPP TR 38.913.

UE energy efficiency, which means the capability of a UE to sustain much better mobile broadband data rate while minimizing the UE modem energy consumption both qualitative and quantitative KPIs are required. Although only a qualitative KPI is captured now, it is also specified that based on the study, more detailed quantitative assessment can be performed.

Although it is possible to measure power consumption of a UE directly, such direct measurement is vendor dependent, i.e. only represent the power consumption of a specific UE model, and it can only represent UE part of the power saving efforts. There is a need for a UE energy efficiency (UE EE) measurement to represent the network part of the power saving efforts, which can be used for network optimization. R1 power consumption model agreed in TR 38.840 can be used as a baseline for such vendor-independent UE energy efficiency measurement. For network optimization, a reliable quantitative KPI UE energy efficiency (UE EE) shall be consistent across UE vendors.

The objective of this use case is to measure the network performance on UE energy efficiency through RAN data collection, therefore, the effect of power saving features can be objectively reflected, which in the end leads to network and UE to work together to maximize UE battery lifetime and enhance user experience.

5.x.2 Solution description

5.x.3 Conclusion

# References

- GTI Sub-6GHz 5G Device White Paper, Chapter 13 Power Consumption

- RP-181463, WID: Study on UE Power Saving in NR

- 3GPP TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”, V14.3.0 (2017-06)

- Draft TR 38.840 – Study on UE Power Saving (CATT)

- R1-1901298, Adaptation Designs for NR UE Power Saving, MediaTek Inc.

- RP-182105, WID: Study on RAN-centric Data Collection and Utilization for LTE and NR

- R2-1901331	New use case: UE energy saving	MediaTek Inc.	discussion

- R2-1903367	Summary of email discussion [105#43][NR/RD-CU] Use case of UE energy saving (MediaTek)     MediaTek Inc.