3GPP TSG-RAN2 Meeting #106	R2-1908163

Reno, Nevada, USA, 13 May - 17 May 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1708 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE capability signalling for FD-MIMO processing capabilities for EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-05-23 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The CR is based on LS from RAN1 in R2-1908429/R1-1907628, section 1.1. Background: RAN1 has previously sent another LS to RAN2 (in R2-1905505/R1-1905577) indicating the need to add FD-MIMO capability signalling. As a result, RAN2 agreed CRs in R2-1908425 (TS 36.331) and R2-1908428 (TS 36.306). The CRs introduced the following parameters in UE-EUTRA-Capability on a per-UE basis: - relWeightTwoLayers-r13 - relWeightFourLayers-r13 - relWeightEightLayers-r13 - totalWeightedLayers-r13 Further, the CR also introduced the capability restriction inequality in TS 36.306. See the CRs indicated above for further details and signalling examples. Current change: The new LS is requesting to specify that the “Total number of weighted layers” can also be signalled in EN-DC band combination. It is also noted that this applies to only LTE part of EN-DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clarification on totalWeightedLayers in case per BC capability signalling in CA-ParametersEUTRA is included for EN-DC. Impact Analysis: Impacted 5G architecture options: EN-DC Impacted Functionality: UE EUTRA capability for FD-MIMO Interoperability: - If the network implements the CR but the UE does not, no interoperability is foreseen as the UE will not include the FD-MIMO processing capabilities per band combination, i.e., legacy capabilities apply. - If the UE implements the CR but the network does not, UE may include the FD-MIMO processing capability per band combination that the network ignores, which means the network may wrongly overestimate the UE’s FD-MIMO processing capabilities. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN2 specification remains misaligned with RAN1 agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.28.yy (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 38.306 CR 0133 TS 38.331 CR 1114 |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | In 4.3.28.yy, the additional changes from this CR compared to previously agreed CR 1695 is marked by yellow highlight. |  |  |  |  |  |  |  |  |

First change

#### 4.3.28.yy	totalWeightedLayers-r13

This field indicates total number of weighted layers the UE can process for FD-MIMO, as described in equation 4.3.28.yy-1 below and TS 36.331 [5] subclause 6.3.6, NOTE X in UE-EUTRA-Capability field descriptions.

The FD-MIMO processing capability condition is satisfied if:

i ∈ configured CCswi∙li≤totalWeightedLayers

where:

-	totalWeightedLayers is total number of weighted layers as indicated by totalWeightedLayers in ca-ParametersEUTRA, if present, for EN-DC band combination (see TS 38.331 [xx] and TS 38.306 [32]); otherwise as indicated by totalWeightedLayers-r13,

- 	li is the maximum number of DL layers configured for CC i, and

- wi=relWeightTwoLayers,  if CC i is configured with FD-MIMO and li=2relWeightFourLayers,  if CC i is configured with FD-MIMO and li=4relWeightEightLayers,  if CC i is configured with FD-MIMO and li=81,   if CC i  is not configured with FD-MIMO.

Equation 4.3.28.yy-1: FD-MIMO processing capability condition.

End of changes