3GPP TSG RAN2 Meeting #106	R2-1908250

# Reno, US, 13th - 17th of May 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.305 | CR | 0082 | rev | 1 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Minor updates for motion sensors |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sony, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LCS_LTE_acc_enh-Core |  |  |  |  |  |  | Date: |  | 2019-04-30 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | At RAN2#105 meeting CR 0080 was agreed, R2-1902723 to include support for motion sensor based positioning in stage 2 specification. At RAN2#105bis, similar description was added to 38.305 in CR 0009, R2-1905311 that was in principle agreed. In addittion some minor updates were added. Corresponding minor updates are proposed in this CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Minor updates adding Reference position and Reference time. Impact analysis Impacted functionality: Sensor positioning. Inter-operability: No interoperability issues. Alignment of stage 2 specification with 38.305 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between LTE and NR stage 2 remains |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR….CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Beginning of change

ns.

## 8.10	Motion sensor positioning method

### 8.10.1	General

Motion sensors can be used to estimate the location of the UE. With the combination of other positioning methods (hybrid) a more accurate position of the UE can be computed. UE using one or more motion sensors provides the movement information. The movement information comprises displacement results estimated at an ordered series of points.

The positioning modes supported are UE-Assisted, UE-Based, and Standalone.

### 8.10.2	Information to be transferred between E-UTRAN Elements

This subclause defines the information (e.g., assistance data, position and/or measurement data) that may be transferred between E-UTRAN elements.

#### 8.10.2.1	Information that may be transferred from the UE to E-SMLC

The information transferred from the UE to the E-SMLC consists of capability information and location measurements or UE position. The supported information elements are given in Table 8.10.2.1-1.

Table 8.10.2.1-1: Sensor Measurement Information that may be transferred from UE to the E-SMLC

| Information | UE-assisted | UE-based/ Standalone |
| --- | --- | --- |
| - Displacement Timestamp | - Yes | - Yes |
| - Displacement Information | - Yes | - Yes |
| - Reference Position | - Yes | - Yes |
| - Reference Time | - Yes | - Yes |

##### 8.10.2.1.1	UE-assisted, UE-based, Standalone mode

In the UE-assisted, UE-Based, and Standalone mode, the UE reports, displacement information, displacement timestamp, reference position and reference time stamp.

##### 8.10.2.1.2	UE Displacement and Movement Information

The UE may report movement and displacement information which comprises an ordered series of direction and distance travelled by the target device and the time intervals when these measurements are taken.

#### 8.10.2.2	Information that may be transferred from the E-SMLC to the UE

In this release, no information, e.g. assistance data is transferred to the UE.

End of Change