3GPP TSG-RAN WG2#106	Tdoc R2-1907310

Reno, Nevada, USA, 13th – 17th May 2019

Agenda Item:	12.3.2.1

Source:	Ericsson

Title:	Enhancements to Make-Before-Break for dual active protocol stacks

Document for:	Discussion, Decision

# Introduction

At the RAN#80 meeting in June 2018, a new Rel-16 WI was approved [1] with the purpose to further enhance LTE mobility by reducing handover interruption time and improve mobility robustness.

At the RAN2#105 meeting it was agreed to specify the “non-split bearer” candidate solution (a.k.a. the enhanced Make-Before-Break (MBB) solution) for the Rel-16 E-UTRA enhancements minimizing the interruption time during mobility.

Although no decision has been taken yet on the two candidate protocol stack solutions (single active protocol stack vs dual active protocol stacks), it was concluded at the RAN2#105bis meeting that any solution specified will be modelled as a single PDCP entity on the UE side.

A comparison between the two candidate solutions are done in the companion paper [2].

In this paper we explore on some further details on the enhanced MBB handover solution using a “compromise solution” of the dual active protocol stack solution.

For the FFS “How to do data forwarding (early/late, including handling the SN, security, CHO impact)” we refer to our paper on Data forwarding at reduced handover interruption [3], although data forwarding is also (briefly) discussed in this document.

# Discussion

## Dual active protocol stack solution

If a single active protocol stack solution is not agreeable to companies in RAN2, a “compromise solution” of the dual active protocol stack solution may be a way forward.

The somewhat simplified dual active protocol stack solution proposed is characterized by the following:

- No capability coordination to take place between the source node and the target node.

- No TDM patterns will be defined to support simultaneous UL transmission in the source cell and in the target cell. If simultaneous UL transmission is not possible e.g. in case of intra-frequency handover, a simple approach in which the UE prioritize UL transmission in the target cell shall be applied.

- After the UE has completed the random-access procedure, new and retransmitted PDCP packets (i.e. packets that were not successfully received in the source eNB) are only sent to the target cell, whereas the UE may still receive PDCP packets from both the source cell and the target cell.

- The connection to the source cell should be released without requiring an additional reconfiguration message in the target cell.

## Enhanced MBB in Rel-16

Even if MBB and RACH-less HO (both concepts introduced in Rel-14) are combined in the same handover procedure, it is still not possible to accomplish seamless mobility, i.e. true 0ms HO interruption time, in all scenarios. In order to meet this target, the UE need to maintain the connection with the source eNB until the connection to the target eNB is successfully established.

In other words, the UE need to keep its connection with the source cell until it is able to receive and transmit packets in the target cell. In principle, this means that the source eNB, after sending the RRCConnectionReconfiguration message (Handover Command) to the UE, continues to send and receive packets to/from the UE until the UE is able to receive and transmit packets in the target cell.

Starting data forwarding at an early stage (e.g. when sending the Handover Command to the UE) means that the target eNB can always start DL transmission immediately when the UE has completed the handover to the target cell, which (due to X2 latency) may not be the case if data forwarding is started later in the Handover Execution phase (e.g. if data forwarding is triggered by the UE or by the target eNB when the UE performs random-access in the target cell). But an early start of data forwarding also implies that the number of DL PDCP duplications sent and buffered in the target eNB (and possibly sent to the UE in the target cell) will increase compared to a late start of data forwarding.

However, this issue can be resolved if the DL PDCP duplication check is done in the target eNB before transmitting the DL PDCP PDUs to the UE, thus sending of duplicated packets on the radio interface will be avoided.

The principle to support 0ms HO interruption time by means of an enhanced MBB procedure using a “compromise solution” of the dual active protocol stack solution is illustrated in Figure 1 below.

Figure 1 Enhanced MBB procedure to support 0ms HO interruption time.

The key steps to support 0ms HO interruption time by means of enhanced MBB procedure are as follows:

Step 1-2:	The source eNB decides to request for a handover with enhanced MBB, thus a HANDOVER REQUEST message with an eMBB indicator is sent to the target eNB.

Step 3-5:	As in legacy, the target eNB performs admission control and generates the RRCConnectionReconfiguration message (Handover Command) to be sent by the source eNB to the UE. The presence of an eMBB indicator in the RRC message serves as a request to the UE to perform a handover with enhanced MBB.

If some MBB Rel-16 specific interaction is needed between source and target node (e.g. early sending of the SN STATUS TRANSFER message, see [3] and step 6 below), the source eNB need to know whether the target eNB supports enhanced MBB for each individual handover request. For that purpose, an “eMBB support indicator” (or similar) may be needed in the HANDOVER REQUEST ACK message. Consequently, if the target eNB (for some reason) do not support enhanced MBB for this specific handover request, a fallback to a legacy handover (or legacy MBB as defined in Rel-14) is an option.
Obviously, this need to be addressed in the upcoming RAN3 discussions.

Step 6-7:	As proposed in [3], the source eNB applies early start of DL data forwarding to the target eNB while it continues to send and receive PDCP PDUs to/from the UE. As in legacy, the exact point in time when the source eNB starts data forwarding is left for implementation.

The source eNB does not apply any form of selective data forwarding, i.e. as a simple approach, all DL PDCP SDUs arriving from the Serving Gateway (SGW) are sent to the UE and forwarded in sequence to the target eNB.

Since all DL PDCP SDUs are forwarded to the target eNB it is sufficient to indicate the PDCP SN of the first forwarded DL PDCP SDU, which can be done using the SN STATUS TRANSFER message. The SN STATUS TRANSFER message also need to be sent to convey HFN transmitter status for COUNT preservation in the target eNB.

Sending the SN STATUS TRANSFER message at this point (before the transmission/reception to/from the UE has been stopped) only conveying the SN of the first forwarded DL PDCP SDU and HFN transmitter status is clearly a deviation from legacy behavior in which case the X2AP message is only sent after the source eNB considers the transmitter/receiver status to be frozen. Obviously, the conditions for sending the SN STATUS TRANSFER message when enhanced MBB is configured need to be discussed in RAN3.

Forwarding of DL PDCP SDUs will continue until the “end marker” packet is received from the SGW on the old path as a result of the DL path switch request triggered by the target eNB.

It is assumed that the source eNB continues to send UL PDCP SDUs to the SGW on the old S1-U path until the “end marker” packet is received from the SGW, although forwarding of UL data to the target eNB (i.e. like forwarding of DL data) is also a possible solution.

For further details on data forwarding including a discussion on data forwarding of UL data, see [3].

Step 8-9:	UE starts to synchronize with the target cell while it continues to receive and transmit packet data in the source cell. UE performs random-access in the target cell and target eNB schedules uplink resources.

Step 10:	After random-access is completed (RAR in case of CFRA, after contention resolution in case of CBRA), the UE stops sending UL PDCP PDUs in the source cell whereas it still may receive DL PDCP PDUs from the source eNB and DL PDCP PDUs from the target eNB. New and retransmitted UL PDCP PDUs are only sent in the target cell.

The UE keeps the source protocol stack active until the source eNB stops transmitting DL PDCP PDUs. The release of the source protocol stack is done implicitly by the UE. Details are FFS.

Step 11-12:	In purpose to assist the target eNB with PDCP duplication check, the UE may send a PDCP SN Status Report [4] for each RLC-AM bearer configured to the UE. Depending on when the source protocol stack is released, the PDCP SN Status Report is either sent together with the RRCConnectionReconfigurationComplete (Handover Complete) message (multiplexed on MAC level if the provided UL grant is sufficiently large) or after the RRC message has been sent. Based on the FMS field (first missing PDCP SN) in the PDCP Status Report, the target eNB starts sending DL PDCP PDUs to the UE from the first missing PDCP SN (i.e. from the last PDCP SN +1 received in the source cell) whereby sending of duplicated packets over the radio interface is avoided.
If the bitmap is present in the PDCP Status Report indicating one or more PDCP SDUs stored out-of-sequence in the UE, then the PDCP PDUs of the missing SNs need to be delivered.

As in legacy, the target eNB now sends a DL path switch request to the MME/SGW.

Step 13:	When receiving the Handover Complete message, the target eNB immediately sends a (new) X2AP message to the source eNB to indicate that the handover has completed, thus based on this information the source eNB can safely stop the transmission/reception to/from the UE.

This is considered as a better alternative than relying on autonomous detection in the source eNB (missing MAC HARQ or RLC ARQ feedback), with the obvious drawback that transmission resources may be tied up in vain.

Yet another trigger alternative to stop transmission/reception to/from the UE could potentially be at reception of the “end marker” packet in the source eNB (received from SGW as a result of the DL path switch request triggered by the target eNB). However, it is expected that the ”end marker” packet will have a longer turn-around time compared to an indicator sent directly over X2, thus with the proposed X2AP message the transmission/reception in the source eNB will be stopped at an earlier point.

Step 14:	The source eNB sends an SN STATUS TRANSFER message to convey the UL PDCP SN receiver status and the HFN receiver status.

As can be understood from above, the UE need to have the ability to simultaneously receive PDCP packets from both source cell and target cell until the connection to the source eNB is released.

In the “compromise solution” the source protocol stack in the UE is kept active after the completion of the random-access procedure, allowing the UE to receive PDCP packets from both the source cell and the target cell until the connection to the source eNB is released. Transmission of new UL PDCP packets and retransmission of PDCP packets that were not successfully received in the source eNB are however only sent to the target eNB.

Early start of data forwarding is applied when enhanced Make-Before-Break is configured. The exact point in time when the source eNB starts data forwarding is left for implementation.

All DL PDCP SDUs received from the Serving Gateway are forwarded in sequence to the target eNB, i.e. the source eNB does not apply any form of selective data forwarding. SN of the first forwarded DL PDCP SDU and HFN transmitter status can be conveyed to the target eNB using the SN STATUS TRANSFER message.

To minimize the number of duplicated DL PDCP PDUs sent over the radio interface, the target eNB performs a PDCP duplication check based on the PDCP Status Report sent by the UE. The PDCP Status Report is sent for each RLC-AM bearer configured to the UE.

After completing the random-access procedure, the UE stops sending UL PDCP PDUs in the source cell whereas it still may receive DL PDCP PDUs from the source eNB. New and retransmitted UL PDCP PDUs are only sent in the target cell.

The UE keeps the source cell connection until the source eNB stops sending DL data. How to trigger the release of the source cell connection in the UE is FFS.

A new X2AP message sent from target eNB to source eNB indicating “handover completion” should be considered. The new message will trigger the source eNB to stop transmitting DL data to the UE. The message will also serve as an indication that the UE has stopped its UL transmission in the source cell. Final decision is up to RAN3.

At reception of the X2AP message indicating “handover completion”, the source eNB sends an SN STATUS TRANSFER message to convey the UL PDCP SN receiver status and the HFN receiver status.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Early start of data forwarding is applied when enhanced Make-Before-Break is configured. The exact point in time when the source eNB starts data forwarding is left for implementation.

Proposal 2	All DL PDCP SDUs received from the Serving Gateway are forwarded in sequence to the target eNB, i.e. the source eNB does not apply any form of selective data forwarding. SN of the first forwarded DL PDCP SDU and HFN transmitter status can be conveyed to the target eNB using the SN STATUS TRANSFER message.

Proposal 3	To minimize the number of duplicated DL PDCP PDUs sent over the radio interface, the target eNB performs a PDCP duplication check based on the PDCP Status Report sent by the UE. The PDCP Status Report is sent for each RLC-AM bearer configured to the UE.

Proposal 4	After completing the random-access procedure, the UE stops sending UL PDCP PDUs in the source cell whereas it still may receive DL PDCP PDUs from the source eNB. New and retransmitted UL PDCP PDUs are only sent in the target cell.

Proposal 5	The UE keeps the source cell connection until the source eNB stops sending DL data. How to trigger the release of the source cell connection in the UE is FFS.

Proposal 6	A new X2AP message sent from target eNB to source eNB indicating “handover completion” should be considered. The new message will trigger the source eNB to stop transmitting DL data to the UE. The message will also serve as an indication that the UE has stopped its UL transmission in the source cell. Final decision is up to RAN3.

Proposal 7	At reception of the X2AP message indicating “handover completion”, the source eNB sends an SN STATUS TRANSFER message to convey the UL PDCP SN receiver status and the HFN receiver status.

# References

RP-181475, New Work Item on even further Mobility enhancement in E-UTRAN, RAN#80

R2-1907308, Comparison of interruption time in single and dual active protocol stack solution, RAN2#106

R2-1907309, Data forwarding at reduced handover interruption, RAN2#106

3GPP TS 36.323, Packet Data Convergence Protocol (PDCP) specification, Release 15