3GPP TSG-RAN WG2 Meeting #107                                                       R2-1908741

Prague, Czech Republic, 26th - 30th August 2019

Source:	CATT

Title:	Resource Allocation Mode Configuration

Agenda Item:	11.4.2

Document for:	Discussion and Decision

Introduction

During the last RAN2 meetings, the details of both mode 1 and mode 2 resource allocation modes were discussed. But till now, how to configure the resource allocation mode still has not been discussed. If the SL resource allocation mode configuration procedure cannot be decided, the system cannot work. Considering the deadline of Rel-16 NR V2X WI, there are only three RAN2 meetings left. Hence, it is time for RAN2 to start the discussion on SL resource allocation mode configuration procedure.

In this contribution, we will discuss the details of the SL resource allocation mode configuration and give our preferences.

Discussion

The discussion includes three parts: the resource allocation mode configuration granularity, the detailed resource allocation mode configuration signaling procedure, and the resource allocation mode collision issue.

Resource allocation mode configuration granularity

In LTE V2X, for RRC_CONNECTED UE, the resource allocation mode is configured based on UE and only one resource allocation mode can be configured to a UE, either mode-3 or mode-4.

In NR V2X, it was agreed that:

RAN2#104:

RAN2 will support the case a UE can be configured to perform both mode-1 and mode-2 at the same

time assuming RAN1 does not have concern on it. FFS on the scenario which it is applicable.

RAN2#105:

Confirm that UE may be configured to perform both network controlled sidelink transmission and UE

autonomous sidelink transmission.

Based on the above agreements, different from LTE, one NR V2X UE can be configured with two resource allocation modes simultaneously. The main motivation of introducing two resource allocation modes configured simultaneously is that NR V2X supports multiple services with different QoS requirements. Different V2X services may need different resource allocation mode in order to meet their different QoS requirements. Hence when configuring the resource allocation mode, there can be two options:

- Option 1: based on SLRB;

- Option 2: based on V2X QoS parameters.

Since RAN2 has already agreed that:

- For V2X transmission in SL unicast, SLRB configurations are NW configured or pre-configured.

- For V2X transmission in SL gouprcast or SL broadcast, SLRB configurations are NW configured or pre-configured.

Apparently, when the UE is in RRC_CONNECTED state, the SLRB configurations should be NW configured and UE will report the V2X QoS parameters to the NW. Hence when configuring the resource allocation mode for RRC_CONNECTED UE, the most directly way is to configure it based on SLRB.

Proposal 1: Resource allocation mode should be configured based on SLRB.

Furthermore, it needs to be decided whether one SLRB can be configured with more than one resource allocation mode simultaneously. If it is allowed, it should depend on UE implementation to determine which resource allocation mode should be used at each transmission occasion. If the resource allocation mode is determined by UE and UE is allowed to change the resource allocation mode frequently, the SR/BSR procedure will be very complex. Hence, one SLRB should be configured with only one resource allocation mode.

Proposal 2:  One SLRB should be configured with only one resource allocation mode.

Resource allocation mode configuration signaling procedure

In LTE, the resource allocation mode configuration procedure for RRC_CONNECTE UE is initiated after the SLRB has been setup, shown in Figure-1:

Figure-1 LTE resource allocation mode configuration procedure

For LTE V2X, the sidelinkUEInformation includes the following parameters:

SidelinkUEInformation-v1430-IEs ::=	SEQUENCE {

v2x-CommRxInterestedFreqList-r14	SL-V2X-CommFreqList-r14				OPTIONAL,

p2x-CommTxType-r14					ENUMERATED {true}					OPTIONAL,

v2x-CommTxResourceReq-r14			SL-V2X-CommTxFreqList-r14			OPTIONAL,

nonCriticalExtension				SidelinkUEInformation-v1530-IEs		OPTIONAL

}

SidelinkUEInformation-v1530-IEs ::=	SEQUENCE {

reliabilityInfoListSL-r15			SL-ReliabilityList-r15					OPTIONAL,

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

In NR, for RRC_CONNECTED UE, the above resource allocation mode configuration procedure can be used as baseline. Besides the frequency requested for V2X transmission, the SLRB identifier should also be carried in the SidelinkUEInformation.

Proposal 3: The resource allocation mode configuration signaling procedure of LTE can be used as baseline for NR V2X, i.e., the resource allocation mode configuration can be based on the SidelinkUEInformation sent by UE.

Furthermore, in order to reduce the latency, enhancement on resource allocation mode configuration signaling procedure can be considered for NR V2X. That is to merge the SLRB configuration procedure and the resource allocation mode configuration procedure into one procedure, as shown in Figure-2:

Figure-2   Optimization for resource allocation mode configuration procedure

Proposal 3a: As an optimization for latency reduction, the SLRB configuration procedure and the resource allocation mode configuration procedure can be merged into one, e.g., merge the sidelinkUEInformation and the SL configuration request message into one message.

Resource allocation mode collision issue

Based on Proposal 1 and Proposal 2, it is suggested that resource allocation mode is configured based on SLRB and only one resource allocation mode should be configured for one SLRB. Considering NR V2X SL has only one BWP, hence it is possible that different RBs use different resource allocation mode on the same BWP, as shown in Figure-3:

Figure-3 Different SLRBs use different resource allocation modes on the same BWP

As shown in Figure-3, if the two SLRBs use different resource allocation mode both have available data simultaneously, the UE may need to transmit more than one TB on one BWP at the same time. But according to UE capability, UE may be unable to support more than one TB on one BWP at the same time. In this case, which resource allocation mode should be used at the collision time should be addressed, which is shown in Figure-4.

Figure-4 Resource allocation mode collision

In order to solve the above resource allocation mode collision issue, some rules should be defined. The possible rules are listed below:

- Option 1: Based on highest SLRB priority using this resource allocation mode and has data available for transmission;

- Option 2: Mode-1 is always prioritized;

- Option 3: Mode-2 is always prioritized.

Compared with Option 2 and Option 3, Option 1 is more attractive because it will prioritize the high priority SL V2X service transmission.

Proposal 4: If two resource allocation modes collide in the time domain and UE is not capable to transmit two TBs simultaneously, the resource allocation mode whose corresponding SLRB has the highest priority and with avaliable data should be prioritized.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: Resource allocation mode should be configured based on SLRB.

Proposal 2:  One SLRB should be configured with only one resource allocation mode.

Proposal 3: The resource allocation mode configuration signaling procedure of LTE can be used as baseline for NR V2X, i.e., the resource allocation mode configuration can be based on the SidelinkUEInformation sent by UE.

Proposal 3a: As an optimization for latency reduction, the SLRB configuration procedure and the resource allocation mode configuration procedure can be merged into one, e.g., merge the sidelinkUEInformation and the SL configuration request message into one message.

Proposal 4: If two resource allocation modes collide in the time domain and UE is not capable to transmit two TBs simultaneously, the resource allocation mode whose corresponding SLRB has the highest priority and with available data should be prioritized.